3GPP TSG-RAN WG4 Meeting # 115						        R4-2507178

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.6.2

Source: 	OPPO

Title: 	On FDD PC2 Redcap

Document for:	Approval

- Introduction

The FDD PC2 Redcap requirement has been fully discussed in previous meetings. A WF [1] has been agreed and in this paper we try to give further discussion on the REFSENS requirement.

- Discussion

During the RAN4#114bis meeting, the agreed WF has been captured as below:

Issue 1-1-1: REFSENS for 1Rx in FD-FDD

Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD

- Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges

  - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB.

  - Companies to evaluate REFSENS for other bands.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

- Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions

  - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1.

  - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

Way forward: It is encouraged for companies to provide analysis for option 1 and 2.

Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

Currently the REFSENS for HD-FDD 1RX for Redcap PC3 has been agreed and this can be the basic PC3 RX noise baseline. The PC3 1TX 2RX FD-FDD for non-Redcap PC3 has also been agreed and this can be the basic PC3 2RX MRC noise baseline. The ΔR 1R has been agreed for PC3 1TX 1RX FD-FDD and it can be the PC3 PRX noise plus PC3 TX interference baseline. With that we can calculate the TX interference for 1TX 1RX for PC3 and with 3dB higher we can get the TX interference for 1TX 1RX PC2.

Observation 1: The PC2 1TX 1RX interference can be calculated for each band and channel bandwidth based on current already defined requirements.

Furthermore, with the PC2 1TX 1RX interference as well as the RX noise baseline, the combined noise with TX+RX can be determined and with the comparison to PC3 1TX 1RX combined noise, the RSD for different channel bandwidth as well as different bands can be derived.

Proposal 1: The PC2 1TX 1RX RSD compared to PC3 1TX 1RX can be derived.

Below we show a table for band n3 for example.

Table 1 Calculation for band n3

| band n3 | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- |
| PC3 REFSENS | -97.0 | -93.8 | -92.0 | -90.8 |
| PC2 1TX 2RX RSD | 0.5 | 0.5 | 0.5 | 0.5 |
| HD FDD 1RX | -95.3 | -92.1 | -90.3 | -89.0 |
| PC3 TX noise in PRX | -101.2 | -95.7 | -93.9 | -93.0 |
| Redcap 1RX REFSENS | -94.5 | -90.8 | -89.0 | -87.8 |
| PC3 RX without TX | -94.3 | -91.1 | -89.3 | -88.0 |
| PC2 1TX 2RX REFSENS | -96.5 | -93.3 | -91.5 | -90.3 |
| PC3 Combined Noise considering both RX and TX | -96.0 | -92.8 | -91.0 | -89.8 |
| PC3 TX+RX noise PRX | -93.5 | -89.8 | -88.0 | -86.8 |
| PC2 Combined Noise | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 TX noise in PRX | -98.2 | -92.7 | -90.9 | -90.0 |
| PC2 TX+RX noise in PRX = PC3 RX noise+PC2 TX noise | -92.2 | -88.0 | -86.2 | -85.1 |
| PC2 1TX 1RX RSD | 1.3 | 1.8 | 1.8 | 1.7 |

Above table 1 has shown the calculation for PC2 1TX 1RX RSD as:

| band n3 | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- |
| PC2 1TX 1RX RSD | 1.3 | 1.8 | 1.8 | 1.7 |

Proposal 2: It is proposed to use below numbers for PC2 RSD for 1TX 1RX for band n3:

| band n3 | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- |
| PC2 1TX 1RX RSD | 1.3 | 1.8 | 1.8 | 1.7 |

3	Conclusions

In this paper, we give discussion on the PC2 Redcap, the observations and proposals are as below.

Observation 1: The PC2 1TX 1RX interference can be calculated for each band and channel bandwidth based on current already defined requirements.

Proposal 1: The PC2 1TX 1RX RSD compared to PC3 1TX 1RX can be derived.

Proposal 2: It is proposed to use below numbers for PC2 RSD for 1TX 1RX for band n3:

| band n3 | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- |
| PC2 1TX 1RX RSD | 1.3 | 1.8 | 1.8 | 1.7 |

4 References

[1] R4-2505231	WF on PC2 RedCap UE RF requirements, China Telecom