# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507188

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for RedCap NTN enhancement

Agenda item:	7.31.5.1

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

RAN4 shall define the RRM requirements based on the latest revised WID as below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Discussion

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

  - 2.1 General consideration on RRM requirements

In above WID description, we know that RAN4 shall combine with R17/R18 RedCap WI and R17/R18 NR NTN WI to discuss this objective. Thus, we shall study the features introduced in RedCap which will impact the definition of requirements.

Issue 1-1-1: SDT requirements

In the previous meeting, we spent several minutes on discussing SDT requirements and the way forward could be seen as below:

- <Way Forward>: - Proposal 1: For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2. - The agreement reached in the last meeting only focus on RAN4’s requirements. - Proposal 2: RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

In fact, we observed that cg-SDT-TA-ValidationConfig is an optional feature as below:

cg-SDT-TA-ValidationConfig-r17 SetupRelease { CG-SDT-TA-ValidationConfig-r17 } OPTIONAL, -- Need M cg-SDT-TA-ValidationConfig Configuration for the RSRP based TA validation. If this field is not configured, then the UE does not perform RSRP based TA validation

Based on this understanding, if cg-SDT-TA-ValidationConfig-r17 is not configured, then the UE does not perform RSRP based on TA validation. Consequently, there are no related RRM requirements if the network configures this feature. From the network perspective, this shall be the network implementation and RAN4 shall not send the LS to RAN2.

Proposal 1: RAN4 shall not send LS to RAN2 on SDT requirements for RedCap UE in NTN scenario..

Issue 1-1-2: MDT requirements

For the discussion of MDT requirements, the way forward can be seen as below:

<Way Forward>: Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

We see majority companies would not like to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs.

To my understanding, for logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

For measurement requirements:

The requirements specified in this clause apply for the following measurements performed and logged by the UE for MDT in RRC_IDLE: - SS-RSRP per cell, - SS-RSRQ per cell, - SS-RSRP per SSB index of the serving cell, - SS-RSRQ per SSB index of the serving cell, - best SSB index of the serving cell, - the number of SSBs with different SSB index which are above the threshold absThreshSS-BlocksConsolidation for all detected cells whose cell-ranking criterion R value is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell. The requirements apply for the measurements included in logged MDT reports and RRC connection establishment failure reports. The measurement values that are used to meet serving cell and reselection requirements as specified in clauses 4.2C.2.24.2C.2.9 shall also apply to values logged for MDT measurements in RRC_IDLE state.

We deem that the legacy measurement requirements shall be reused.

The concrete accuracy requirements (relative time stamp) can be seen as below:

For logged measurement: The relative time stamp for a logged measurement is defined as the time from the moment the MDT configuration was received at the UE until the measurement was logged. The accuracy of the relative time stamping is such that the drift of the time stamping shall be not more than ± 2 seconds per hour For RRC connection establishment failure log reporting: Relative time stamp for RRC connection establishment failure log reporting is defined as the time elapsed from the last RRC connection establishment failure to the time when the log is included in the report. The accuracy of the relative time stamping for RRC connection establishment failure log reporting is such that the drift of the time stamping shall not be larger than ± 0.72 seconds per hour and ± 10 seconds over 48 hours. For radio link failure and handover failure log reporting: Relative time stamp accuracy requirements for timeSinceFailure reported for MDT in a radio link failure or handover failure log are specified. timeSinceFailure determines the time elapsed from the last radio link failure or handover failure in NR to the time when the log is included in the report. The accuracy of the relative time stamping for timeSinceFailure is such that the drift of the time stamping shall not be larger than ± 0.72 seconds per hour and ± 10 seconds over 48 hours.

To my understanding, RAN4 shall define the related timing accuracy requirements. However, the requirements will not be impacted by the reduced UE receiving branches since the accuracy of the relative time stamping is the drift of the time stamping. Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied. Since the differences between 1Rx UE and 2Rx UE in terms of timestamp recording accuracy are primarily determined by their internal hardware configurations, not directly by the number of antennas.

Observation 2: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 3: Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied.

Proposal 2: RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements.

Proposal 3: For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 4: For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

Issue 1-2-3: Other features

In last meeting, RAN4 has discussed the SMTC adjustment and location-based measurement report trigger, the following progress can be seen:

Agreement: - SMTC adjustment in idle/inactive is kept [mandatory] for (e)RedCap UEs with NTN. - From RAN4 perspective, location-based measurement report trigger is kept optional for (e)RedCap UEs with NTN.

For SMTC adjustment, the feature is defined in 38.822:

34-11; SMTC adjustment in idle/inactive; It's optional for UE to perform SMTC adjustment in RRC_IDLE/RRC_INACTIVE; n/a; n/a; n/a; n/a; Optional without capability signalling.

Some companies believe that SMTC adjustment in idle/inactive should be mandatory. However, we discovered that this feature is optional without capability signaling. This feature is used in idle/inactive measurement, aware that the rationale for mandating this feature is to ensure reliable neighbour cell measurement. Yet, the logic appears somewhat unclear here, considering this feature is unrelated to the UE types and optional in NTN scenario, why is it being defined as mandatory in redcap NTN scenario. We recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

Proposal 5: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

  - 2.2 Reduction in the number of UE Rx branches

- RAN4 has spent several meetings in discussing the reduction in the number of UE Rx branches. Some issues are still under discussion and we would like to provide our views on the following issues.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

- For inter-RAT cells with TN carrier, RAN4 has reached several agreements as below:

- Agreement: - For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN, - For 1 Rx and 2 Rx RedCap UEs not configured with eDRX cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN (cl. 4.2C.2.11). - For 1 Rx and 2 Rx RedCap UEs configured with eDRX cycle, RAN4 to reuse the existing requirements (Table 4.2B.2.5-2) defined for (e)RedCap UE with TN (cl. 4.2B.2.5) as a baseline and adopt NTN principles on how measurements are triggered/etc when implementing the requirement in the specification. Note that only eDRX cycles up to 10.24 sec are applicable for (e)RedCap NR-NTN UEs. - Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

- Based on the above agreements, for 1Rx and 2Rx RedCap UEs not configured with eDRX cycle, RAN4 use the legacy requirements defined in FR1-NTN. For 1Rx and 2Rx RedCap UEs configured with eDRX cycle, the legacy requirements defined for RedCap UE with TN is as baseline. RAN4 shall adopt NTN principles on how measurements are triggered when implementing the requirements in sec. We know that the motivation is defining the requirements with NTN scenario. From our perspective, two points shall be noticed:

- The same logic on defining requirements without configuring eDRX cycle.

- Only eDRX cycles up to 10.24s are applicable for redcap NTN UEs.

- Proposal 6: For 1Rx and 2Rx RedCap UEs configured with eDRX cycle, the requirements could be:

Table 4.2B.2.5-2: Tdetect,EUTRAN_RedCap_NTN, Tmeasure,EUTRAN_RedCap_NTN, and Tevaluate,EUTRAN_RedCap_NTN for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Tdetect,EUTRAN_RedCap_NTN [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap_NTN [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap_NTN [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | N/A | N/A | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | N/A | N/A | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | N/A | N/A | 235.52 (23) | 10.24 (1) | 20.48 (2) |

  - 2.3 The impact of HD-FDD

- Compared to FD-FDD UE, HD-FDD operates by using separate frequency bands for uplink and downlink communication, but with a UE only being able to either transmit or receive at any given time – not both simultaneously. In RedCap WI, they discussed the several requirements for HD-FDD UE and define lots of applicable conditions not the new requirements.

Issue 3-1-1: General HD-FDD related impact

- In last meeting, the basic agreements could be seen as below:

- Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements - RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

- For paging reception requirements in RRC Idle/Inactive state, we deem that the legacy condition can be reused. The following reasons shall be considered:

The 1 Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2.2.6 under the following conditions - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration.

- We observe that the 1Rx RedCap HD-FDD meet all applicable requirements specified in clause 4.2.2.6, which has the same requirements in legacy NR NTN except for K. The values of K equals 2 in legacy RedCap UE, but equals 2 or 5 depends on whether the satellite is the same satellite or different satellites. Thus, we believe that the legacy requirements and legacy condition can be reused.

- For handover interruption time, the same logic as paging reception requirements, the legacy applicability rule as following:

For RedCap UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

- To our understanding, the lagacy requirements could be reused provided that the legacy conditions are reused.

- For random access, RRC connection release with redirection to NR and scheduling availability of UE performing intra-/inter- measurements, to our understanding, the same logic as above, which legacy requirements could be reused provided the legacy conditions are reused.

- Proposal 7: The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE for HD-FDD in NTN scenario:

- Paging reception requirements in Idle/Inactive state; - Handover interruption time; - Random access; - RRC connection release with redirection to NR; - Scheduling availability of UE performing intra-/inter- measurement.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR RedCap NTN .

Proposal 1: RAN4 shall not send LS to RAN2 on SDT requirements for RedCap UE in NTN scenario..

Observation 2: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 3: Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied.

Proposal 2: RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements.

Proposal 3: For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 4: For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 5: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

- Proposal 6: For 1Rx and 2Rx RedCap UEs configured with eDRX cycle, the requirements could be:

Table 4.2B.2.5-2: Tdetect,EUTRAN_RedCap_NTN, Tmeasure,EUTRAN_RedCap_NTN, and Tevaluate,EUTRAN_RedCap_NTN for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Tdetect,EUTRAN_RedCap_NTN [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap_NTN [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap_NTN [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | N/A | N/A | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | N/A | N/A | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | N/A | N/A | 235.52 (23) | 10.24 (1) | 20.48 (2) |

- Proposal 7: The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE for HD-FDD in NTN scenario:

- Paging reception requirements in Idle/Inactive state; - Handover interruption time; - Random access; - RRC connection release with redirection to NR; - Scheduling availability of UE performing intra-/inter- measurement.

- References

[1] R4-2502604，WF on RRM requirements for NR_NTN_Ph3_Part1.

[2] TS38.133

[3] TS38.304

[4] TS38.331