3GPP TSG-RAN WG4 Meeting #115                                        R4-2507253

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:              Discussion on performance requirements for NES

Agenda item:			7.27.4

Document for:	Discussion

1. Introduction

As the per TU guidance, discussions on RRM performance requirements shall be started from RAN4 #115. In this meeting, we will give some high-level views on the test case scope for NES OD-SSB and SSB adaptation, separately.

2 Performance requirements for NES OD-SSB

L1 Measurement procedure

- The following agreements were achieved in the previous RAN4 meetings:

#114 Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. Based on OD-SSB periodicity #114-bis Issue 1-4-4: L1 measurement after SCell activation when DRX is configured Agreement - For Scenario 2 and Scenario 2a, when DRX is configured, following the legacy requirements based on DRX with the consideration of OD-SSB is not deactivated.

- The following agreements were achieved in the RAN1 meetings:

#118 Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point #120b Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met. - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst. - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies.

On top of such agreements, new TCs for SSB-based L1-RSRP measurements for Reporting are needed for NES OD-SSB, the impacted factor is Measurement period TL1-RSRP_Measurement_Period_SSB. While, considering following controversial points: 1) How/whether to enhance CSI report configuration for OD-SSB for case 2 that AO-SSB and OD-SSB have the different center frequencies; 2) How UE to measure SSB resources when higher layer parameter timeRestrictionForChannelMeasurements is configured, wherein, timeRestrictionForChannelMeasurements configured is also the assumption in test parameters, were still left open, we have the following proposal:

Proposal 1: RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES OD-SSB

- New TCs are to verify the measurement procedure when OD-SSB transmission is indicated by NW

  - How to/Whether to design test case for case 2 depends on more progresses on CSI measurement and report configurations in RAN1.

SCell Activation and Deactivation Delay

New requirements are defined for SCell Activation Delay for Deactivated SCell (Tactivation_time) with unsettled details, including

- SCell Activation and deactivation for SCell

- Direct SCell activation at SCell addition

- PUCCH SCell activation

- Fast SCell Activation

- SCell Activation with L3 reporting during activation

At the same time,

- For case 2: Different requirements will be defined depending on when UE to receive OD-SSB activation indication (i.e., the time relation in terms of when receiving OD-SSB activation indication and when receiving the SCell activation command)

- For case 1: Although SCell activation delay requirement shall apply based on OD-SSB periodicity, the deactivated SCell measurement procedure will or will not be involved, whether the target SCell supporting OD-SSB can be known or unknown to UE should be discussed case-by-case, accordingly

From this, we have the following proposals:

Proposal 2: RAN4 to introduce new tests for SCell Activation and Deactivation Delay requirements for NES OD-SSB in FR1 and FR2, separately. The test cases involving case #1 and case #2 may capture below:

- SCell Activation and deactivation for SCell

- Direct SCell activation at SCell addition

- PUCCH SCell activation and deactivation

- Fast SCell Activation

- SCell Activation with L3 reporting during activation

Proposal 3: The new test cases for SCell Activation and deactivation for NES OD-SSB in FR1 should contain known SCell and unknown SCell.

- FFS: Whether to/How to update the 160ms/640 ms SCell measurement cycle limitation for known SCell test case

Proposal 4: To define new test cases for SCell Activation and deactivation for NES OD-SSB, scenario #2 and scenario #2a should be considered

Interruption

Based on the following agreements：

Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

New interruption requirements are defined for scenario 2 due to OD-SSB activation/deactivation, the new interruption requirement is related to the definition of FMW. In this sense,

Proposal 5: RAN4 to define new test case for interruptions during measurements on deactivated NR SCC for NES OD-SSB

- How to design the test case depends on the definition of condition, duration and position of the interruption

Measurement performance

Based on the agreements we achieved in last meeting

Issue 4-3: OD-SSB accuracy Agreement: - The legacy measurement accuracy is applied for OD-SSB based measurement.

From our understanding, there is no need to define corresponding test case to verify the measurement performance for NES OD-SSB

Proposal 6: No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB

L3 measurement procedure

The core requirement of OD-SSB based NR intra-frequency measurement was agreed to define in last meeting.

- RAN2 agreed to keep SMTC based L3 RRM framework to perform L3 measurement, below

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

From understanding, at least from activated SCell measurement perspective, no essential distinction in between OD-SSB and SSB adaptation, just with different NW triggering mechanism, in both cases, SMTC based L3 RRM framework should be kept. In this sense, the L3 measurement procedure of OD-SSB based measurement should be tested, especially for the scenario combination case, that is UE to receive the MAC-CE OD-SSB periodicity updating/adaptation indication in scenario #3B and to perform L3 measurement based on adaptative SMTC (i.e., additional SMTC configuration according to SSB adaptation).

- Discussion on how to/whether to define requirements for scenario 3 (OD-SSB and AO-SSB are within different frequencies) was just put on the agenda.

- The frequency location of OD-SSB is the same as the frequency location of AO-SSB at least for the case where AO-SSB is NCD-SSB

In short, we have the following proposals:

Proposal 7: RAN4 prioritize to introduce new test cases for intra-frequency measurement requirements for NES OD-SSB, the test case may capture below:

- SA event triggered reporting test without gap under DRX with NCD-SSB

  - FFS: Which configured event should be considered

3 Performance requirements for NES SSB adaptation

Measurement performance

Same proposal can be given here

Proposal 8: No new test case for A.7.7 Measurement Performance requirements for NES SSB adaptation

L1 Measurement procedure

Based on the following agreements,

Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD

L1-RSRP measurement on SCell were defined with the consideration of adapted SSB periodicity, which is triggered/indicated by NW signalling DCI 2_9. And for adaptation of SSB in time-domain, the UE can be configured with at most 2 additional SSB burst periodicities, which is different from what the periodicity can to be configured for OD-SSB. Besides, for L1-SINR, the functionality difference to legacy SSB based measurement can be verified by L1-RSRP test, so there is no need to define additional L1-SINR one. At the same time, whether additional RRC signaling for offset of SSB for additional SSB periodicity is needed is still suspending in RAN1. From this, we have the following proposal:

Proposal 9: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation

- New TCs are to verify the measurement procedure when SSB adaptation is activated by NW signalling DCI 2_9

  - Whether additional RRC signaling for offset of SSB for additional SSB periodicity should be considered in the new test case depends on RAN1 progress.

L3 Measurement procedure

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell. And mapping relationship between X additional SSB burst periodicity with Y additional SMTC configuration(s), provided via RRC

- The following scenario: Frequency location is same before and after SSB adaptation; No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation are supported for SSB adaptation; Time domain positions of SSBs within a burst is same before and after SSB adaptation; SSB adaptation in SCell for NCD-SSB.

Proposal 10: RAN4 to introduce new test cases for intra-frequency measurement requirements for NES SSB adaptation, the test case may capture below:

- SA event triggered reporting test without gap under DRX with NCD-SSB

  - FFS: Which configured event should be considered/how to trigger the measurement report

4 Conclusion

In this contribution, we provided our viewpoints on RRM performance requirements for NES OD-SSB and SSB adaptation. The following observations and proposals are obtained:

Performance requirements for NES OD-SSB

Proposal 1: RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES OD-SSB

- New TCs are to verify the measurement procedure when OD-SSB transmission is indicated by NW

  - How to/Whether to design test case for case 2 depends on more progresses on CSI measurement and report configurations in RAN1.

Proposal 2: RAN4 to introduce new tests for SCell Activation and Deactivation Delay requirements for NES OD-SSB in FR1 and FR2, separately. The test cases involving case #1 and case #2 may capture below:

- SCell Activation and deactivation for SCell

- Direct SCell activation at SCell addition

- PUCCH SCell activation and deactivation

- Fast SCell Activation

- SCell Activation with L3 reporting during activation

Proposal 3: The new test cases for SCell Activation and deactivation for NES OD-SSB in FR1 should contain known SCell and unknown SCell.

- FFS: Whether to/How to update the 160ms/640 ms SCell measurement cycle limitation for known SCell test case

Proposal 4: To define new test cases for SCell Activation and deactivation for NES OD-SSB, scenario #2 and scenario #2a should be considered

Proposal 5: RAN4 to define new test case for interruptions during measurements on deactivated NR SCC for NES OD-SSB

- How to design the test case depends on the definition of condition, duration and position of the interruption

Proposal 6: No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB

Proposal 7: RAN4 prioritize to introduce new test cases for intra-frequency measurement requirements for NES OD-SSB, the test case may capture below:

- SA event triggered reporting test without gap under DRX with NCD-SSB

  - FFS: Which configured event should be considered

Performance requirements for NES SSB adaptation

Proposal 8: No new test case for A.7.7 Measurement Performance requirements for NES SSB adaptation

Proposal 9: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation

- New TCs are to verify the measurement procedure when SSB adaptation is activated by NW signalling DCI 2_9

  - Whether additional RRC signaling for offset of SSB for additional SSB periodicity should be considered in the new test case depends on RAN1 progress.

Proposal 10: RAN4 to introduce new test cases for intra-frequency measurement requirements for NES SSB adaptation, the test case may capture below:

- SA event triggered reporting test without gap under DRX with NCD-SSB

  - FFS: Which configured event should be considered/how to trigger the measurement report

5 References