3GPP TSG-RAN WG4 Meeting #115                                    R4-2507257

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:           Test case on measurement delay reduction for SSB based inter-frequency measurement

Agenda item:			7.18.5

Document for:	Discussion

1. Introduction

Discussion on RRM performance requirements shall be started from RAN4 #115. In this meeting, we will give some high-level views on the test case design for measurement delay reduction for SSB based Inter-frequency measurement by optimizing BSF.

2. Discussion

In the existing spec., there are no test cases to verify delay reduction requirements for SSB based Inter-frequency measurement by optimizing BSF. From test coverage point of view, it is necessary to discuss and define test cases on the enhancement on PSS/SSS detection period requirements, time index detection period requirements, and measurement period requirements for inter-f with gaps in connected mode with the following agreed optimization for UE supporting [fast BSF for L3 measurement].

#114-bis Scenarios; Factors. 1; Mpss/sss_sync_w/o_gaps = [reduced N]*3 Mmeas_period_w/o_gaps = [reduced N]*3. 2; Mpss/sss_sync_with_gaps = [reduced N]*3 Mmeas_period_with_gaps = [reduced N]*3. 3; Mpss/sss_sync_inter = [reduced N]*3 Mmeas_period_inter = [reduced N]*3 MSSB_index_inter = [reduced N]*3. 4; Mpss/sss_sync_inter = [reduced N]*5 Mmeas_period_inter = [reduced N]*5 MSSB_index_inter = [reduced N]*3. 5; If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then N in the HO requirement will be replaced by [reduced N] when the target cell is in FR2-1 and the PC3 UE supports [Rel-19 Rx FBS].. 7; RRC Re-establishment/redirection Can be discussed directly in the draft CR..

Specifically, for inter-frequency with gap, one possible way is to cover the following cases:

| - Without SSB time index detection when DRX is not used | - Without SSB time index detection when DRX is used |
| --- | --- |
| - With SSB time index detection when DRX is not used | - With SSB time index detection when DRX is used |

Observation 1: To define test case on measurement delay reduction for SSB based Inter-frequency measurement

with MG by optimizing BSF, one possible way is to cover the following cases:

- Without SSB time index detection when DRX is not used

- Without SSB time index detection when DRX is used

- With SSB time index detection when DRX is not used

- With SSB time index detection when DRX is used

From our understanding, since all the TCs are intended to test the same [reduced N], there is no need to define all the test cases, prefer to define when DRX is used case since power saving is one of the critical issues of PC3 UE

Proposal 1: RAN4 to introduce following new test cases to verify SSB based Inter-frequency measurement with MG performance requirements for Rx BSF optimization

- Without SSB time index detection when DRX is used

- With SSB time index detection when DRX is used

Recall test setup conclusions in Rel-15 NR_newRAT-Perf WI, the following information can be found:

- For SA NR inter-frequency measurement test case:  Setup # 1 in non-DRX and short DRX. Setup # 3 for long DRX [from R4-1904783],

from our understanding, since multi-rx simultaneous reception is supported, the legacy Setup #1 and Setup #3 are not applicable anymore. Considering in Rel-18 L1 multi-Rx Setup #5, 2 AoAs for simultaneous reception with different QCL Type-D was introduced, it is possible to consider the same test setup in our fast BSF based L3 measurement case, then we have

Proposal 2: For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

Since a new mechanism (i.e., FR2 serving cell’s signal quality based triggering condition) to trigger UE using FBS for L3 measurement are introduced, rather than the existing one (event-triggered reporting with Event A3), please find below

# 113 Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19. # 114 Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

in this sense, the test requirements should be defined by using serving cell’s signal quality based triggering condition, i.e., serving cell’s signal quality is worse than a threshold, including RSRP, RSRQ threshold.

Proposal 3: To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

Different from L1 multi-Rx simultaneous reception, how to define conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” is still suspending. From our understanding, such “status switching ” would be better to be verified.

Proposal 4: Prefer to define test cases to verify the process of quitting FBS, when the corresponding core requirement definition is stable

3 Conclusion

In this contribution, we provided our viewpoints on delay reduction requirements for SSB based Inter-frequency measurement by optimizing BSF test case. The following observations and proposals are obtained:

Observation 1: To define test case on measurement delay reduction for SSB based Inter-frequency measurement

with MG by optimizing BSF, one possible way is to cover the following cases:

- Without SSB time index detection when DRX is not used

- Without SSB time index detection when DRX is used

- With SSB time index detection when DRX is not used

- With SSB time index detection when DRX is used

Proposal 1: RAN4 to introduce following new test cases to verify SSB based Inter-frequency measurement with MG performance requirements for Rx BSF optimization

- Without SSB time index detection when DRX is used

- With SSB time index detection when DRX is used

Proposal 2: For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

Proposal 3: To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

Proposal 4: Prefer to define test cases to verify the process of quitting FBS, when the corresponding core requirement definition is stable

4 References

[1] R4-2504899 “WF on RRM requirements for NR_RRM_Ph5_Part1”, Apple