3GPP TSG-RAN WG4 Meeting #115	R4-2507289

Malta, MT, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on other RRM aspects of R19 NR MIMO Phase 5

Agenda Item:	7.24.4.2

Document for:	Discussion

- Introduction

In last meeting, around other RRM aspects of NR MIMO Phase 5, two sub-topics were confirmed to identify the RRM aspects and the following agreements were achieved and summarized in [1], we listed them as below:

<Topic 1: UE reporting enhancement for CJT calibration> Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration Agreement: - Update the report resolution to (MD = 256, MFO = 256). Moderator’s note: simulation results are collected in R4-2504955, companies can use the table structure in the excel doc to update their results. Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified) <Topic 2: Enhancement for asymmetric DL sTRP/UL mTRP scenarios> Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2. Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2? Agreement: - RAN4 to define timing requirements for FR1 as well as FR2. Issue 2-5: Whether to define UL TCI state activation requirement based on SRS? Agreement: No RRM impacts on TCI state switching requirement.

- In this paper, we provide our further views on the sub-topic of enhancement for asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

To improve UL throughput and power consumption saving, heterogeneous NW can be deployed with macro nodes and micro nodes in non-co-located deployment. Wherein the micro nodes only apply to provide UL load balance. Not any DL transmission happens in the micro link, so that the micro node can turn off the DL Tx to save power consumption. The following picture depicts the scenario as below:

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

Based on previous RAN1 progress, two scenarios are considered:

Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

It has been confirmed that “One downlink reference timing is supported and applied to both TAGs” is applicable to Scenario #1, and whether it is applicable to Scenario #2, RAN4 sent LS to RAN1 for clarification. In latest RAN1#120bis meeting, we noticed that RAN1 has already achieved consensus regarding the LS reply and the exact agreements are as below:

As a result, UE would maintain two DL timing for Scenario#2 and whether the offset of the two DL timing is confined to CP length, it is up to UE capability. Based on such RAN1 progress, the timing assumption for both Scenario #1 and 2 can be concluded as the following proposal1.

Proposal 1: For Scenario #1, a single DL reference timing is shared for both TAGs/TRPs. For Scenario #2, two DL reference timing are maintained by UE, and each DL reference timing per TAG/TRP.

Based on the DL timing assumption for each scenario, how to update the requirements of gradual timing especially for Scenario #1 should be figured out. For Scenario #1, since no DL signal sent by the UL TRP, a common DL reference timing is shared by the two TRPs. Since the DL&UL TRP and UL TRP is non co-located, if UE sends PRACH in the UL TRP with the common DL reference timing as reference timing, it is probably the early or late arrival of PRACH at the UL TRP. But it seems not any optimization to address the issue of PRACH transmission in UL TRP is approved in RAN1, so such early or late PRACH reception has to be handled by the CP and GP coverage. For the gradual timing adjustment, in R18 mTRP with two TAs, the gradual timing adjustment requirements apply for each TAG. Here we believe similar as R18 mTRP, the gradual timing adjustment requirements should also apply for each TAG/TRP.

Proposal 2: For Scenario #1, the gradual timing adjustment requirements should apply for each TAG/TRP.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: For Scenario #1, a single DL reference timing is shared for both TAGs/TRPs. For Scenario #2, two DL reference timing are maintained by UE, and each DL reference timing per TAG/TRP.

Proposal 2: For Scenario #1, the gradual timing adjustment requirements should apply for each TAG/TRP.

- Reference

- R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_part 2, MediaTek inc. RAN4#114bis

- R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN1#119