# 3GPP TSG-RAN WG4 Meeting #115	R4-2507323
Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.10.3.2.2

Source: 	OPPO

Title: 	Discussion on Rx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

The main issues for NR NTN HPUE Rx RF requirements in FR1-NTN bands are Reference Sensitivity Degradation for PC2, PC1.5 and PC1, and have discussed several meetings.

This contribution further discussed the UE Rx RF requirements for NR NTN HPUE.

- Discussion

We propose remove the square brackets for the RSD based on the WF [1] in last meeting.

Proposal 1:  RSD requirements for PC2 on n255 adopt following values

|  | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| RSD@1Tx | 0.8 | 0.8 | 0.9 | 0.9 |
| RSD@2Tx | 1.6 | 1.6 | 1.7 | 1.7 |

Proposal 2:  RSD requirements for PC1.5 with 2Tx adopt following values

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

Proposal 3:  RSD requirements for PC1 with 1Tx adopt following values

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 3 | 3 | 3 | 3 |
| n255 | 4 | 4 | 4 | 4 |
| n254 | 3 | 3 | 3 |  |

- Conclusion

In this paper we give further discussion on the RX requirement for HPUE NTN. The proposals are as following:

Proposal 1:  RSD requirements for PC2 on n255 adopt following values

|  | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| RSD@1Tx | 0.8 | 0.8 | 0.9 | 0.9 |
| RSD@2Tx | 1.6 | 1.6 | 1.7 | 1.7 |

Proposal 2:  RSD requirements for PC1.5 with 2Tx adopt following values

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

Proposal 3:  RSD requirements for PC1 with 1Tx adopt following values

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 3 | 3 | 3 | 3 |
| n255 | 4 | 4 | 4 | 4 |
| n254 | 3 | 3 | 3 |  |

- Reference

[1] R4-2505113, WF on HPUE requirements for NR-NTN, Samsung, RAN4#114bis.