# 3GPP TSG-RAN WG4 Meeting #115	R4-2507324
Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.10.3.3.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for IoT-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with IoT NTN operating in IoT-NTN bands were discussed and some agreements are approved in WF [1].

This contribution further discussed the UE Tx RF requirements for IoT NTN HPUE.

- Discussion

In last meeting, it was agreed that

- Use TN PC2 MPR requirements for Category M1 for PC1 as starting point

And the ACLR for eMTC based Iot-NTN for PC1 was discussed as below options

- Option 1: Re-use PC3 ACLR 30dBc. (xiaomi, vivo)

- Option 2: 31dBc ACLR (MTK, Qualcomm)

- Option 3: 37dBc ACLR (OPPO)

If PC1 eMTC based IoT NTN reuse the TN PC2/PC3 MPR requirements, the ACLR for PC1 eMTC based IoT-NTN should keep the same with ACLR for LTE TN PC1, since the MPR  is related with the ACLR.

Proposal 1: Reuse eMTC TN PC1 37dB ACLR for eMTC based Iot-NTN for PC1.

Proposal 2: Reuse eMTC TN PC2/PC3 MPR requirements for eMTC based Iot-NTN for PC1.

About PC1 NB-Iot based Iot-NTN, it agreed it can be applied to both handheld and non-handheld. And the open issue needs further discuss:

  - FFS whether to limit to the use case with single tone and a limited number of tones

Based on companies’ test results, the output power can’t reach up to 31dBm for PC1, and the MPR for PC3 NB-Iot based Iot-NTN is defined for 1tone, 3tones, 6tones and 12tones separately. To make PC1 NB-Iot based Iot-NTN and apply to handheld UE, the maximum transmission bandwidth could be limited to 6tones

Proposal 3: Limit that the maximum transmission bandwidth for UE is 6 tones for 15kHz SCS for PC1 NB-IoT based IoT-NTN.

- Conclusion

In this paper we give further discussion on the TX requirement for IoT NTN HPUE. The proposals are as following:

Proposal 1: Reuse eMTC TN PC1 37dB ACLR for eMTC based Iot-NTN for PC1.

Proposal 2: Reuse eMTC TN PC2/PC3 MPR requirements for eMTC based Iot-NTN for PC1.

Proposal 3: Limit that the maximum transmission bandwidth for UE is 6 tones for 15kHz SCS for PC1 NB-IoT based IoT-NTN.

- Reference

[1] R4-2505114, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114bis.