3GPP TSG-RAN WG4 Meeting #115	R4-2507355

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	Discussion on requirements for (e)RedCap support in NR NTN

Agenda item:	7.31.5.1

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_Ph3). The most recent version of the WID can be found in [2].

In this document we provide our views on the 5th objective, namely the support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems, related to the RAN4 RRM impact considering the open issues identified in the latest WF from RAN4 #114 [3] and further detailed in the Topic summary [4].

Discussion

In the present document we will discuss the open issues presented in the WF [3], as listed below:

- Issue 1-1-1: SDT requirements

- Issue 1-1-2: MDT requirements

- Issue 1-2-1-1a: The proximity distance between two SMTCs

- Issue 1-2-3: Other features

- Issue 3-1-1: General HD-FDD related impact

- Issue 3-2-2: Impact on other requirements due to HD-FDD

- Issue 4-1-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

General consideration on RRM requirements

RAN4 has discussed general aspects for RRM requirements for (e)RedCap UE with FR1-NTN.

SDT requirements

Following agreement was achieved for SDT requirements at RAN4 #114 [5].

#### Issue 1-1-2: SDT requirements Online agreement: Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS

At RAN4#114bis, the following way forward was provided by Moderator [3].

Issue 1-1-1: SDT requirements <Way Forward>: - Proposal 1: For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2. - The agreement reached in the last meeting only focus on RAN4’s requirements. - Proposal 2: RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

In fact, RAN4 has taken an earlier decision in Rel-18 to remove the RSRP change criterion for TA validation for UE in IoT-NTN performing CG-SDT in RRC_INACTIVE state, due to the limited near-far effect in NTN and the instability of the TA and its association with a validity timer and ephemeris.

The discussion was not progressed at RAN4 #114bis. Since the impact is only to RAN4 specification and for sake of progress, we can compromise to Proposal 1.

For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2.

  - The agreement reached in the last meeting only focus on RAN4’s requirements.

MDT requirements

Following agreement was achieved for MDT requirements [3].

RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

At RAN4#114bis, the following way forward was provided by Moderator [3].

Issue 1-1-2: MDT requirements <Way Forward>: - Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

In fact, RAN4 has taken an earlier decision to deprioritize MDT requirements for (e)RedCap in FR1 NTN. Though, we understand the concern raised in [6]. Moreover, changes to TS 38.133 are fairly small, as only applicability to (e)RedCap UE has to be added in the existing clause 4.3C (RRC_IDLE) and clause 5.3C (RRC_INACTIVE) for UE’s supporting MDT. Hence RAN4 should agree to specify MDT requirements for (e)RedCap UE supporting MDT. In addition, RAN4 can discuss whether to send LS to RAN2 / RAN3 to inform them on the taken decision.

RAN4 to specify MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

Proximity distance between two SMTCs

Following agreement on parallel SMTCs was achieved at RAN4 #114bis [3].

##### Issue 1-2-1-1: Feature 25-1(parallelSMTC) Online agreement Agreement: - For Feature 25-1 (parallelSMTC), not differentiate 1Rx and 2Rx RedCap UEs for this UE capability. - RedCap UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier - ‘Parallel’ means the SMTCs are TDM with proximity distance.

The determination of the proximity distance between two SMTC occasions was further discussed, and following way forward was proposed by Moderator [3].

##### Issue 1-2-1-1a: The proximity distance between two SMTCs <Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

For normal NTN UE (Rel-17 or beyond), the proximity distance is 4ms, as specified in clause 9.1C.9.3 in TS 38.133. For RedCap UE with FR1-NTN, a relaxation to 5ms is under discussion due to the reduced processing capability of the device. However, this will unnecessarily restrict network’s flexibility in organizing SMTCs. For instance, if 2 SMTCs are configured with 5ms proximity distance, they can be received by normal NTN UE but not by RedCap UE with FR1-NTN requiring a proximity distance larger than 5ms. This is a drawback when penetration of RedCap UEs with FR1-NTN increases, so the NW needs to take into account different capability of handling collisions for normal NTN UE and RedCap UE with FR1-NTN related to colliding SMTC configurations. The relaxation to 5ms between SMTCs is justified in case the RedCap UE receives two satellites in one SMTC and hence longer processing time is required due to presence of different receiver chains, so processing time after receiving both satellites in the SMTC occasion is longer. Thereagainst, if the RedCap UE is merely receiving one satellite with a single receiver chain, processing time is shorter and hence no relaxation of the 4ms proximity distance is justified. In short, we propose to distinguish two scenarios:

- Scenario A: RedCap UE with FR1-NTN is configured with 2 SMTCs and is configured to measure 1 satellite in each of the SMTC occasions. Then the proximity distance between 2 SMTC occasions is Y=4ms.

- Scenario B: RedCap UE with FR1-NTN is configured with 2 SMTCs and is configured to measure 2 satellites in at least one of the SMTC occasions. Then the proximity distance between 2 SMTC occasions is relaxed to Y=5ms.

RAN4 to specify for 1Rx and 2Rx RedCap UE with FR1-NTN, that proximity distance between two SMTCs configured on a SAN carrier is defined as:

- i) 4ms in case UE is configured to measure 1 satellite in each of the SMTC occasions.

- ii) 5ms in case UE is configured to measure 2 satellites in at least one of the SMTC occasions.

Other features

Following issue was discussed and agreed [3].

#### Issue 1-2-3: Other features Online agreement Agreement: - SMTC adjustment in idle/inactive is kept [mandatory] for (e)RedCap UEs with NTN. - From RAN4 perspective, location-based measurement report trigger is kept optional for (e)RedCap UEs with NTN.

There is an ongoing discussion whether the SMTC adjustment can be considered a mandatory feature for NTN UEs in idle/inactive mode. In fact, the NTN support described in TS 38.306 does not require the SMTC adjustment to be mandatorily supported, and in TS 38.300 the following is specified:

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. A UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information in SIB19.

The highlighted “can” on the box above is our own, but it means that the UE was not mandated to adjust the SMTC window based on its location and ephemeris information. However, the requirements are made transparent to UE implementation, i.e., whether the UE implements the shifting SMTC or not, this does not impact the applicability of the requirements. In NTN, the requirements are applicable considering that a UE is adjusting its SMTCs, but it does not preclude other UE implementations, as long as the UE comply with the idle/inactive mode mobility requirements. The same principle should be valid for RedCap over NTN

Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

Impact of HD-FDD

General HD-FDD related impact

At RAN4 #114bis, following was agreed or left open on general HD-FDD related impact [3].

#### Issue 3-1-1: General HD-FDD related impact Online agreement Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

TS 38.133, clause 8.1B.6 specifies the requirements for L1 indication for RLM for HD-FDD device in TN.

When DRX is not used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10 ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 8.1B.3 if the RLM-RS resource is CSI-RS, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. In case DRX is used for HD-FDD RedCap UEs, TIndication_interval,RedCap is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX_cycle_length is greater than 320ms, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

TS 38.133, clause 8.5B.4 specifies the requirements for L1 indication for LR for HD-FDD device in TN.

When DRX is not used, TIndication_interval_BFD_Redcap is max(2ms, TSSB-RS,M) ) or max(2ms, TCSI-RS,M), where TSSB-RS,M and TCSI-RS,M is the shortest periodicity of all RS resources in set q0 for the accessed cell, corresponding to either the shortest periodicity of the SSB in the set q0 or CSI-RS resource in the set q0. When DRX is used, for SSB based link quality measurement, - TIndication_interval_BFD_Redcap = Max(1.5 × DRX_cycle_length, 1.5 × TSSB-RS,M), if DRX_cycle_length ≤ 320 ms, - TIndication_interval_BFD_Redcap = DRX_cycle_length, if DRX_cycle_length > 320 ms. When DRX is used, for CSI-RS based link quality measurement, - TIndication_interval_BFD_Redcap = Max(1.5 × DRX_cycle_length, 1.5 × TCSI-RS,M), if DRX_cycle_length ≤ 320 ms, - TIndication_interval_BFD_Redcap = DRX_cycle_length, if DRX_cycle_length > 320 ms. For HD-FDD UE, the above conditions and requirements TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period.

The above requirements for DRX/no DRX on the availability of the RLM-RS/BFD-RS in the indication period should also apply to an HD-FDD RedCap device with FR1-NTN.

RAN4 to reuse the L1 indication requirements specified for RedCap HD-FDD device in TN, in DRX/no DRX for RLM in clause 8.1B.6 and for LR in clause 8.5B.4, for RedCap HD-FDD device in FR1-NTN.

Impact on other requirements due to HD-FDD

At RAN4 #114bis, following way forward was proposed by Moderator [3].

#### Issue 3-2-2: The impact on other requirements due to HD-FDD <Way Forward>: - Proposal 1: Clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected. - Proposal 2: For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured. - Proposal 3: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

Requirements for scheduling availability for HD-FDD RedCap device in TN

- for intra-frequency measurements are specified in clause 9.2B.5.3.4,

- for inter-frequency measurements are specified in clauses 9.3B.7.3 and 9.3B.7.3.4,

- for L1-RSRP measurements are specified in clauses 9.5B.6.1 and 9.5B.6.2.

Above requirements apply for HD-FDD RedCap UE with CBD evaluation prioritized over UL transmission. The same should apply for HD-FDD RedCap device with FR1-NTN.

RAN4 to reuse the following scheduling availability requirements specified for RedCap HD-FDD device in TN for RedCap HD-FDD device in FR1-NTN.

  - for intra-frequency measurements are specified in clause 9.2B.5.3.4,

  - for inter-frequency measurements are specified in clauses 9.3B.7.3 and 9.3B.7.3.4,

  - for L1-RSRP measurements are specified in clauses 9.5B.6.1 and 9.5B.6.2.

Others

UE capabilities for (e)RedCap UE with FR1-NTN bands

Following way forward was proposed by Moderator [3].

#### Issue 4-1-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands? <Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: RAN4 to not discuss or conclude on UE capability signaling and structure for RedCap NTN, unless specific requests are received from other working groups via LSs.

Although we were the proponents of the clarification above, it came to our attention that RAN1 is implicitly adding this discussion on their current feature list discussion [7]. Currently the RedCap over NTN feature group is supported as follows:

| 65. NR_NTN_Ph3 | 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | [1. Support handling collision Case 3] [2. Support handling collision Case 4] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state | FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | FFS | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FDD only Note: Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The highlighted part indicates that the current working assumption to be confirmed in RAN1 is that a UE that supports the feature group for RedCap operations over NTN must support FG 26 (NR NTN) and FG 28 (RedCap).

RAN1 working assumption is that the UE that supports NR NTN and RedCap shall support RedCap over NTN.

Therefore, we propose to wait for RAN1 to confirm the current working assumption and do not discuss the issue in RAN4 unless needed.

RAN4 to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN).

Conclusion

This paper has analysed the open issues provided in the latest WF for this WI [3], and the following set of observations and proposals are made:

- For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2, as the agreement reached in the last meeting only focus on RAN4’s requirements.

RAN4 to specify MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

RAN4 to specify for 1Rx and 2Rx RedCap UE with FR1-NTN, that proximity distance between two SMTCs configured on a SAN carrier is defined as:

- i) 4ms in case UE is configured to measure 1 satellite in each of the SMTC occasions.

- ii) 5ms in case UE is configured to measure 2 satellites in at least one of the SMTC occasions.

Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

RAN4 to reuse the L1 indication requirements specified for RedCap HD-FDD device in TN, in DRX/no DRX for RLM in clause 8.1B.6 and for LR in clause 8.5B.4, for RedCap HD-FDD device in FR1-NTN.

RAN4 to reuse the following scheduling availability requirements specified for RedCap HD-FDD device in TN for RedCap HD-FDD device in FR1-NTN.

  - for intra-frequency measurements are specified in clause 9.2B.5.3.4,

  - for inter-frequency measurements are specified in clauses 9.3B.7.3 and 9.3B.7.3.4,

  - for L1-RSRP measurements are specified in clauses 9.5B.6.1 and 9.5B.6.2.

- RAN1 working assumption is that the UE that supports NR NTN and RedCap shall support RedCap over NTN.

RAN4 to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN).

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #114bis

- R4-2503631, Topic summary for [114bis][221] NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #114bis

- R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #114

- R4-2503385, Discussion on the RRM requirement of (e)Redcap UE for NTN phase3, CMCC

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.), RAN WG1 #120bis, Wuhan, China, April 7th – 11th, 2025