3GPP TSG-RAN WG4 Meeting #115	R4-2507359

Malta, MT, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Performance parameters for Less than 5 MHz test cases in NTN

Agenda item:	7.10.4.5

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

Throughout the first meetings in this WI good progress was achieved by reusing most of the requirements from the less than 5 MHz WI for terrestrial networks with the necessary adaptation for NTN. The current paper aims at discussing the performance requirements for less than 5 MHz operation.

Discussion

The extensiveness of the list of test case that are applicable for NTN UEs also depend whether the UE supports or not terrestrial coverage:

- If UEs support NTN only, the full list of NTN test cases (full clause A.14) shall be supported according to clause A.3.36.2

- If UE supports both, NTN + TN, only a complementary list is expected to be supported (Table A.3.36.2-1)

For less than 5 MHz operation, this principle shall be maintained, but also, it is important to remember that the list of test cases for less than 5 MHz operation is a complementary list: only UEs that support his feature (i.e. only UEs that support support3MHz-ChannelBW-Symmetric-r18) are expected to pass this test. In order to avoid ambiguous interpretation about the applicability of the test cases, this has to be clarified in the NTN principles:

Clarify in clause A.3.36.2 that only UEs the additional list of test cases that are applicable for NTN UEs that support the less than 5 MHz feature

Approve the text proposal in Annex A.

Test Configuration

The detailed configuration for each test case was left FFS in the previous WF. But as a baseline, all test configurations shall re-use existent channel configuration for less than 5 MHz tests. Therefore, we propose:

Unless a special case is identified, the following channel configurations shall be reused for the NTN test cases in less than 5 MHz

  - SR.1.2 for PDSCH

  - CR.1.3 for RMSI Coreset

  - SSB.13 for SSB

Conclusion

In this paper, we provided our views on the performance test cases of Less than 5 MHz operation in NTN by means of the following proposals:

Proposal 1: Clarify in clause A.3.36.2 that only UEs the additional list of test cases that are applicable for NTN UEs that support the less than 5 MHz feature

Proposal 2: Approve the text proposal in Annex A.

Proposal 3: Unless a special case is identified, the following channel configurations shall be reused for the NTN test cases in less than 5 MHz

a.	SR.1.2 for PDSCH

b.   CR.1.3 for RMSI Coreset

c.	SSB.13 for SSB

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- RP-241985, " Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105

- R4-2504896, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi), Wuhan, Hubei, China, 07th – 11th April, 2025

- Annex A – Text Proposal:

### A.3.36.2	Principle of testing different RRM requirements

In annex A, RRM test cases related to satellite access are defined for all applicable RRM requirements. The testing principle for these test cases is as follows:

-	A UE capable of NTN only is required to pass all the test cases defined in clause A.14.

-	A UE capable of both TN and NTN is required to pass the test cases for NTN specific requirements in table A.3.36.2-1.

Table A.3.36.2-1: Test cases for NTN specific requirements

| Clause | Test case slogan |
| --- | --- |
| A.14.1.2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.3 | Time-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.4 | Location-based cell reselection to FR1 intra-frequency NR cell |
| A.14.1.7 | Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.8 | Time-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.1.9 | Location-based Cell reselection to FR1 inter-frequency NR satellite access case |
| A.14.2.1.3 | Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.4 | Inter-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.5 | Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.2.1.6 | Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.3.1.1 | NR UE Transmit Timing Test for FR1 |
| A.14.5.1.1 | SA event triggered reporting tests without gap under non-DRX |
| A.14.5.1.2 | SA event triggered reporting tests without gap under DRX |
| A.14.5.1.3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.14.5.1.4 | SA event triggered reporting tests with single measurement gap under non-DRX for satellite access |
| A.14.5.1.5 | SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access |
| A.14.5.1.6 | SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access |
| A.14.5.2.1 | Event triggered reporting test without gap under non-DRX |
| A.14.5.2.2 | Event triggered reporting tests without gap under DRX |
| A.14.6.3.1 | SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.3.2 | SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell |
| A.14.6.4.1 | SSB based L1-RSRP measurement |
| A.14.6.4.2 | CSI-RS based L1-RSRP measurement on resource set with repetition off |

In addition to Table A.3.36.2-1, NTN UEs that support the capability support3MHz-ChannelBW-Symmetric-r18 shall also support the list of test cases provided in Table A.3.36.2-X:

Table A.3.36.2-1: Test cases for NTN UEs that support support3MHz-ChannelBW-Symmetric-r18

| Clause | Test case slogan |
| --- | --- |
| TBD | - HO-1: Intra-frequency HO, [unknown cell] |
| TBD | CHO-1: [Inter-frequency CHO], time based with L3 measurement |
| TBD | SatSwitch-1: Hard switch |
| TBD | RLM-1: RLM OOS for 15RB |
| TBD | RLM-2: RLM IS for 15RB |
| TBD | BFR-1: BFR |
| TBD | MR-1: Intra-frequency with SSB index reading |
| TBD | [MR-2: Inter-frequency without SSB index reading] |