3GPP TSG-RAN4 Meeting #115	R4-2507365

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2819 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for SL intra-band CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Oppo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_intraB_CA_ITS-Core |  |  |  |  |  | Date: |  |  | 2025-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the endorsed draft CRs, and agreed WF/TR. Introduce UE RF requirements for PC2 SL intra-band contiguous CA Introduce UE RF requirements for PC3/PC2 SL intra-band non-contiguous CA - Endorsed draft CRs - R4-2401687, R4-2402161 - Agreed WF - R4-2505133, R4-2417090 - Agreed TR - TR 38.787 v.0.1.0 For endorsed R4-2401687, - Typo in Table 6.2E.2.1A.1-3 is corrected with agreement in R4-2417090. Modulation; MPR for bandwidth class B(dB). inner; outer. CP-OFDM; QPSK; ≤ 2.5 3.5; ≤ 5.5 7.0. 16QAM; ≤ 3.0 3.5; ≤ 5.5 7.0. 64QAM; ≤ 4.0 4.5; ≤ 5.5 7.0. 256QAM; ≤ 7.0 7.5; ≤ 7.0 8.0. For intra-band non-contiguous SL CA, remove [ ] from PC3/PC2 PSSCH/PSCCH MPR for -13dBm/MHz |
| Summary of change: |  | - Introduce UE RF requirements for PC2 SL intra-band contiguous CA - MOP/MPR - Introduce UE RF requirements for PC3 & PC2 intra-band non-contiguous CA - Operating band, CBW, SL CA configuration and BW combination - Tx requirements - MOP/MPR/A-MPR/Configured transmitted power - Output power dynamics - Transmit signal quality - Output RF spectrum emissions - Rx requirements - REFSENS - Maximum input level - ACS - IBB/OBB - Spurious response - Intermodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For SL intra-band contiguous CA, PC2 UE RF requirements are missing. For SL intra-band non-contiguous CA, PC3/PC2 UE RF requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2E.1A, 5.3E.1A, 5.5E.1A, new 5.5E.1A.2 6.2E.1A, 6.2E.2.1A, new 6.2E.2.1A.1, new 6.2E.2.1A.2, new 6.2E.3.2A, 6.2E.4A, 6.3E.1.1A, 6.3E.2.1A, 6.3E.3.1A, 6.3E.4.1A, 6.4E.1.1A, 6.4E.2.2A, 6.4E.2.3A, 6.4E.2.4A, 6.5E.1.1A, 6.5E.2.2.1A, new 6.5E.2.3.1A, 6.5E.2.4.1A, 6.5E.3.1A, 6.5E.3.2A, new 6.5E.3.4A, 7.3E.2A.1, 7.4E.1A, 7.5E.1A, 7.6E.2.1A, 7.6E.3.1A, 7.7E.1A, 7.8E.2.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

5.2E.1A	Sidelink CA operating bands

For NR sidelink intra-band CA operation is designed to operate in the operating bands in FR1 defined in Table 5.2E.1A-1 and Table 5.2E.1A-2.

Table 5.2E.1A-1 Intra-band contiguous SL CA operating bands in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SLCA_n47 | n47 | PC5 |

Table 5.2E.1A-2 Intra-band non-contiguous SL CA operating bands in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SLCA_n47(*) | n47 | PC5 |
| NOTE 1: The notation SLCA_nX(*) in this table indicates SL intra-band non-contiguous CA for band nX. The configurations for each band are in 5.5E.1A.2. |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

5.3E.1A	Channel bandwidth for Sidelink CA

For sidelink intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single ITS operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets specified in clause 5.5E.1A.1.

For sidelink intra-band non-contiguous carrier aggregation, a carrier aggregation configuration is a single ITS operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets specified in clause 5.5E.1A.2

The sidelink intra-band carrier aggregation bandwidth class follows Table 5.3A.5-1. For each carrier aggregation configuration, requirements are specified for all aggregated channel bandwidths contained in a bandwidth combination set.

< END OF CHANGE #2 >

< START OF CHANGE #3 >

5.5E.1A	Configurations for Sidelink CA

For NR SL CA operation, the SL CA channel bandwidths for intra-band contiguous are specified in clause 5.5E.1A.1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path.

5.5E.1A.1	Configurations for Sidelink intra-band contiguous CA

Table 5.5E.1A.1-1 NR SL CA configurations and bandwidth combination set for SL intra-band contiguous CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SLCA_n47B | SLCA_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | [20] | [20,30] |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

5.5E.1A.2	Configurations for Sidelink intra-band non-contiguous CA

Table 5.5E.1A.1-2 NR SL CA configurations and bandwidth combination set for SL intra-band non-contiguous CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SLCA_n47(2A) | SLCA_n47(2A) | 10 | 10,20 |  |  | 30 | 0 |

< END OF CHANGE #3 >

< START OF CHANGE #4 >

6.2E.1A	UE maximum output power for Sidelink CA

For SL intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2E.1A-1.

For SL intra-band non-contiguous carrier aggregation with two sidelink carriers, the maximum output power is specified in Table 6.2E.1A-2.

Table 6.2E.1A-1: UE Power Class for SL intra-band contiguous CA

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLCA _n47B |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

Table 6.2E.1A-2: UE Power Class for SL intra-band non-contiguous CA

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLCA _n47(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

< END OF CHANGE #4 >

< START OF CHANGE #5 >

6.2E.2	UE maximum output power reduction for V2X

6.2E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, this clause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

6.2E.2.1A	MPR for sidelink CA

6.2E.2.1A.1	MPR for sidelink intra-band contiguous CA

For SL intra-band contiguous CA of PSCCH and PSSCH simultaneous transmission with contiguous RB allocation, the allowed MPR for the maximum output power are specified in Table 6.2E.2.1A.1-1 for UE power class 3. The MPR with contiguous RB allocation is specified in Table 6.2E.2.1A.1-2 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2E.2.1A.1-3 for power class 2 with TxD supported.

Table 6.2E.2.1A.1-1: MPR for power class 3 SL CA with contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.5 | ≤ 7.0 |

Table 6.2E.2.1A.1-2: MPR for power class 2 SL CA with contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 5.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64QAM | ≤ 4.0 | ≤ 5.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Table 6.2E.2.1A.1-3: MPR for power class 2 SL CA with contiguous RB allocation with 2Tx

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 7.0 |
|  | 16QAM | ≤ 3.5 | ≤ 7.0 |
|  | 64QAM | ≤ 4.5 | ≤ 7.0 |
|  | 256QAM | ≤ 7.5 | ≤ 8.0 |

The contiguous allocation rule of inner and outer for SL intra-band contiguous CA refers to that for NR intra-band contiguous CA in 6.2A.2.1 in TS38.101-1.

For SL intra-band contiguous CA of PSCCH and PSSCH simultaneous transmission with non-contiguous RB allocation, the allowed MPR for the maximum output power are specified in Table 6.2E.2.1A.1-4 for UE power class 3. The MPR with non-contiguous RB allocation is specified in Table 6.2E.2.1A.1-5 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2E.2.1A.1-6 for power class 2 with TxD supported.

Table 6.2E.2.1A.1-4: MPR for power class 3 SL CA with non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

Table 6.2E.2.1A.1-5: MPR for power class 2 SL CA with non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 7.0 | ≤ 10.0 |
|  | 16QAM | ≤ 4.0 | ≤ 7.0 | ≤ 10.0 |
|  | 64QAM | ≤ 5.5 | ≤ 7.0 | ≤ 10.0 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 10.0 |

Table 6.2E.2.1A.1-6: MPR for power class 2 SL CA with non-contiguous RB allocation with 2Tx

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16QAM | ≤ 4.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 | ≤ 11.5 |
|  | 256QAM | ≤ 9.5 | ≤ 9.5 | ≤ 11.5 |

The non-contiguous allocation rule of inner, outer1, and outer2 for SL intra-band contiguous CA refers to that for NR intra-band contiguous CA in 6.2A.2.1 in TS38.101-1.

For SL intra-band CA of PSFCH with single RB transmission on each carrier, the required MPR are specified as Table 6.2E.2.1A.1-7 for UE power class 3. The MPR is specified in Table 6.2E.2.1A.1-8 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR is specified in Table 6.2E.2.1A.1-9 for power class 2 with TxD supported.

Table 6.2E.2.1A.1-7: PSFCH MPR for power class 3 intra-band SL CA

| MPR (dB) | R (Ratio of gap bandwidth) |
| --- | --- |
| ≤ 2.5 | 0< R ≤ 0.3 |
| ≤ 7.5 | 0.3< R ≤ 0.5 |
| ≤ 12 | 0.5< R ≤ 1 |

Table 6.2E.2.1A.1-8: PSFCH MPR for power class 2 intra-band SL CA

| MPR (dB) | R (Ratio of gap bandwidth) |
| --- | --- |
| ≤ 5.0 | 0< R ≤ 0.3 |
| ≤ 10.0 | 0.3< R ≤ 0.5 |
| ≤ 14.0 | 0.5< R ≤ 1 |

Table 6.2E.2.1A.1-9: PSFCH MPR for power class 2 intra-band SL CA with 2Tx

| MPR (dB) | R (Ratio of gap bandwidth) |
| --- | --- |
| ≤ 6.0 | 0< R ≤ 0.3 |
| ≤ 11.0 | 0.3< R ≤ 0.5 |
| ≤ 15.0 | 0.5< R ≤ 1 |

Where,

R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier.

When single S-SSB is transmitted on intra-band contiguous carriers, required MPR for single cell V2X in Table 6.2E.2.2-1 and Table 6.2E.2.2-2 is reused for UE power class 3 and for UE power class 2 respectively. For two S-SSB transmissions on intra-band contiguous carriers, the required MPR are specified as Table 6.2E.2.1A.1-10 for UE power class 3. The MPR is specified in Table 6.2E.2.1A.1-11 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR is specified in Table 6.2E.2.1A.1-12 for power class 2 with TxD supported.

Table 6.2E.2.1A.1-10: S-SSB MPR for power class 3 intra-band SL CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 3.5 | ≤ 9.0 | ≤ 13.0 |

Table 6.2E.2.1A.1-11: S-SSB MPR for power class 2 intra-band SL CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |

Table 6.2E.2.1A.1-12: S-SSB MPR for power class 2 intra-band SL CA with 2Tx

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |

6.2E.2.1A.2	MPR for sidelink intra-band non-contiguous CA

6.2E.2.1A.2.0	General

For SL intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determines the MPR type as follows:

For UE indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0)

MPR defined in Clause 6.2E.2.2 for PC3 and PC2 UE.

Else If AND (FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2E.2.1A.2.1 to meet -13dBm/MHz for PC3 and PC2 UE.

Else

MPR defined in Clause 6.2E.2.1A.2.1 to meet -30dBm/MHz for PC3 and PC2 UE.

For UE without indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0 )

MPR defined in Clause 6.2E.2.2 for PC3 and PC2 UE.

Else If AND (FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2E.2.1A.2.2 to meet -13dBm/MHz for PC3 and PC2 UE.

Else

MPR defined in Clause 6.2E.2.1A.2.2 to meet -30dBm/MHz for PC3 and PC2 UE.

where

-	LCRB1 is for CC1, which is the component carrier with lower frequency

-	LCRB2 is for CC2, which is the component carrier with higher frequency

-	B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

MPRs in section 6.2E.2.1A.2.2 are applicable only when the Gap between the component carriers is ≤ the overall channel bandwidth summed across all the component carriers.

The definition of the gap is between the component carriers in a spectrum that is not part of any configured component carrier that is located in between the lowest edge of the component carrier with higher center frequency and the highest edge of the component carrier with center frequency that is located lower in frequency.

6.2E.2.1A.2.1	MPR with indicating dualPA-Architecture supported

MPR in this clause is for SL intra-band non-contiguous CA for UEs indicating IE dualPA-Architecture supported.

The allowed maximum output power reduction for PSSCH/PSCCH is defined in Table 6.2E.2.1A.2.1-1 for power class 3 and Table 6.2E.2.1A.2.1-2 for power class 2.

Table 6.2E.2.1A.2.1-1: PSSCH/PSCCH MPR for power class 3 intra-band SL CA

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ 6.5 | ≤ 13.5 |
| 5.04 ≤ B < 10.08 |  | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

Table 6.2E.2.1A.2.1-2: PSSCH/PSCCH MPR for power class 2 intra-band SL CA

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ 7.0 | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

The allowed maximum output power reduction for PSFCH is defined in Table 6.2E.2.1A.2.1-3 for power class 3 and Table 6.2E.2.1A.2.1-4 for power class 2.

Table 6.2E.2.1A.2.1-3: PSFCH MPR for power class 3 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 9.0 | ≤ 16.0 |

Table 6.2E.2.1A.2.1-4: PSFCH MPR for power class 2 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 9.5 | ≤ 16.5 |

The allowed maximum output power reduction for S-SSB is defined in Table 6.2E.2.1A.2.1-5 for power class 3 and Table 6.2E.2.1A.2.1-6 for power class 2.

Table 6.2E.2.1A.2.1-5: S-SSB MPR for power class 3 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 8.5 | ≤ 15.0 |

Table 6.2E.2.1A.2.1-6: S-SSB MPR for power class 2 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 9.0 | ≤ 17.0 |

6.2E.2.1A.2.2	MPR without indicating dualPA-Architecture supported

MPR in this clause is for SL intra-band non-contiguous CA for UEs without indicating IE dualPA-Architecture supported.

The allowed maximum output power reduction for PSSCH/PSCCH is defined in Table 6.2E.2.1A.2.2-1 for power class 3 and Table 6.2E.2.1A.2.2-2 for power class 2.

Table 6.2E.2.1A.2.2-1: PSSCH/PSCCH MPR for power class 3 intra-band SL CA

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ 10.5 | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤ 9.5 | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

Table 6.2E.2.1A.2.2-2: PSSCH/PSCCH MPR for power class 2 intra-band SL CA

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B < 5.04 | ≤ 11.5 | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ 11.0 | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

The allowed maximum output power reduction for PSFCH is defined in Table 6.2E.2.1A.2.2-3 for power class 3 and Table 6.2E.2.1A.2.2-4 for power class 2.

Table 6.2E.2.1A.2.2-3: PSFCH MPR for power class 3 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 12.0 | ≤ 18.0 |

Table 6.2E.2.1A.2.2-4: PSFCH MPR for power class 2 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 15.0 | ≤ 20.0 |

The allowed maximum output power reduction for S-SSB is defined in Table 6.2E.2.1A.2.2-5 for power class 3 and Table 6.2E.2.1A.2.2-6 for power class 2.

Table 6.2E.2.1A.2.2-5: S-SSB MPR for power class 3 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 13.0 | ≤ 21.0 |

Table 6.2E.2.1A.2.2-6: S-SSB MPR for power class 2 intra-band SL CA

| MPR (dB) |  |
| --- | --- |
| -13dBm/MHz | -30dBm/MHz |
| ≤ 14.0 | ≤ 23.0 |

< END OF CHANGE #5 >

< START OF CHANGE #6 >

6.2E.3	UE additional maximum output power reduction for V2X

6.2E.3.1	General

For the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in clause 6.2E.2 and A-MPR requirements specified in current clause.

Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Outer and inner allocation notation used in clause 6.2E.3 is defined in clause 6.2E.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2E.3.1-1: Additional Maximum Power Reduction (A-MPR) for PC3 NR V2X

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2E.1-1 | 10, 20, 30, 40 | Table 5.3.2-1 | N/A |
| NS_06 | 6.5.2.3.4 (A-SEM) | n14 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_33 | 6.5E.2.3.1 (A-SEM) 6.5E.3.4 (A-SE) | n47 | 10 | Clause 6.2E.3.2 |  |
| NS_52 | 6.5E.2.3.2 (A-SEM) | n47 | 40 | Clause 6.2E.3.3 |  |

Table 6.2E.3.1-2: Mapping of network signaling label

| NR V2X operating bands | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n142 | NS_01 | NS_06 |  |  |  |  |  |  |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n47 | NS_01 | NS_33 | NS_52 |  |  |  |  |  |
| NOTE 1: [additionalSpectrumEmission] corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: For the NR PS UE in n14, same A-MPR shall be applied for PC1 PS UE since PC1 PS UE for Band n14 is not targeted for smartphone form factor. |  |  |  |  |  |  |  |  |

For UE with two transmit antenna connectors, the A-MPR values specified in clause 6.2E.3.2 and 6.2E.3.3 shall apply to the maximum output power specified in Table 6.2E.1.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2E.4 apply.

6.2E.3.2	A-MPR for V2X UE by NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2E.3.2-1, 6.2E.3.2-2 is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in annex I in [11]. The A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector.

For the contiguous PSSCH and PSCCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements specified in Table 6.2E.3.2-1 and 6.2E.3.2-2 for power class 3. And A-MPR requirements specified in Table 6.2E.3.2-2a and 6.2E.3.2-2b for power class 2 are allowed for NR V2X UE.

Table 6.2E.3.2-1: PC3 A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz, and 5920MHz)

| Carrier frequency [MHz] | Region |  | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | LCRB* 12*SCS [MHz] | RBstart*12*SCS [MHz] | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 1.8 and ≤ 2.7 | 0 | ≤ 24.0 |  |  |
|  |  | ≥ 0.18 and ≤ 0.54 | ≤19.0 |  |  |
|  |  | ≥ 4.68 | ≤ 6.0 |  |  |
|  | ≥ 1.8 and ≤ 3.6 | ≥ 2.16 and ≤ 2.52 | ≤11.0 |  |  |
|  |  | ≥ 2.7 and ≤ 3.42 | ≤ 9.5 |  |  |
|  |  | ≥ 3.6 and ≤ 4.5 | ≤ 8.0 |  |  |
|  | > 2.7 and < 4.5 | ≥ 4.5 | ≤ 8.0 |  |  |
|  | ≥ 1.8 and < 7.2 | ≥ 0.72 and ≤1.26 | ≤ 16.0 |  |  |
|  |  | ≥ 1.44 and ≤ 1.98 | ≤ 13.5 |  |  |
|  | ≥ 3.6 and < 7.2 | ≥ 0 and ≤ 0.54 | ≤ 22.0 |  |  |
|  | ≥ 4.32 and < 7.2 | ≥ 2.88 and ≤ 3.78 | ≤ 9.5 |  |  |
|  |  | ≥ 3.96 and ≤ 4.86 | ≤ 8.0 |  |  |
|  | ≥ 4.32 and ≤ 7.2 | ≥ 2.16 and ≤ 2.7 | ≤ 12.0 |  |  |
|  | 7.2 and 8.1 | ≥ 0 and ≤ 0.18 | ≤ 19.0 |  |  |
|  |  | ≥ 0.36 and ≤ 0.9 | ≤ 16.0 |  |  |
|  |  | ≥ 1.08 and ≤ 1.98 | ≤ 13.5 |  |  |
|  | > 8.1 | ≥ 0 | ≤ 16.0 |  |  |
| Carrier frequency [MHz] | LCRB* 12*SCS [MHz] | RBend*12*SCS [MHz] | QPSK/16QAM | 64QAM | 256QAM |
| 5920 | ≥ 1.8 and ≤ 2.7 | 9.36 | ≤ 24.0 |  |  |
|  |  | ≥ 8.82 and ≤ 9.18 | ≤19.0 |  |  |
|  |  | ≤ 4.68 | ≤ 6.0 |  |  |
|  | ≥ 1.8 and ≤ 3.6 | ≥ 6.84 and ≤ 7.2 | ≤11.0 |  |  |
|  |  | ≥ 5.94 and ≤ 6.66 | ≤ 9.5 |  |  |
|  |  | ≥ 4.86 and ≤ 5.76 | ≤ 8.0 |  |  |
|  | > 2.7 and < 4.5 | ≤ 4.86 | ≤ 8.0 |  |  |
|  | ≥ 1.8 and < 7.2 | ≥ 8.1 and ≤8.64 | ≤ 16.0 |  |  |
|  |  | ≥ 7.38 and ≤ 7.92 | ≤ 13.5 |  |  |
|  | ≥ 3.6 and < 7.2 | ≥ 8.82 and ≤ 9.36 | ≤ 22.0 |  |  |
|  | ≥ 4.32 and < 7.2 | ≥ 5.58 and ≤ 6.48 | ≤ 9.5 |  |  |
|  |  | ≥ 4.5 and ≤ 5.4 | ≤ 8.0 |  |  |
|  | ≥ 4.32 and ≤ 7.2 | ≥ 6.66 and ≤ 7.2 | ≤ 12.0 |  |  |
|  | 7.2 and 8.1 | ≥ 9.18 and ≤ 9.36 | ≤ 19.0 |  |  |
|  |  | ≥ 8.46 and ≤ 9.0 | ≤ 16.0 |  |  |
|  |  | ≥ 7.38 and ≤ 8.28 | ≤ 13.5 |  |  |
|  | > 8.1 | ≤ 9.36 | ≤ 16.0 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 6.2E.3.2-2: PC3 A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.2 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

Table 6.2E.3.2-2a: PC2 A-MPR for PSCCH/PSSCH by NS_33 (at Fc=5860MHz, and 5920MHz)

| Carrier frequency [MHz] | LCRB* 12*SCS [MHz] | RBstart*12*SCS [MHz] | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 1.8 and ≤ 2.7 | ≥ 0 and ≤ 0.18 | ≤ 24 |  |  |
|  |  | ≥ 0.36 and ≤ 0.54 | ≤ 22 |  |  |
|  |  | 0.72 | ≤ 20 |  |  |
|  | ≥ 1.8 and ≤ 4.5 | ≥ 0.9 and ≤ 1.26 | ≤ 17.5 |  |  |
|  | ≥ 1.8 and ≤ 5.4 | 1.8 | ≤ 16 |  |  |
|  | ≥ 1.8 | ≥ 1.44 and ≤ 1.62 | ≤ 16 |  |  |
|  |  | ≥ 1.98 and ≤ 2.52 | ≤ 14.5 |  |  |
|  |  | ≥ 2.7 and ≤ 3.42 | ≤ 13 |  |  |
|  |  | ≥ 3.6 and ≤ 4.32 | ≤ 11.5 |  |  |
|  |  | ≥ 4.5 and ≤ 5.22 | ≤ 10 |  |  |
|  |  | ≥ 5.4 | ≤ 8.5 |  |  |
|  | ≥ 3.6 and ≤ 4.32 | 0.18 | ≤ 22 |  |  |
|  | ≥ 3.6 and ≤ 5,4 | 0 | ≤ 22 |  |  |
|  |  | ≥ 0.36 and ≤ 0.54 | ≤ 20 |  |  |
|  |  | 0.72 | ≤ 17.5 |  |  |
|  | ≥ 25 and ≤ 40 | 0.18 | ≤ 20 |  |  |
|  | ≥ 5.4 | ≥ 0.9 and ≤ 1.26 | ≤ 16 |  |  |
|  | ≥ 6.48 | 0 | ≤ 20 |  |  |
|  |  | ≥ 0.36 and ≤ 0.72 | ≤ 17.5 |  |  |
|  |  | 1.8 | ≤ 14.5 |  |  |
|  | ≥ 8.1 | 0.18 | ≤ 17.5 |  |  |
| Carrier frequency [MHz] | LCRB* 12*SCS [MHz] | RBend*12*SCS [MHz] | A-MPR(dB) |  |  |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5920 | ≥ 1.8 and ≤ 2.7 | ≥ 9.18 and ≤ 9.36 | ≤ 24 |  |  |
|  |  | ≥ 8.82 and ≤ 9.0 | ≤ 22 |  |  |
|  |  | 8.64 | ≤ 20 |  |  |
|  | ≥ 1.8 and ≤ 4.5 | ≥ 8.1 and ≤ 8.46 | ≤ 17.5 |  |  |
|  | ≥ 1.8 and ≤ 5.4 | 7.56 | ≤ 16 |  |  |
|  | ≥ 1.8 | ≥ 7.74 and ≤ 7.92 | ≤ 16 |  |  |
|  |  | ≥ 6.84 and ≤ 7.38 | ≤ 14.5 |  |  |
|  |  | ≥ 5.94 and ≤ 6.66 | ≤ 13 |  |  |
|  |  | ≥ 5.04 and ≤ 5.76 | ≤ 11.5 |  |  |
|  |  | ≥ 4.14 and ≤ 4.86 | ≤ 10 |  |  |
|  |  | ≥ 3.96 | ≤ 8.5 |  |  |
|  | ≥ 3.6 and ≤ 4.32 | 9.18 | ≤ 22 |  |  |
|  | ≥ 3.6 and ≤ 5,4 | 9.36 | ≤ 22 |  |  |
|  |  | ≥ 8.82 and ≤ 9.0 | ≤ 20 |  |  |
|  |  | 8.64 | ≤ 17.5 |  |  |
|  | ≥ 25 and ≤ 40 | 9.18 | ≤ 20 |  |  |
|  | ≥ 5.4 | ≥ 8.1 and ≤ 8.46 | ≤ 16 |  |  |
|  | ≥ 6.48 | 9.36 | ≤ 20 |  |  |
|  |  | ≥ 8.64 and ≤ 9.0 | ≤ 17.5 |  |  |
|  |  | 7.56 | ≤ 14.5 |  |  |
|  | ≥ 8.1 | 9.18 | ≤ 17.5 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 6.2E.3.2-2b: PC2 A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 5870,5910 | outer | ≤ 8.5 |  |  | ≤ 8.5 |
|  | inner | ≤ 6.0 |  |  |  |
| 5880,5890,5900 | outer | ≤ 6.0 |  |  | ≤ 6.5 |
|  | inner | ≤ 3.5 |  | ≤ 4.5 |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.1 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

For the simultaneous PSFCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements specified in Table 6.2E.3.2-3 for power class 3 and in Table 6.2E.3.2-3a for power class 2.

Table 6.2E.3.2-3: PC3 A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860, 5920 | NRB =1 | 19.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

Table 6.2E.3.2-3a: PC2 A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.2 | 0.2≤ NGap / NRB < 0.4 | 0.4≤ NGap / NRB ≤ 1 |  |
| 10 | 5860, 5920 | NRB =1 | 25.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 16.5 | 12 | 20 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

For the S-SSB transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements specified in Table 6.2E.3.2-4 for power class 3 and in Table 6.2E.3.2-5 for power class 2.

Table 6.2E.3.2-4: PC3 A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |
| Carrier Frequency (MHz) | RBend * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| 5920 | ≥ 8.36 | ≤ 25 | 0.6 |
|  | ≥ 7.36 and < 8.36 | ≤ 19 |  |
|  | ≥ 6.12 and < 7.36 | ≤ 12 |  |
|  | ≥ 5.76 and < 6.12 | ≤ 10 |  |
|  | < 5.76 | ≤ 9 |  |

Table 6.2E.3.2-5: PC2 A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |
| Carrier Frequency (MHz) | RBend * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| 5920 | ≥ 8.36 | ≤ 25 | 0.6 |
|  | ≥ 7.36 and < 8.36 | ≤ 19 |  |
|  | ≥ 6.12 and < 7.36 | ≤ 12 |  |
|  | ≥ 5.76 and < 6.12 | ≤ 10 |  |
|  | < 5.76 | ≤ 9 |  |

6.2E.3.2A	A-MPR for sidelink CA by NS_33

6.2E.3.2A.1	A-MPR for sidelink intra-band non-contiguous CA

6.2E.3.2A.1.0	General

Table 6.2E.3.2A.1.0-1 specifies the additional requirements with their associated network or pre-configured signalling values and the allowed A-MPR and applicable NR SL CA band(s) for each SLCA_NC_NS value. For any NR SL CA band not listed in Table 6.2E.3.2A.1.0-2 the network or pre-configured signalling label SLCA_NC_NS_01 applies.

Table 6.2E.3.2A.1.0-1: Additional Maximum Power Reduction (A-MPR) for SL CA

| NR SL CA Network Signalling value | Requirements (clause) | NR Band | All applicable NR SL CA configuration | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| SLCA_NC_NS_01 | 6.5E.2.2.1A (SEM) 6.5E.3.1A (SE) | Table 5.2E.1A-1 | Table 5.5E.1A.1-2 | N/A |
| SLCA_NC_NS_33 | 6.5E.2.3.1A (A-SEM) 6.5E.3.4.2A (A-SE) | SLCA_n47 | SLCA_n47(2A) | Clause 6.2E.3.2A.1.1 |

The mapping of NR SL CA band numbers and values of the additionalSpectrumEmission to network or pre-configured signalling labels is specified in Table 6.2E.3.2A.1.0-2.

Table 6.2E.3.2A.1.0-2: Mapping of network or pre-configured signaling label

| NR SL CA operating bands | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| SLCA_n47 | SLCA_NC_NS_01 | SLCA_NC_NS_33 |  |  |  |  |  |  |
| NOTE 1: [additionalSpectrumEmission] corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2E.3.2A.1.1	A-MPR for SLCA_NC_NS_33 (SLCA_n47(2A))

6.2E.3.2A.1.1.0	General

For SL intra-band non-contiguous CA_n47(2A) and it receives SLCA_NC_NS_33, the UE determines the allowed Additional Maximum Power Reduction (A-MPR) for the maximum output power as specified in this clause. The A-MPR is specified into 2 types: A-MPR to meet -30dBm/MHz and -37dBm/100kHz. The UE determines the A-MPR type as follows:

For UE indicating dualPA-Architecture supported

If AND (FIM3,low_block,low > SEM-37/100kHz,low ,  FIM3,high_block,high < SEM-37/100kHz,high )

A-MPR defined for FIM3_Inner in Clause 6.2E.3.2A.1.1.1 for PC3 and PC2 UE.

Else

A-MPR defined for FIM3_Outer in Clause 6.2E.3.2A.1.1.1 for PC3 and PC2 UE.

For UE without indicating dualPA-Architecture supported

If AND (FIM3,low_block,low > SEM-37/100kHz,low ,  FIM3,high_block,high < SEM-37/100kHz,high)

A-MPR defined for FIM3_Inner in Clause 6.2E.3.2A.1.1.2 for PC3 and PC2 UE.

Else

A-	MPR defined for FIM3_Outer in Clause 6.2E.3.2A.1.1.2 for PC3 and PC2 UE.

where

-	LCRB1 is for CC1, which is the component carrier with lower frequency

-	LCRB2 is for CC2, which is the component carrier with higher frequency

-	B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-37/100kHz,low = Threshold frequency where lower spectral emission mask below the lower channel corresponds to -37dBm/100kHz, as specified in Clause 6.5E.2.3.1A.

-	SEM-37/100kHz,high = Threshold frequency where upper spectral emission mask above the upper channel corresponds to -37dBm /100kHz, as specified in Clause 6.5E.2.3.1A.

-    FIM3_Inner  is  “AND (FIM3,low_block,low > SEM-37/100kHz,low ,  FIM3,high_block,high < SEM-37/100kHz,high)”

-    FIM3_Outer  is  “AND (FIM3,low_block,low ≤  SEM-37/100kHz,low ,  FIM3,high_block,high ≥ SEM-37/100kHz,high)”

A-MPRs in section 6.2E.3.2A.1.1.2 are applicable only when the Gap between the component carriers is ≤ the overall channel bandwidth summed across all the component carriers.

The definition of the gap is between the component carriers in a spectrum that is not part of any configured component carrier that is located in between the lowest edge of the component carrier with higher center frequency and the highest edge of the component carrier with center frequency that is located lower in frequency.

6.2E.3.2A.1.1.1	A-MPR with indicating dualPA-Architecture supported

A-MPR in this clause is for SL intra-band non-contiguous CA for UEs indicating IE dualPA-Architecture supported.

The allowed additional maximum output power reduction for PSSCH/PSCCH is defined in Table 6.2E.3.2A.1.1.1-1 for power class 3 and Table 6.2E.3.2A.1.1.1-2 for power class 2.

Table 6.2E.3.2A.1.1.1-1: PSSCH/PSCCH A-MPR for power class 3 intra-band SL CA

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 13.5 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 12.5 | ≤ 17.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 13.0 | ≤ 14.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 10.5 | ≤ 12.5 |
|  | 10.08 ≤ B |  | ≤ 11.5 |

Table 6.2E.3.2A.1.1.1-2: PSSCH/PSCCH A-MPR for power class 2 intra-band SL CA

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 15.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.0 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 14.0 | ≤ 15.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 11.5 | ≤ 13.0 |
|  | 10.08 ≤ B |  | ≤ 12.0 |

The allowed additional maximum output power reduction for PSFCH is defined in Table 6.2E.3.2A.1.1.1-3 for power class 3 and Table 6.2E.3.2A.1.1.1-4 for power class 2.

Table 6.2E.3.2A.1.1.1-3: PSFCH A-MPR for power class 3 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 16.5 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 17.0 | ≤ 16.0 |

Table 6.2E.3.2A.1.1.1-4: PSFCH A-MPR for power class 2 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 17.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 17.5 | ≤ 16.5 |

The allowed additional maximum output power reduction for S-SSB is defined in Table 6.2E.3.2A.1.1.1-5 for power class 3 and Table 6.2E.3.2A.1.1.1-6 for power class 2.

Table 6.2E.3.2A.1.1.1-5: S-SSB A-MPR for power class 3 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 14.5 | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 12.0 | ≤ 15.0 |

Table 6.2E.3.2A.1.1.1-6: S-SSB A-MPR for power class 2 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 15.5 | ≤ 20.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 14.0 | ≤ 16.0 |

6.2E.3.2A.1.1.2	A-MPR without indicating dualPA-Architecture supported

A-MPR in this clause is for SL intra-band non-contiguous CA for UEs not indicating IE dualPA-Architecture supported.

The allowed additional maximum output power reduction for PSSCH/PSCCH is defined in Table 6.2E.3.2A.1.1.2-1 for power class 3 and Table 6.2E.3.2A.1.1.2-2 for power class 2.

Table 6.2E.3.2A.1.1.2-1: PSSCH/PSCCH A-MPR for power class 3 intra-band SL CA

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 18.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 16.5 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 17.0 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.5 | ≤ 16.5 |
|  | 10.08 ≤ B |  | ≤ 15.5 |

Table 6.2E.3.2A.1.1.2-2: PSSCH/PSCCH A-MPR for power class 2 intra-band SL CA

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 20.0 | ≤ 22.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 18.5 | ≤ 22.0 |
|  | 10.08 ≤ B |  | ≤ 20.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 18.5 | ≤ 19.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 15.0 | ≤ 18.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |

The allowed additional maximum output power reduction for PSFCH is defined in Table 6.2E.3.2A.1.1.2-3 for power class 3 and Table 6.2E.3.2A.1.1.2-4 for power class 2.

Table 6.2E.3.2A.1.1.2-3: PSFCH A-MPR for power class 3 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 18.0 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 18.0 | ≤ 18.0 |

Table 6.2E.3.2A.1.1.2-4: PSFCH A-MPR for power class 2 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 20.0 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 20.0 | ≤ 20.0 |

The allowed additional maximum output power reduction for S-SSB is defined in Table 6.2E.3.2A.1.1.2-5 for power class 3 and Table 6.2E.3.2A.1.1.2-6 for power class 2.

Table 6.2E.3.2A.1.1.2-5: S-SSB A-MPR for power class 3 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 21.0 | ≤ 21.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 17.5 | ≤ 21.0 |

Table 6.2E.3.2A.1.1.2-6: S-SSB A-MPR for power class 2 intra-band SL CA

| Carrier Frequency Combination | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- |
| [MHz] | FIM3_Inner | FIM3_Outer |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | ≤ 23.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | ≤ 20.0 | ≤ 25.0 |

< END OF CHANGE #6 >

< START OF CHANGE #7 >

6.2E.4A	Configured transmitted power for Sidelink CA

For intra-band contiguous/non-contiguous SL CA operation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.2E.2 and subclause 6.2E.3 respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤  PCMAX,  ≤  PCMAX_H

For SL transmission of intra-band contiguous/non-contiguous CA when same slot pattern is used in all aggregated component carriers.

PCMAX_L = MIN{10 log10 ∑ pEMAX,C - TC,PEMAX,CA,  PPowerClass, SL_CA – MAX(MAX(MPR,  A-MPR) + ΔTIB,c+TC, P-MPR ), PRegulatory }

PCMAX_H = MIN{10 log10 ∑ pEMAX,C , PEMAX,CA, PPowerClass, SL_CA, PRegulatory }

where

-	For the total transmitted power PCMAX,PSSCH/PSCCH, pEMAX, C is the linear value of PEMAX,c given by the IE sl-maxTransPower for each component carrier and PEMAX, CA is the value given by the IE sl-maxTransPower-CA for maximum transmitted power of SL CA, defined by TS 38.331;

-	For the total transmitted power PCMAX,S-SSB, the PCMAX_L and PCMAX_H are defined as follows:

PCMAX_L = MIN {PPowerClass, SL_CA – MAX(MAX(MPR , A-MPR) + TIB,c , P-MPR), PEMAX,CA , PRegulatory}

PCMAX_H = MIN {PPowerClass, SL_CA,  PEMAX,CA , PRegulatory}

-	For the total transmitted power PCMAX,PSFCH, pEMAX,C is the linear value of PEMAX,c given by IE sl-maxTransPower when single resource pool configured is transmitted at a given time and sum of the IEs sl-maxTransPower when multiple resource pools configured are transmitted at a given time, defined by TS 38.331;

-	PPowerClass, SL_CA is the maximum UE power specified in Table 6.2E.1A-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.2E.2 and subclause 6.2E.3 respectively;

-	TIB,c  and P-MPR are specified in clause 6.2.4 in TS38.101-1;

-	TC is the highest value TC,c among all component carriers c in the subframe over both timeslots. TC,c = 1.5 dB when NOTE 3 in Table 6.2.1-1 in TS38.101-1 applies, otherwise TC,c = 0 dB;

-	PRegulatory= 10 - Gpost connector dBm when V2X UE is within the protected zone in ETSI TS 102 792 of CEN DSRC tolling system and operating in Band n47; PRegulatory= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For intra-band SL CA operation, when at least one different numerology/slot pattern is used in aggregated cells, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

The measured configured maximum output power PUMAX,c for sidelink CA operation, when at least one slot has a different transmission numerology or slot pattern, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

< END OF CHANGE #7 >

< START OF CHANGE #8 >

6.3E.1	Minimum output power for V2X

6.3E.1.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the minimum output power is specified in Table 6.3E.1.1-1. The minimum output power is defined as the mean power in at least one sub-frame 1 ms.

Table 6.3E.1.1-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 51 | -30 | 4.515 |
| 10 | -30 | 9.375 |
| 20 | -30 | 19.095 |
| 30 | -28.2 | 28.815 |
| 40 | -27 | 38.895 |
| Note 1: The CBW is only applicable to PS UE in n14. |  |  |

For NR V2X UE with two transmit antenna connectors, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified for single carrier.

If the UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

6.3E.1.1A	Minimum output power for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the minimum output power requirement as specified in Table 6.3E.1.1A -1 shall be applied per component carrier.

Table 6.3E.1.1A -1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -30 | MBW=REF_SCS*(12*NRB+1)/1000 |
| 20 | -30 |  |
| 30 | -28.2 |  |
| 40 | -27 |  |

6.3E.1.2	Minimum output power for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.3.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.1.1 shall apply for the sidelink in licensed band or Band n47.

For intra-band concurrent NR V2X operation, the minimum output power is defined per carrier and the requirement for NR uplink is specified in clause 6.3.1 and the requirement for NR sidelink is specified in clause 6.3E.1, respectively.

6.3E.1F	Minimum output power for Sidelink Unlicensed

The requirements for minimum output power in clause 6.3.1 apply.

6.3E.1F.1	Minimum output power for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in clause 6.3.1 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.3E.1F shall apply for NR sidelink operation in unlicensed band.

6.3E.2	Transmit OFF power for V2X

6.3E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the requirements specified in current clause apply.

Table 6.3E.2.1-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 51 | -50 | 4.515 |
| 10 | -50 | 9.375 |
| 20 | -50 | 19.095 |
| 30 | -50 | 28.815 |
| 40 | -50 | 38.895 |
| Note 1: The CBW is only applicable to PS UE in n14. |  |  |

For NR V2X UE supporting SL MIMO or Tx Diversity, the transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3E.2.1-1 for single carrier. Transmit off power is defined as the mean power in at least one sub-frame 1 ms.

6.3E.2.1A	Transmit OFF power for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the transmit OFF power requirement as specified in Table 6.3E.2.1A -1 shall be applied per component carrier.

Table 6.3E.2.1A -1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50 | MBW=REF_SCS*(12*NRB+1)/1000 |
| 20 | -50 |  |
| 30 | -50 |  |
| 40 | -50 |  |

6.3E.2.2	Transmit OFF power for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.3.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.2.1 shall apply for the sidelink in licensed band or Band n47.

For intra-band concurrent NR V2X operation, the transmit OFF power requirement is defined per carrier and the requirement for NR uplink is specified in clause 6.3.2 and the requirement for NR sidelink is specified in clause 6.3E.2, respectively.

6.3E.2F	Transmit OFF power for Sidelink Unlicensed

The requirements for Transmit OFF power in clause 6.3.2 apply for SL-U operation.

6.3E.2F.1	Transmit OFF power for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in TS 38.101-1 clause 6.3.2 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.3E.2F shall apply for NR sidelink operation in unlicensed band.

6.3E.3	Transmit ON/OFF time mask for V2X

6.3E.3.1	General

For NR V2X UE, additional requirements on ON/OFF time masks for V2X physical channels and signals are specified in this clause.

6.3E.3.1A	Transmit ON/OFF time mask for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the SL ON/OFF time masks specified in clause 6.3E.3.2, 6.3E.3.3 and 6.3E.3.4 are applicable to each component carrier during the ON power period and the transient periods. The OFF period shall only be applicable to each component carrier when all the component carriers are OFF.

6.3E.3.2	General time mask

The General ON/OFF time mask defines the observation period between the Transmit OFF and ON power and between Transmit ON and OFF power for PSCCH, and PSSCH transmissions in a slot wherein the last symbol is punctured to create a guard period.

Figure 6.3E.3.2-1: General PSCCH/PSSCH time mask for NR V2X UE

For NR V2X UE supporting SL MIMO or Tx Diversity, the ON/OFF time mask requirements apply at each transmit antenna connector.

For UE with two transmit antenna connectors, the general ON/OFF time mask requirements specified in current subclause apply to each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the general ON/OFF time mask requirements apply to the active antenna connector.

6.3E.3.3	S-SSB time mask

The S-PSS/S-SSS/PSBCH time mask for NR V2X UE defines the observation period between transmit OFF and ON S-PSS power and between transmit ON PSBCH and OFF power in a slot wherein the last symbol is punctured to create a guard period.

Figure 6.3E.3.3-1: S-SSB time mask for NR V2X UE

For NR V2X UE supporting SL MIMO or Tx Divesity, the ON/OFF time mask requirements apply at each transmit antenna connector.

For UE with two transmit antenna connectors, the S-SSB ON/OFF time mask requirements specified in current subclause apply to each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the S-SSB ON/OFF time mask requirements apply to the active antenna connector.

6.3E.3.4	Transmit ON/OFF time mask for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.3.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.3.2 and 6.3E.3.3 shall apply for the sidelink in licensed band or Band n47.

For intra-band V2X concurrent operation band specified in subclause 5.2.E.2, the general output power ON/OFF time mask is defined per carrier during the ON power period and the transient periods. The ON/OFF time mask specified in clause 6.3.3.1 is applicable for NR uplink and the ON/OFF time mask in 6.3E.3.1 is applicable for NR sidelink. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

For the TDM operation in same carrier with same bandwidth, the switching time mask in Figure 6.3E.3.4-1 shall be applied.

Figure 6.3E.3.4-1: Time mask for switching between Uu and SL for
same carrier case with same bandwidth

For intra-band V2X concurrent operation band specified in subclause 5.3.E.2, the switching time mask in Figure 6.3E.3.4-2 shall apply for the different carrier case. The switching time shall be located on the RAT of low priority when NR Uu and NR SL have different priorities based on priority information specified in TS 38.321 and TS38.213. It is up to UE implementation when NR Uu and NR SL have the same priority based on priority information specified in TS 38.213.

Figure 6.3E.3.4-2: Time mask for switching between Uu and SL for different carrier case

In the real field, there is a timing advance difference, i.e. NTA∙Tc between NR Uu slot and NR SL slot due to different timing advance of NR Uu and NR SL. The switching time masks do not include timing advance difference but the timing advance difference should be considered with the switching time for same carrier case and different carrier case.

6.3E.3F	Transmit ON/OFF time mask for Sidelink Unlicensed

6.3E.3F.1	General

The transmit power time mask defines the transient period(s) allowed between transmit OFF power as defined in clause 6.3E.2F and transmit ON power symbols (transmit ON/OFF).  The transmit power ON/OFF time mask specified in clause 6.3E.3F.2 supercedes the ON/OFF masks specified in clause 6.3.3; however, between continuous ON-power transmissions the requirements in clause 6.3.3 apply. Unless otherwise stated the requirements in clause 6.5F apply also in transient periods.

6.3E.3F.2	General ON/OFF time mask

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS as illustrated below in Figure 6.3E.3F.2-1. ON/OFF scenarios include: contiguous, and non-contiguous transmission, etc.

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over the duration of at least one slot excluding any transient period and non-transmitted symbols.  The leading transient period starts 5us before the beginning of the first symbol of transmission and extends 10us into the transmission including the CP extension if applicable. The last symbol is punctured to create a guard period where the trailing transient period of 10us is located inside.

Figure 6.3E.3F-1 General ON/OFF time mask for SL-U PSSCH and PSCCH

6.3E.3F.3	S-SSB time mask

The S-PSS/S-SSS/PSBCH time mask for NR Sidelink Unlicensed UE defines the observation period between transmit OFF and ON S-PSS power and between transmit ON PSBCH and OFF power in a slot wherein the last symbol is punctured to create a guard period. The leading transient period starts 5us before the beginning of the first symbol of transmission and extends 10us into the transmission including the CP extension if applicable. The last symbol is punctured to create a guard period where the trailing transient period of 10us is located inside.

Figure 6.3E.3F-2 ON/OFF time mask for SL-U S-SSB

6.3E.3F.4	Transmit ON/OFF time mask for NR SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in clause 6.3.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.3F.2 and 6.3E.3F.3 shall apply for NR sidelink operation in unlicensed band.

6.3E.4	Power control for V2X

6.3E.4.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the following requirements are applied for NR V2X sidelink transmission.

For NR V2X UE supporting SL MIMO or Tx Diversity, the power control tolerance for single carrier shall apply to the sum of output power at each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

The relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL is the ability of the NR V2X UE operating with 30kHz SCS to control the output power of transmitted slots during PSCCH/PSSCH transmission consisting of two slots overlapping with an LTE SL subframe (500us). The reference slot is the 1st slot overlapping with LTE SL subframe and target slot is the subsequent NR SL slot overlapping with the LTE SL subframe. The measurement period for reference and target slot is one NR SL slot with guard symbol omitted. The power of the target slot must be the same or lower than the power of the reference slot using the tolerance equal to relaxation given for Table 6.2.4-1 values in 6.2E.4.1.

6.3E.4.1A	Power control for sidelink CA

For SL intra-band contiguous/non-congiguous CA, the power control requirement as specified in clause 6.3E.4.2 shall be applied per component carrier.

6.3E.4.2	Absolute power tolerance

The requirements in clause 6.3.4.2 shall apply for NR V2X transmission.

6.3E.4.3	Power control for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.3.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4.1 and 6.3E.4.2 shall apply for the sidelink in licensed band or Band n47.

For the intra-band concurrent NR V2X operation, the requirements specified in clause 6.3.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4 shall apply for the sidelink in licensed band.

6.3E.4F	Power control for Sidelink Unlicensed

6.3E.4F.1	General

The requirements on power control accuracy apply under normal conditions.

6.3E.4F.2	Absolute power tolerance

The absolute power tolerance requirements of clause 6.3.4.2 apply at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 40 ms.

6.3E.4F.3	Power control for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in clause 6.3.4 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.3E.4F.1 and 6.3E.4F.2 shall apply for NR sidelink operation in unlicensed band.

< END OF CHANGE #8 >

< START OF CHANGE #9 >

6.4E	Transmit signal quality for V2X

6.4E.1	Frequency error for V2X

6.4E.1.1	General

The UE modulated carrier frequency for NR V2X sidelink transmissions in Table 5.2E.1-1, shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of 1 ms compared to the carrier frequency received from the gNB or V2X synchronization reference UE in case of using the gNB or V2X synchronization reference UE sidelink synchronization signals.

For NR V2X UE supporting SL MIMO or Tx Diversity, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of 1 ms in case of using GNSS synchronization source. The same requirements apply over a period of 1 ms compared to the relative frequency in case of using the NR gNode B or V2X synchronization reference UE sidelink synchronization signals.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

6.4E.1.1A	Frequency error for sidelink CA

For SL intra-band contiguous/non-contiguous CA, ±0.1 PPM observed over a period of 1 ms will be applied per CC compared to the absolute frequency in case of using GNSS synchronization source. The same requirements will be applied to all SL synchronous reference sources (the gNB or V2X synchronization reference UE).

6.4E.1.2	Frequency error for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.4.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.1.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band concurrent NR V2X operation, the requirements specified in clause 6.4.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.1 shall apply for the sidelink in licensed band.

6.4E.1F	Frequency error for Sidelink Unlicensed

The requirements for frequency error in 6.4E.1 apply for SL-U operation.

6.4E.1F.1	Frequency error for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in clause 6.4.1 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.4E.1 shall apply for NR sidelink operation in unlicensed band.

6.4E.2	Transmit modulation quality for V2X

6.4E.2.1	General

The transmit modulation quality requirements in this clause apply to V2X sidelink transmissions.

For NR V2X UE supporting SL MIMO or Tx Diversity, the transmit modulation quality requirements for single carrier shall apply to each transmit antenna connector.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier apply to the active antenna connector.

6.4E.2.2	Error Vector Magnitude for V2X

For V2X sidelink physical channels PSCCH and PSSCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in Table 6.4.2.1-1 except pi/2-BPSK for NR V2X operating bands in Table 5.2E.1-1. When sidelink transmissions are shortened due to transmission gap of one symbol at the end of the slot, the EVM measurement interval is reduced by one symbol, accordingly.

6.4E.2.2A	Error Vector Magnitude for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the EVM requirement as specified in clause 6.4E.2.2 shall be applied per component carrier.

6.4E.2.3	Carrier leakage for V2X

Carrier leakage of NR V2X sidelink transmission, the requirements for NR PUSCH in Table 6.4.2.2-1 shall be applied.

6.4E.2.3A	Carrier leakage for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the carrier leakage requirement as specified in clause 6.4E.2.3 shall be applied per component carrier when only one SL transmission carrier is activated in a time.

6.4E.2.4	In-band emissions for V2X

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the In-band emissions requirements shall be as specified for PUSCH in subclause 6.4.2.3 for the corresponding modulation and transmission bandwidth. When V2X transmissions are shortened due to transmission gap of one symbol at the end of the subframe, the In-band emissions measurement interval is reduced by one symbol, accordingly.

6.4E.2.4A	In-band emissions for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the In-band emission requirement as specified in clause 6.4E.2.4 shall be applied to the SL aggregated transmission bandwidth. This is same as NR intra-band CA UE.

6.4E.2.5	EVM equalizer spectrum flatness for V2X

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the EVM equalizer spectrum flatness requirements shall be as specified for PUSCH in clause 6.4.2.4 for the corresponding modulation and transmission bandwidth.

6.4E.2.6	Transmit modulation quality for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.4.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.2.1 through 6.4E.2.5 shall apply for the sidelink in licensed band or Band n47.

For the intra-band concurrent NR V2X operation, the requirements specified in clause 6.4.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.2 shall apply for the sidelink in licensed band.

6.4E.2F	Transmit modulation quality for Sidelink Unlicensed

6.4E.2F.0	General

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the UE. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

-	In-band emissions for the non-allocated RB

All the parameters defined in clause 6.4.2 are defined using the measurement methodology specified in Annex F.

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in clause 6.4E.2F.2 and 6.4E.2F.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

6.4E.2F.1	Error Vector Magnitude

The requirements for Error Vector Magnitude in clause 6.4E.2.2 apply for SL-U operation.

6.4E.2F.2	Carrier leakage

The requirements for carrier leakage in clause 6.4.2.2 apply for SL-U operation.

6.4E.2F.3	In-band emissions

The requirements for in-band emission in clause 6.4F.2.3 apply for SL-U operation.

6.4E.2F.4	EVM equalizer spectrum flatness

The requirements for EVM equalizer spectrum flatness in clause 6.4.2.4 apply for SL-U operation.

6.4E.2F.5	Transmit modulation quality for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in clause 6.4.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.2F.0 through 6.4E.2F.4 shall apply for NR sidelink operation in unlicensed band.

< END OF CHANGE #9 >

< START OF CHANGE #10 >

6.5E	Output RF spectrum emissions for V2X

6.5E.1	Occupied bandwidth for V2X

6.5E.1.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.1 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in Table 6.5.1-1.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

6.5E.1.1A	Occupied bandwidth for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the aggregated CBW. The occupied bandwidth shall be less than the aggregated channel bandwidth.

6.5E.1.2	Occupied bandwidth for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.5.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.1.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band concurrent NR V2X operation, the requirements specified in clause 6.5.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.1 shall apply for the sidelink in licensed band.

6.5E.1F	Occupied bandwidth for Sidelink Unlicensed

The requirements for occupied bandwidth in clause 6.5.1 apply for the specified SL-U channel bandwidths in Table 5.3E.1F-1.

6.5E.1F.1	Occupied bandwidth for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the requirements specified in clause 6.5.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.1F shall apply for NR sidelink operation in unlicensed band.

6.5E.2	Out of band emission for V2X

6.5E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5E.2.2.1, 6.5E.2.3 and 6.5E.2.4.1 apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector.

6.5E.2.2	Spectrum emission mask

6.5E.2.2.1	General

For NR V2X UE, the existing NR general spectrum emission mask in subclause 6.5.2.2 applies for all supporting NR V2X channel bandwidths. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB), the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth for NR V2X operating bands in Table 5.2E.1-1.

6.5E.2.2.1A	Spectrum emission mask for sidelink CA

For SL intra-band contiguous CA, the SEM requirement for NR intra-band contiguous CA as specified in clause 6.5A.2.2.1 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

For SL intra-band non-contiguous CA, the SEM requirement for NR intra-band non-contiguous CA as specified in clause 6.5A.2.2.2 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

6.5E.2.2.2	Spectrum emission mask for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the general/additional SEM requirements specified in clause 6.5.2 shall apply for the uplink in licensed band and the general/additional SEM requirements specified in clause 6.5E.2.2.1 shall apply for the sidelink in licensed band or Band n47.

For intra-band NR V2X transmission with bandwidth class B where Uu and SL overlap in time the specifications in section 6.5A.2.2.1 and 6.5A.2.2.2 apply.

6.5E.2.3	Additional Spectrum emission mask

6.5E.2.3.1	Requirements for network signalling value "NS_33"

The additional spectrum mask in Table 6.5E.2.3.1-1 applies for NR V2X UE within 5855 MHz to 5925 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1-1.

Table 6.5E.2.3.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for NR V2X overrides any other requirements in frequency range 5855-5925 MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

6.5E.2.3.1A	Requirements for network or pre-configured signalling value “SLCA_NC_NS_33”

For SL intra-band non-cotiguous CA_n47(2A), the additional SEM requirements in subclause 6.5E.2.3.1 (indicated by NS_33) applies in each uplink CC.

6.5E.2.3.2	Requirements for network signalling value "NS_52"

The additional spectrum mask in Table 6.5E.2.3.2-1 applies for NR V2X UE within 5 765 MHz to 6 005 MHz according to FCC regulation. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_52" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.2-1.

Table 6.5E.2.3.2-1: Additional spectrum mask requirements for 40MHz channel bandwidth (fc = 5885MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0-2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40-100 | -50 | 100kHz |

NOTE:	The ASE requirements for NS_52 will not be verified until the corresponding regulation release a formal rule for C-V2X emission limits.

6.5E.2.3.3	Requirements for network signalling value "NS_06"

The additional spectrum mask are signalled by the network to indicate that the public safety (PS) UE in NR band n14 shall meet an additional for a specific deployement scenarios.

When "NS_06" is indicated by serving cell or pre-configured radio parameters, the power of any PS UE emission shall not exceed the levels specified in Table 6.5.2.3.4-1.

6.5E.2.4	Adjacent channel leakage ratio

6.5E.2.4.1	General

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

For NR V2X UE, the existing ACLR requirement for NR uplink transmission in clause 6.5.2.4 are applied for NR V2X UE for NR V2X operating bands in 5.2E.1-1.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

6.5E.2.4.1A	ACLR for sidelink CA

For SL intra-band contiguous CA, the general NR CA ACLR requirements for CA Bandwidth Class B specified in subclause 6.5A.2.4.1.1 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

For SL intra-band non-contiguous CA, the general NR CA ACLR requirements for CA Bandwidth Class B specified in subclause 6.5A.2.4.1.2 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

6.5E.2.4.2	ACLR for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the ACLR requirement specified in clause 6.5.2.4 shall apply for the uplink in licensed band and the ACLR requirement specified in clause 6.5E.2.4.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band NR V2X operation with bandwidth classes B where Uu and SL transmission overlaps in time, the ACLR requirement specified in clause 6.5A.2.4.1 shall apply for the both uplink and sidelink transmission in licensed band.

6.5E.2F	Out of band emission for Sidelink Unlicensed

6.5E.2F.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5E.2F.2	Spectrum emission mask for operation with shared spectrum channel access

The requirements for spectrum emission mask in clause 6.5F.2.2 apply for SL-U operation.

6.5E.2F.2.1	Spectrum emission mask for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the general/additional SEM requirements specified in clause 6.5.2 shall apply for NR Uu operation in licensed band and the general/additional SEM requirements specified in clause 6.5E.2F shall apply for NR sidelink operation in unlicensed band.

6.5E.2F.3	Additional spectrum emission mask

There are no additional spectrum emission mask requirements for SL-U operation. in this version of the specification.

6.5E.2F.4	Adjacent channel leakage ratio

The requirements for ACLR in clause 6.5F.2.4 apply for SL-U operation.

6.5E.2F.4.1	Adjacent channel leakage ratio for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the ACLR requirement specified in clause 6.5.2.4 shall apply for NR Uu operation in licensed band and the ACLR requirement specified in clause 6.5E.2F.4 shall apply for NR sidelink operation in unlicensed band.

6.5E.3	Spurious emissions for V2X

6.5E.3.1	General spurious emissions

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the general spurious emission requirements in clause 6.5.3.1 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector.

6.5E.3.1A	Spurious emissions for sidelink CA

For SL intra-band contiguous/non-contiguous CA, the general NR CA general SE for CA Bandwidth specified in subclause 6.5A.3.1 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

6.5E.3.2	Spurious emissions for UE co-existence

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.3.2 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector.

6.5E.3.2A	Spurious emissions band UE co-existence for sidelink CA

For SL intra-band contiguous CA, the protection operating band lists for n47 transmission is defined in Table 6.5.3.2-1 which shall be applied to NR SL intra-band contiguous CA UE.

For SL intra-band non-contiguous CA, the protection operating band lists for n47 transmission is defined in Table 6.5.3.2-1 which shall be applied to NR SL intra-band non-contiguous CA UE.

6.5E.3.3	Spurious emissions for UE co-existence for V2X concurrent operation

This clause specifies the additional requirements for inter-band concurrent V2X operation with the single CC uplink assigned to two NR bands for coexistence with protected bands for the specified simultaneous transmission of the inter-band concurrent V2X configurations in Table 6.5E.3.3-1. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified simultaneous transmission of the inter-band concurrent V2X. Intersection of a requirement means that both UL or sidelink transmission constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For the inter-band concurrent NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.3-1 apply for the corresponding inter-band concurrent operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

NOTE:	For inter-band concurrent V2X operation with uplink assigned to NR band and slidelink transmission assigned to NR V2X operating bands, the requirements in Table 6.5E.3.3-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5E.3.3-1 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band concurrent UE to UE co-existence requirements.

Table 6.5E.3.3-1: Requirements for inter-band concurrent V2X operation

| concurrent operating band configuration | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n1A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n3A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n8A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n34A-n47A | E-UTRA Band 1, 3, 7, 8, 22, 26, 28, 39, 40, 41, 42, 44, 45, 65, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n34A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n40A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n41A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n71A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n78A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: Void. |  |  |  |  |  |  |  |

For the intra-band NR V2X transmission where Uu and SLoverlap in time , the UE-coexistence requirements in Table 6.5A.3.2.1-1 apply for the corresponding intra-band concurrent operation for the both uplink and sidelink transmission in licensed band.

6.5E.3.4	Additional spurious emissions requirements for V2X

6.5E.3.4.1	General

This clause specifies additional spurious emission requirements for V2X operation

6.5E.3.4.2	Requirements for network signalling value "NS_33"

Table 6.5E.3.4.2-1: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5945 | -30 | 1 |  |
| Frequency range | 5835 |  | - | 5855 | -30 | 1 |  |
| NOTE 1: Void. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Void. |  |  |  |  |  |  |  |

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

6.5E.3.4A	Additional spurious emissions requirements for sidelink CA

6.5E.3.4A.1	General

This clause specifies additional spurious emission requirements for sidelink intra-band non-contiguous CA.

6.5E.3.4A.2	Requirements for network signalling value "SLCA_NC_NS_33"

Table 6.5E.3.4A.2-1: Additional requirements for "SLCA_NC_NS_33"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 | - | 5945 | -30 | 1 |  |
| Frequency range | 5835 | - | 5855 | -30 | 1 |  |
| NOTE 1: Void. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Void. |  |  |  |  |  |  |

When "SLCA_NC_NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4A.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

6.5E.3.4.3	Void

6.5E.3F	Spurious emissions for Sidelink Unlicensed

6.5E.3F.0	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [9] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5E.3F.1	General spurious emissions

The requirements for general spurious emission requirements in clause 6.5.3.1 apply for SL-U operation..

6.5E.3F.2	Spurious emissions for UE co-existence

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation with shared spectrum channel access as identified in Table 5.2-1.

6.5E.3F.2.1	Spurious emissions for UE co-existence for SL-U concurrent operation

For NR SL-U inter-band concurrent operation, the UE-coexistence requirements in Table 6.5A.3.2.3-1 apply for the corresponding inter-band concurrent operation with transmission assigned to both uplink in licensed band and NR sidelink in unlicensed band.

6.5E.3F.3	Additional spurious emissions

6.5E.3F.3.0	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

Editor’s note: Further new NS values with new requirements can be added here.

6.5E.4	Transmit intermodulation

6.5E.4.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.4 apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector.

6.5E.4.1A	Transmit intermodulation for sidelink CA

For SL intra-band contiguous CA, the general NR CA Transmit Intermodulation requirements for CA Bandwidth Class B specified in clause 6.5A.4.2.1 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

6.5E.4.2	Transmit intermodulation for V2X concurrent operation

For the inter-band concurrent NR V2X operation, the requirements specified in clause 6.5.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.4.1 shall apply for the sidelink in licensed band or Band n47.

For the intra-band NR V2X operation where Uu and SLtransmission overlaps in time, the requirements specified in clause 6.5A.4 shall apply for both uplink and sidelink in licensed band.

### 6.5E.4F	Transmit intermodulation for sidelink Unlicensed

The requirements for transmit intermodulation in clause 6.5.4 apply for SL-U operation.

#### 6.5E.4F.1	Transmit intermodulation for SL-U concurrent operation

For NR-U SL inter-band concurrent operation, the requirements specified in clause 6.5.4 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.5E.4F shall apply for NR sidelink operation in unlicensed band.

< END OF CHANGE #10 >

< START OF CHANGE #11 >

7.3E.2A	Minimum requirements for Sidelink CA

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3E.2A.1	Reference sensitivity power level for Sidelink CA

For intra-band contiguous NR SL CA operation, the reference sensitivity requirement specified in Table 7.3E.2-1 shall apply for each component carrier with all carriers active. The requirement is applied for each carrier reception when 2 carrier transmissions are activated at the same time.

For intra-band non-contiguous NR SL CA UE, the reference sensitivity requirement specified in Table 7.3E.2-1 shall apply for each sub-block with all carriers active. The requirement is applied for each sub-block reception when 2 sub-block transmissions are activated at the same time.

< END OF CHANGE #11 >

< START OF CHANGE #12 >

7.4E.1A	Maximum input level for Sidelink CA

For intra-band contiguous NR SL CA operation, the following maximum input level requirement shall be applied to the SL CA bandwidth class B.

Table 7.4E.1A-1 Maximum input levels for intra-band contiguous CA UE

| Rx Parameter | Units | SL CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | A | B |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  | -25 + 10log10(BWChannel /20)Note 1 |
|  |  |  | -27 + 10log10(BWChannel /20)Note 2 |
| NOTE 1: Reference measurement channel is A.7.3 for 64 QAM. NOTE 2: Reference measurement channel is A.7.4 for 256 QAM. NOTE 3: 10log10(x) is rounded to the nearest 0.5dB |  |  |  |

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A.7.3 and A.7.4. The requirements apply with all downlink carriers active.

For intra-band non-contiguous NR SL CA, the maximum input level requirements in Section 7.4E.1 will be applied to each sub-block. The throughput (>= 95% T-put) of each CC shall meet or exceed the minimum requirements for the specified reference measurement channel in A.7.3 and A.7.4. The requirements apply with all downlink carriers active.

< END OF CHANGE #12 >

< START OF CHANGE #13 >

7.5E.1A	Adjacent channel selectivity requirement for Sidelink CA

For intra-band contiguous NR SL CA operation, the UE shall fulfil the minimum requirement specified in Table 7.5E.1A-1 to Table 7.5E.1A-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2.

Table 7.5E.1A-1 ACS for intra-band contiguous NR SL CA UE

|  |  | SL CA bandwidth class |
| --- | --- | --- |
| Rx Parameter | Units | B |
| ACS | dB | 20.0 |

Table 7.5E.1A-2 Test parameters for intra-band contiguous SL CA UE, case 1

| Rx Parameter | Units | SL CA bandwidth class |
| --- | --- | --- |
|  |  | B |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | PREFSENS_SL + 14 dB |
| PInterferer | dBm | Aggregated power + 18.5 dB |
| BWInterferer | MHz | 10 |
| FInterferer (offset) | MHz | 5+Aggreagted BWChannel/2 / -(5+Aggregated BWChannel/2) |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |

Table 7.5E.1A-3 Test parameters for intra-band contiguous SL CA UE, case 2

| Rx Parameter | Units | SL CA Bandwidth Class |
| --- | --- | --- |
|  |  | B |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -43.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 |
| BWInterferer | MHz | 10 |
| FInterferer (offset) | MHz | 5+Aggreagted BWChannel/2 / -(5+Aggregated BWChannel/2) |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |

For intra-band non-contiguous NR SL CA, the UE shall fulfil the minimum requirement specified in Table 7.5E.1-1 to Table 7.5E.1-3 per sub-block where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2.

For the ACS requirement, the UE shall meet the requirements for each sub-block as specified in clauses 7.5E for one component carrier per sub-block. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all sidelink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5E-2 for one component carrier per sub-block. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5E-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer is -25 dBm (Table 7.5E-3) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer like for Case 1.

< END OF CHANGE #13 >

< START OF CHANGE #14 >

7.6E.2.1A	In-band blocking for Sidelink CA

For intra-band contiguous SL CA operation, the UE shall fulfil the minimum requirement specified in Table 7.6E.2.1A-1 to Table 7.6E.2.1A-2 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2.

Table 7.6E.2.1A-1 In-band blocking parameters for intra-band contiguous SL CA UE

| Rx Parameter | Units | SL CA bandwidth class |
| --- | --- | --- |
|  |  | B |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | PREFSENS_SL + NR SL CA bandwidth class specific value below |
|  | dB | 16.0 |
| BWInterferer | MHz | 10 |
| FIoffset, case 1 | MHz | 15 |
| FIoffset, case 2 | MHz | 25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |

Table 7.6E.2.1A-2 In-band blocking for intra-band contiguous SL CA UE

| NR SL CA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -44 | -44 |
| SL_n47B | FInterferer (offset) | MHz | - BWchannel CA/2 – FIoffset,case 1 & + BWchannel CA/2 + FIoffset,case 1 | ≤- BWchannel CA/2 – FIoffset,case 2 & ≥+ BWchannel CA/2 + FIoffset,case 2 |
|  | FInterferer (Range) | MHz | NOTE 2 | FDL_low – 30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 30 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency - BWchannel CA/2 - FIoffset, case 1 and b. the carrier frequency + BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

For intra-band non-contiguous SL CA operation, the UE shall meet the requirements for each sub -block as specified in clause 7.6E.2 for one component carrier per sub-block. The requirements apply for in-gap and out-of-gap interferers while all sidelink carriers are active. The UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with the test parameters defined in clause 7.6E.2.1 per sub-block while all downlink carriers are active.

< END OF CHANGE #14 >

< START OF CHANGE #15 >

7.6E.3.1A	Out-of-band blocking for Sidelink CA

For intra-band contiguous SL CA operation, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with parameters specified in Tables 7.6E.3.1A-1 and 7.6E.3.1A-2.

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6E.3.1A-2, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of MHz with NRB the number of resource blocks in the transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7E.1A apply.

Table 7.6E.3.1A-1: Out-of-band blocking parameters for intra-band contiguous SL CA UE

| Rx Parameter | Units | SL CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B |  |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | PREFSENS_SL + SL CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 |  |  |  |  |

Table 7.6E.3.1A-2: Out of band blocking for intra-band contiguous SL CA UE

| NR SL CA band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| V2X_47B | FInterferer (CW) | MHz | -60 < f – FDL_low < -30 or 30 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: The requirement is applied for multi-carrier intra-band concurrent receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |

For intra-band non-contiguous SL CA operation, the UE shall meet the requirements for each sub-block as specified in clauses 7.6E.3 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active. The UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.6E.3.1 per sub-block while all downlink carriers are active.

< END OF CHANGE #15 >

< START OF CHANGE #16 >

7.7E.1A	Spurious response requirements for Sidelink CA

For intra-band contiguous SL CA operation, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with parameters specified in Table 7.7E.1A-1 and Table 7.7E.1A-2.

Table 7.7E.1A-1: Spurious response parameters for intra-band contiguous SL CA UE

| Rx Parameter | Units | SL CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B |  |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | PREFSENS_SL + SL CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 |  |  |  |  |
| NOTE 1: The requirement is applied for multi-carrier intra-band concurrent receptions when 2 carrier transmissions are activated at the same time. NOTE 2: Reference measurement channel is A.7.2 |  |  |  |  |  |  |

Tables 7.7E.1A-2: Spurious response for intra-band contiguous SL CA UE

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |
| NOTE 1: The requirement is applied for multi-carrier intra-band concurrent receptions when 2 carrier transmissions are activated at the same time. |  |  |

For intra-band non-contiguous SL CA operation, the UE shall meet the requirements for each sub-block as specified in clauses 7.7E for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active. The UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.7E.1 per sub-block while all downlink carriers are active.

< END OF CHANGE #16 >

< START OF CHANGE #17 >

7.8E.2.2A	Wide band intermodulation for Sidelink CA

For intra-band contiguous SL CA operation, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with parameters specified in Table 7.8E.2.1A-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8E.2.2A-1: Wide band intermodulation for intra-band contiguous SL CA UE

| Rx parameter | Units | SL CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B |  |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | PREFSENS_SL + SL CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
| BWInterferer 2 | MHz | 10 |  |  |  |  |
| FInterferer 1 (Offset) | MHz | –Foffset-15 / + Foffset+15 |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The requirement is applied for multi-carrier intra-band concurrent receptions when 2 carrier transmissions are activated at the same time. NOTE 2: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |

For intra-band non-contiguous SL CA operation, the UE shall meet the requirements for each sub-block as specified in clauses 7.8E.2 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active. The UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.8E.2.1 per sub-block while all downlink carriers are active.

< END OF CHANGE #17 >