3GPP TSG-RAN WG4 Meeting #115						R4-2507374

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	On simultaneous RxTx and capability for overlapping FDD bands

Agenda item:	6.11.2

Document for:	Discussion

Introduction

Simultaneous Rx/Tx UE Capability

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory, and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

The Simultaneous Rx/Tx UE capability is conditional mandatory and therefore it needs to be accurately noted in the RAN4 specification whether it is expected supported by the UE for a given band combination.

At RAN4#108bis a discussion was raised in [1] on the ambiguity of the conditional mandatory simultaneous Rx/Tx UE capability. This led to a WF being agreed in [2] which was further discussed in [3] at RAN4#109. Further discussion led to a WF [4] at RAN4#109. During RAN4#110 another WF [5] was agreed based on the discussion captured in [6]. The WF resulted in a draftCR [7] being endorsed. This led to an improvement of the notes by removing redundant notes with a CR [8] in RAN4#111.

At RAN4#112 it was possible to remove yet another redundant note with the draftCR in [9] as well as clarification on the applicability related to FDD combinations in [10] and [11]. However, the endorsed CRs was not implemented to the TS after the meeting.

Endorsed improvements for simultaneous Rx-Tx in R4-2412541, R4-2414266 and R4-2414267 was not implemented to the TSs after RAN4#112.

At RAN4#112bis it was in fact noted by some companies that the above mentioned CRs were not implemented. The missing implementation meant that a CR in [12] was postponed.

The previously not implemented endorsed draftCRs meant the CR in R4-2416359 were postponed.

At RAN4#113 a draftCR [13] were endorsed seemingly combining the content of [10] and [12]. This, [13], was implemented into the BigCR for 38.101-1 (Note the Tdoc title says 38.101-3 but the CR is for 38.101-1) together with the content of [9] in [14] which were endorsed. Similarly, a bigCR for 38.101-3 were endorsed with the content of [11] was endorsed in [15]. However, none of the two bigCRs were implemented, meaning none of the additional endorsed improvements for simultaneous Rx-Tx is visible in the TSs.

BigCRs R4-2417985 and R4-2417986 was not implemented to the specifications after RAN4#113 as MCC focused on other CRs and wanted to postpone the implementation of these.

At RAN4#114 BigCR for 38.101-1 was agreed in [16] and for 38.101-3 in [17]. These two bigCRs should include all the endorsed changes in [9-15]. These CRs are reflected in the 19.1.0 version of TS 38.101-1 and TS 38.101-3.

All previous endorsed changes are included in the agreed BigCRs R4-2500714 and R4-2500715 from RAN4#114 which are reflected in the 19.1.0 version of the specification.

At RAN4#114bis a draftCR to 38.101-1 [18] was agreed capturing the additional agreements made. This draftCR is capturing the clarification on the additional CA types which do not need explicit indication for support of simultaneous RxTx.

At RAN4#114bis R4-2505226 clarifies the CA types which do not need explicit indication for support of simultaneous RxTx.

RAN4 shall ensure that endorsed R4-2505226 is captured correctly to the TS via a CR at RAN4#115.

Even the listed CRs until now have made huge improvements to the clarity of the simultaneous RxTx capability there are still ways to improve even further the TS to clarify which band combinations support simultaneous RxTx. One proposal presented in [19]. This is further discussed later in this contribution.

Further at RAN4#114bis it was decided that the discussion related to simultaneous Rx-Tx issues for UL CA_n5-n8 was to be treated under the agenda for the WI LTE_NR_R19_Simult_RxTx.

Discussions related to simultaneous Rx-Tx issues for UL CA_n5-n8 are decided treated under the agenda for the WI LTE_NR_R19_Simult_RxTx.

The discussion related to CA_n5-n8 is captured later in this contribution.

WF from previous meeting

At RAN4#114bis a WF for simultaneous Rx-Tx [20] includes the following:

For FDD-FDD and FDD-SDL inter-band CA band combinations with partial overlapping frequency or without sufficient separation between DL in one band and UL in the other band, such as, CA_n5-n8 (n8 UL and n5 DL) and CA via low band switching(n12UL and n29DL, n26DL and n106 UL), etc., that have problems in supporting simultaneous Rx-Tx in UL of band nA and DL of band nB, a signalling-related solution should be considered.

Discussion

Indication of mandatory support of simultaneous Rx/Tx

Together with the agreements made for FDD-FDD, TDD-SDL and FDD-SDL all CA configuration types should be covered. This as TDD-TDD and TDD-FDD cases are already indicated via the conditional mandatory capability described in the introduction.

With current RAN4 agreements and the existing framework for conditional mandatory support of simultaneous Rx-Tx all types of CA configurations seem covered.

In summary our understanding is the following for the different types of CA configurations:

- TDD + TDD -> Mandatory support for simultaneous RxTx with signaling

- FDD + TDD -> Mandatory support for simultaneous RxTx with signaling

- TDD + SDL -> Mandatory support for simultaneous RxTx with signaling

- FDD + FDD -> Mandatory support for simultaneous RxTx without signaling

- FDD + SDL -> Mandatory support for simultaneous RxTx without signaling

- The need for signalling is in our view linked to when RAN4 would need to add notes or other indication in the RAN4 specification. It could be beneficial to capture this understanding the TR 38.793.

RAN4 shall agree a TP to TR 38.793 to capture for which type of CA configurations indication is needed captured in the TS.

As captured in the WF [21] RAN4 can consider abandoning the use of notes to indicate the mandatory support of simultaneous Rx/Tx with text in a new column in the BC tables. Here an important point to first address is whether there is a need for more than the two-band BC table, as there currently are also notes for simultaneous Rx/Tx in the 3, 4 and 5-band BC tables in TS 38.101-1.

RAN4 shall discuss whether indication of mandatory support of simultaneous Rx/Tx are needed for other than two-band BCs.

Whether or not adopting a new column, presented in [19], in the BC table(s) can be further discussed after the clarification on above.

Simultaneous Rx-Tx issues for UL CA_n5-n8

The issues related to UL transmissions for CA_n5-n8 due to the overlap of the two bands, as illustrated in figure 2-1 and the fact that they in principle should support simultaneous RxTx are further discussed in [22].

Figure 2-1: FDD-FDD inter-band CA_n5-n8

The discussion in our view should now focus on the exceptions related to e.g. CA_n5-n8. Our understanding is that RAN2 is currently working on enabling the needed capability with indication of these exceptions to mandatory support of simultaneous Rx-Tx FDD-FDD combinations.

RAN2 is working on needed capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

RAN4 shall ensure RAN2 understands the need for a capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: The Simultaneous Rx/Tx UE capability is conditional mandatory and therefore it needs to be accurately noted in the RAN4 specification whether it is expected supported by the UE for a given band combination.

Observation 2: Endorsed improvements for simultaneous Rx-Tx in R4-2412541, R4-2414266 and R4-2414267 was not implemented to the TSs after RAN4#112.

Observation 3: The previously not implemented endorsed draftCRs meant the CR in R4-2416359 were postponed.

Observation 4: BigCRs R4-2417985 and R4-2417986 was not implemented to the specifications after RAN4#113 as MCC focused on other CRs and wanted to postpone the implementation of these.

Observation 5: All previous endorsed changes are included in the agreed BigCRs R4-2500714 and R4-2500715 from RAN4#114 which are reflected in the 19.1.0 version of the specification.

Observation 6: At RAN4#114bis R4-2505226 clarifies the CA types which do not need explicit indication for support of simultaneous RxTx.

Proposal 1: RAN4 shall ensure that endorsed R4-2505226 is captured correctly to the TS via a CR at RAN4#115.

Observation 7: Discussions related to simultaneous Rx-Tx issues for UL CA_n5-n8 are decided treated under the agenda for the WI LTE_NR_R19_Simult_RxTx.

Observation 8: With current RAN4 agreements and the existing framework for conditional mandatory support of simultaneous Rx-Tx all types of CA configurations seem covered.

The need for signalling is in our view linked to when RAN4 would need to add notes or other indication in the RAN4 specification. It could be beneficial to capture this understanding the TR 38.793.

Proposal 2: RAN4 shall agree a TP to TR 38.793 to capture for which type of CA configurations indication is needed captured in the TS.

Proposal 3: RAN4 shall discuss whether indication of mandatory support of simultaneous Rx/Tx are needed for other than two-band BCs.

Observation 9: RAN2 is working on needed capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

Proposal 4: RAN4 shall ensure RAN2 understands the need for a capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

- References

- R4-2316097, Discussion on Simultaneous Rx/Tx Notes, Nokia

- R4-2317576, WF on simultaneous Rx-Tx requirements, Apple

- R4-2320020, Discussion on Simultaneous Rx/Tx, Nokia

- R4-2321992, WF on simultanesou Rx-Tx requirements, Apple

- R4-2403606, WF on simultaneous Rx-Tx issues, Huawei.

- R4-2402087, Discussion on Simultaneous Rx/Tx, Nokia

- R4-2406577, draftCR to 38.101-1 Update to Simultaneous Rx/Tx Notes, Nokia

- R4-2409193, CR to 38.101-1 Update to Simultaneous Rx/Tx Notes, Nokia

- R4-2412541, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2414266, Draft CR to 38.101-1 to clarify simultaneous Rx/Tx applicability for FDD-TDD combinations, Apple

- R4-2414267, Draft CR to 38.101-3 to clarify simultaneous Rx/Tx applicability for band combinations, Apple

- R4-2416359, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2420505, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2417985, BigCR to 38.101-3: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2417986, BigCR to 38.101-3: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2500714, BigCR to 38.101-1: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2500715, BigCR to 38.101-3: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2505226, draftCR to 38.101-1: Simultaneous Rx-Tx, Huawei

- R4-2501993, On simultaneous RxTx requirements, Skyworks Solutions, Inc., Nokia.

- R4-2505093, WF on simultaneous Rx-Tx issues for UL CA_n5-n8 and others, Huawei

- R4-2502840, WF on simultaneous Rx-Tx, Huawei

- R4-2504124, On LS response to RAN1 on UE capability for non-simultaneous RxTx FDD-FDD inter-band CA, Nokia