3GPP TSG-RAN WG4 Meeting #115									                          R4- 2507393

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:			7.5.4

Source:	Ericsson

Title:	Discussion on impact on UE RF requirement of fragmented carriers

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring separate Rx chains per CC may limit the UE CA capabilities if part of complex band combinations with many DL CCs is combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss the PCell DL performance for intra-band non-contiguous CCs supported by a shared Rx chain.

# Discussions

## Discussion on applicability for PCell and SCell

A more detailed proposal based on the tentative agreement in online discussion [2]: - To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects: - To limit the use of this feature to CA configurations where the impact is limited - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity)

The limit of the impact on the DL performance for PCell seems a bit vague, as it could bring different interpretation. To be clearer, we reiterate the importance of maintaining the PCell DL performance, as PCell anchors critical function like coverage and signaling control, etc.,[3].

REFSENS is a conducted noise factor test carried out with up to full allocation in the UL and always full allocation in the DL. Nevertheless, REFSENS performance affects the DL link budget and thus DL coverage.  For REFSENS, the key difference between the single Rx chain and the separate Rx chain lies in their filter capabilities to reject the Tx blocker as detailed in [4] and [5]. Using a single Rx chain for the fragmented carriers in the same band can lead to significant REFSENS degradation in the PCell. This degradation is due to the insufficient rejection of the UE’s own Tx blocker, which requires a lower Rx gain regardless of the UL allocation, since the blocker power is what matters (blocker power depends on the Tx signal strength). The AGC gain has to be reduced, which thereby increases the overall noise floor in the receiver.

Adding an SCell in the fragmented FDD bands (e.g. with 10 MHz gaps) with single Rx chain can degrade the PCell’s REFSENS by up to 13 dB due to insufficient rejection of own Tx blocker according to [4]. This fragmented carriers’ degradation is not equivalent to REFSENS difference from bandwidth scaling in single carriers’ case (e.g. 20/30 MHz vs. 10 MHz allocations), as Tx blocker rejection is the primary issue, a detailed explanation can be found in [3]. All in all, the degradation could lead radio link failure when adding an SCell in the fragmented FDD band because the PCell’s coverage maybe lost after the CA configuration, at least when the Tx is active.

- Adding an SCell in an FDD band may cause significant REFSENS degradation in PCell DL due to its own Tx blocker, potentially leading to PCell coverage loss after CA configuration, at least when the Tx is active.

- A band combination supported by a single Rx chain with REFSENS degradation in PCell DL is not feasible.

In practice, the blocker rejection is larger, the 13 dB discussed above is an minimum RF requirement based on the in-band blocking minimum performance, the actual performance may vary from 0 to 13 dB. Hence the network cannot use this RF minimum requirement. RF requirements should make sure that the UE is functional in most of the scenarios in which it is intended to operate for the feature configured (non-contiguous CA in this case).

The PCell REFSENS should be maintained for non-contiguous CA regardless of the UE implementation. In the case of the fragmented carriers in TDD bands or FDD bands where the PCell (Tx) operates in another band of a composite band combination (BC), a slight SCell degradation of a few dB in the absence of an Tx blocker in the same band may be considered and the ACS and in-band blocking requirements can be maintained.

- In the case of fragmented carriers in the TDD band or FDD band with PCell operating in another band, a slight REFSENS degradation of a few dB may be considered in SCell but not in PCell.

- The PCell REFSENS should be maintained for non-contiguous CA regardless of the UE implementation.

# Proposal

In the previous sections, we made the following observations:

Observation 1	Adding an SCell in an FDD band may cause significant REFSENS degradation in PCell DL due to its own Tx blocker, potentially leading to PCell coverage loss after CA configuration, at least when the Tx is active.

Observation 2	In the case of fragmented carriers in the TDD band or FDD band with PCell operating in another band, a slight REFSENS degradation of a few dB may be considered in SCell but not in PCell.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	A band combination supported by a single Rx chain with REFSENS degradation in PCell DL is not feasible.

Proposal 2	The PCell REFSENS should be maintained for non-contiguous CA regardless of the UE implementation.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- R4-2505109, WF on NR FR1 DL fragmented carriers study, Mediatek, Apple, Qualcomm.

- R4-2504037, Discussion on the DL PCell REFSENS degradation of fragmented carriers, Ericsson.

- R4-2500219, In-Gap Blocking for Fragmented Carrier, Murata Manufacturing Co. Ltd.

- R4-2501397, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc.