3GPP TSG-RAN WG4 Meeting #115									                          R4- 2507397

St Julian’s, Malta, 19th – 23rd May 2025

Agenda item:			7.25.2

Source:	Ericsson

Title:	Discussion on SBFD symbol on UL duty cycle evaluation and MPR impact on SBFD UL resource muting

Document for:	Discussion

# Introduction

During the last RAN4 #114bis meeting, several issues related to SBFD UE RF were discussed and the part of the WF are captured below [1]:

Issue 1-7-1: Update for HPUE duty cycle evaluation Way forward: RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE. Issue 2-1-1: RF requirement impact for PUSCH with resource muting Way forward: RAN4 has discussed this issue and will try to draft a reply LS to RAN1 based on existing results in next meeting. - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

# Discussion

## SBFD symbols on the UL duty cycle evaluation

In the RAN4 #114bis meeting, one company proposed to update 38.101-1 clause 6.2.1 by adding the “SBFD symbol” together with the existing “UL symbol transmitted “in the UL duty cycle evaluation. The current specification of the UL duty cycle evaluation was excerpted below [2]:

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

From UE’s perspective, any UL symbols transmitted—whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation. At this stage, we do not anticipate any impact on RAN4 UE specification, nor do we see a need for clarification. Furthermore, there is no definition of SBFD-aware UE or SBFD-capable UE, adding such a term in Rel-19 makes the clause redundant.

- Any UL symbols transmitted—whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation.

- The SBFD symbol does not affect the UL duty cycle evaluation in the RAN4 UE specification, and no further clarification is needed.

## Transmit signal quality/MPR impact

Besides the previous LS sent by RAN1, a new LS was received to further clarify on the UL resource muting [3] [4].

Agreement For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L.

Based on the RAN1 technical report, the following options for UL resource muting are studied to enhance gNB-to-gNB co-channel CLI measurement and/or channel measurement.

-	Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource)

-	Option 2: Non-transparent UL resource muting method (e.g., define UL resource muting pattern with one or more RE/RB muting patterns)

For option 2, A comb offset {0, 1} is configured for up to two UL muting symbols for CP-OFDM, which means that resources are muted in an alternative pattern of up to two uplink symbols. It is illustrated in Figure 1.

Figure 1. An example combo offset {0, 1} configuration for UL PUSCH muting with two muting symbols.

Only up to two symbols out of 14 were applied, and the alternating pattern distributes power evenly in the muting symbol. The physical RB locations remain unchanged. MPR is primarily tied to the frequency-domain RB allocation, e.g., edge versus inner RBs, and other factors, such as power classes, modulation and coding scheme, channel bandwidth, etc.

Another agreement was that “3 dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.” It means there is a 3 dB power boost on the active REs in muted symbols, the total transmitted power and average power remain the same.

The impact can be assessed by comparing the transmitting power and calculating the PAPR for the symbols with and without UL resource muting. QPSK with PA clipping is used for simplicity, and PRB allocation is 16, each with 12 subcarriers. The SCS is 15 kHz.

The comparisons in transmitted signal and PAPR between muting and without muting are shown in Figure 2 and Figure 3. Boosting the active subcarriers by 3 dB means each active subcarrier has double the power. However, in the time domain, the amplitude is determined by the sum of the IFFT of the subcarriers. The signal is effectively with double SCS, but with an FFT with greater granularity. The SC-FDMA properties are not destroyed though for DFT-s-OFDM, and for CP-OFDM the PAPR does not change much once there are a large enough number of sub-carriers.

Figure 2. Signal envelope for the first 14 symbols.

Figure 3. Simulated PAPR per symbol w and w/o UL resource muting.

- MPR remains unaffected by the introduction of UL resource muting.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Any UL symbols transmitted—whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation.

Observation 2	MPR remains unaffected by the introduction of UL resource muting.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The SBFD symbol does not affect the UL duty cycle evaluation in the RAN4 UE specification, and no further clarification is needed.

# References

- R4-2504751, Way Forward for [114bis][306] NR_duplex_evo_General, Samsung.

- TS 38.101-1, v19.1.0.

- R1-2410889, LS on impact on transmit signal quality/MPR requirement.

- R1-2503089, LS on impact on transmit signal quality/MPR requirement.