3GPP TSG-RAN WG4 Meeting #115	R4-2507429

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 1

Agenda item:	7.21.2

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 continued the discussion on the general aspects of AI/ML based on the objectives agreed in RAN4#107, WI ‎‎[1] “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was agreed.

In this paper, we share our further views on the following topics, mainly for UE sided model cases:

- Post deployment testing

- Life-Cycle Management related aspects

- On Generalization, number of tests and non-stationary conditions

Discussion

Post deployment testing

Following agreement was reached during RAN4#114bis meeting:

Agreement: - Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing” - Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

The moderator suggested the way forward in RAN4#114 for the post deployment testing. We updated our suggestions in the table below based on the agreement reached in RAN4#114bis meeting. In the table, the updates are highlighted in green.

The following table presents the description of options 1 and 2 and sub-options as already agreed. A new name for “option 1” and “option 2” is proposed to reduce ambiguity when discussing in the future. The descriptions of the options are the same as the already agreed description. The “further potential clarifications” box captures issues that have been raised that could further clarify the options. It does not represent any agreement on the issues, or that all are relevant, but is intended to stimulate input and discussion to future meetings. The “significant issues” line is intended to capture issues that have been raised that should be answered in order to make a decision. Option 1; Option 2. New name; Post-deployment pre-activation functionality update testing Post-deployment = the UE is operating in the field, connected to a mobile network. Pre-activation = the functionality/configuration is available in (supported by) the UE device, and it is not RRC/MAC activated. The functionality/configuration requires testing before it can be RRC/MAC activated by the NW. RRC/MAC Activation – used in the field - functionality is already RRC configured in the UE - Functionality is just available in the UE? - Functionality is configured only for test but not for active use Test-active = the functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing. Check Proposals 1-3. The ML-enabled functionality will provide ground-truth along with the inference output to the NW as a part of the post deployment testing. If not Option 1 is not feasible.; Post-deployment post-activation functionality testing based on performance monitoring Post-deployment = the UE is operating in the field, connected to a mobile network. Post-activation = the functionality/ configuration is available in (supported by the) UE, it has been RRC configured by the NW and, has been RRC/MAC activated by the NW. Test-active = the functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing. Check Proposal 1 and 4.. Description; Conduct the validation of a change in AI functionality that is already deployed in the UE before its RRC/MAC activation to be used for any inference.. • Validation takes into account the UE capabilities in which the updated functionality is to be activated. Check Proposal 2 & 3.; The UE is configured and provides monitoring reports. The NW takes ML management (“LCM”) actions (RRC re-configuration) based on monitoring reports and the in/applicability indications received from the UE. Check Proposal 1 and 4. - Performance monitoring will be designed in other groups (RAN1-RAN2) - Functionality in/applicability configuration procedure is designed in other groups (RAN2) The post-deployment testing can be triggered by the NW when the UE indicates the applicability of a functionality/configurations which was previously indicated as in-applicable to the NW. (RAN2 procedures) Check Proposal 1. - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the active and applicable functionality, sent from the UE, and set requirements as feasible/needed. - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the test-active functionality, sent from the UE, and set requirements as feasible/needed. Check Proposal 4.. Sub-options; Possible collection of input data to a model during conformance testing for use later on.; Possible re-configuration of the performance monitoring to enable monitoring of the test-active functionality/configuration.. Further potential clarifications (not resolved); The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. Neither. Check Proposal 5 and 7. o Whether models that have been updated post conformance testing should be kept in a device. Yes. Check Proposal 8. o Whether parallel operating of one model and (in-device) testing of another can be assumed. Yes. Check Proposal 9. o Whether signaling from a UE of changes/updates are needed. No. Check Proposal 10. - Whether there is any relation to GCF procedures. Possibly. Check Proposal 12. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. Not in 3GPP scope. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. Not in Rel 19 scope. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy. Yes. Check Proposal 18.; The following proposals have been made for option 2 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. Check Proposal 4, 6 and 11. - Whether monitoring used for post-deployment testing should be NW sided only. - Whether signaling from a UE of changes/updates are needed. Check Proposal 11. - Whether there is any relation to GCF procedures. Possibly. Check Proposal 12. Even in case of the Option 2, parallel operation of ML-enabled functionalities is feasible. Explained below.. Significant issues; Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates.; Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) No. Check Proposal 9. Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. Yes. Check Proposal 10..

For both Option 1 and Option 2, we need to differentiate between the functionality/configuration which is being tested and any other functionality/configuration for the same UE feature (sub use-case) which has been configured for normal operation and reporting (RAN1-2). For this we propose to introduce the term “test-active” defined as “the functionality/configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”

The RAN4 discussions, for both Option 1 and Option 2, to use “test-active” defined as the “functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”

The details, and high-level steps, for Option 1 are explained further in Section 2.1.1.1. The details, and high-level steps, for Option 2 are explained further in Section 2.1.1.2.

Details of Description

      - Post-deployment - Option 1

In case of post-deployment Option 1, the main steps are described below. The first 3 steps are in accordance with currently discussed RAN2 procedures, Step 4 and Step 7 are common for the post-deployment Option 1 and Option 2, while Steps 5-6 are the post-deployment Option 1 testing related (high-level) steps:

- The UE is assumed to have provided its relevant (ML-enabled feature) capabilities to the gNB/NW.

- The gNB/NW has configured the UE for the ML-enabled feature, including inapplicability/applicability indications and the monitoring reports

- Based on the received inapplicability/applicability indications and/or monitoring reports the gNB/NW take ML management (“LCM”) actions (e.g. RRC re-configurations)

- The gNB/NW can enable the functionality for post-deployment testing. This is based on the applicability conditions (applicability report, RAN2) of the ML-enabled feature which the gNB/NW can use to decide on the tests that are required to test the feature.

Note: It is up to NW implementation what available information to use to decide on the need for triggering the post-deployment testing of the functionality/configurations declared as applicable by the UE.

- If the gNB/NW decides that a post-deployment test is required for the ML-enabled feature, then it will configure a test-active mode to run the post-deployment test for the feature.

        - The test-active configuration in Option 1 consists of configuration of RRC reports including ground-truth information for the feature being tested

- The test-active reports results are collected.

- Based on the test results:

    - If the KPIs are met, then the tested ML-enabled feature configuration (functionality) can be RRC/MAC activated to be used for inference

    - If the KPIs are not met, then the tested ML-enabled feature configuration (functionality) will not be activated, and another configuration/functionality or the fallback mechanism can be activated instead.

To realize the steps above a robust framework is necessary to enable the UE to be configured for test-active mode (Step 4-5) and then the post-deployment tests can be run (Step 6-7).

RAN4 should discuss on the framework related to the test-active mode of the ML-enabled feature to realize the post-deployment testing in Option 1.

Since the ML-enabled feature is already available in the UE and needs to be tested in the field, there can be various in-field conditions that govern the usage of the feature. And this will reflect in the tests that need to be conducted to ensure the feature works as expected.

In RAN4 context this translates to requirements that need to be defined as configurable conditions in which a ML-enabled feature is required to undergo a post-deployment test (Option 1)

RAN4 should further discuss the possible in-field configurable conditions and requirements for a post-deployment test of an ML-enabled feature in Option 1.

      - Post-deployment Option 2

In our understanding, in Option 2 the performance monitoring mechanisms/procedures discussed in RAN1-2 are to be, and can be, used. Particularly, the use of the performance monitoring mechanisms/procedures implies that the functionality/configuration has been RRC/MAC activated by the gNB/NW and, the UE can provide the corresponding performance monitoring metrics to the gNB/NW.

The main steps for this Option 2 are described below. The first 3 steps are in accordance with currently discussed RAN2 procedures, Step 4 and Step 7 are common for the post-deployment Option 1 and Option 2, while Steps 5-6 are the post-deployment testing Option 2 related (high-level) steps:

- The UE is assumed to have provided its relevant (ML-enabled feature) capabilities to the gNB/NW

- The gNB/NW has configured the UE for the ML-enabled feature, including inapplicability/applicability indications and the monitoring reports

- Based on the received inapplicability/applicability indications and/or monitoring reports the gNB/NW take ML management (“LCM”) actions (e.g. RRC re-configurations)

- The post-deployment testing can be triggered by the NW when the UE indicates the applicability of a functionality/configurations which was previously indicated as inapplicable to the NW.

Note: It is up to NW implementation what available information to use to decide on the need for triggering the post-deployment testing of the functionality/configurations declared as applicable by the UE.

- If the gNB/NW decides that a post-deployment test is required for the ML-enabled feature, then it will configure a test-active mode to run the post-deployment test for the feature.

  - The test-active configuration in Option 2 consists of supported RRC reports and performance monitoring reports configurations for the feature being tested

- The performance monitoring results corresponding to the test-active configuration are collected.

- Based on the test results:

    - If the performance KPI(s) are met, then the tested ML-enabled feature configuration (functionality) can be RRC/MAC activated to be used for inference

    - If the performance KPI(s) are not met, then the tested ML-enabled feature configuration (functionality) will not be activated, and another configuration/functionality or the fallback mechanism can be activated instead.

In RAN4 context there are two related aspects to be considered for Option 2. First, we assume that RAN4 will address the requirements related to performance monitoring as appropriate for all use cases.

RAN4 to consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the configured, active and applicable functionality/ configuration, sent from the UE, and set requirements as needed.

Secondly, due to the similarity with the RAN2 discussed performance monitoring (for the configured, active and applicable functionality/ configuration), we propose to also address in RAN4 the similar requirements for the configured and test-active functionality/ configuration.

Based on the current RAN1 and RAN2 agreements for release 19 on performance monitoring, RAN4 to consider the need and feasibility of requirements and tests to ensure the required functioning of performance monitoring procedures for the test-active functionality/ configuration, and set performance monitoring reporting requirements as needed

The introduction of the test-active functionality/ configuration can lead to the need to define the corresponding procedures e.g., re-configuration messages of the performance monitoring to enable monitoring of the test-active functionality/configuration, in RAN2. In RAN4 context, however, the actual test requirements may be the same as for the normal applicable functionality/ configuration (Observation 1). This indicates a potentially easier adoption of Option 2 in the time-frame of Release 19.

Details on Further potential clarifications (not resolved)

In this section we discuss about issues that need further clarifications (as listed in the table row 4) and are not resolved in the past meetings as identified and noted down in the last meeting.

      - Post-deployment Option 2

Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.)

Considering the general scope of the RAN4 performance testing and post-deployment testing it is not to address explicit ML model performance (based on RAN1- RAN2 agreements on performance monitoring), the answer to this question is clearly: not possible.

Based on the current RAN1- RAN2 agreements for Release 19, RAN4 to not purse testing procedures for individual UE-side ML model performance testing purposes.

Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs.

Performance monitoring procedures, specific to ML-enabled features, are being designed in RAN1-RAN2 for release 19 use cases. Therefore, it is very reasonable to assume that RAN4 can address the necessity of testing requirements for performance monitoring procedures, as already suggested in Proposal 4.

Furthermore, the Option 2 solutions also require the monitoring procedures for the test-active configurations as described in Section 2.1.2.2.

      - Common between Post-deployment Option 1 and Post-deployment Option 2

Whether the testing of a new model is on a device or in a lab.

We are of the opinion that the UE device should be tested in the field and not in the lab. As a test cycle, testing of implementation changes/updates of functionalities are not feasible in the lab due to the following reasons:

- Frequent updates to the AI/ML models supporting the ML-enabled feature configurations: The companies can come up with very frequent ML model updates and this will increase the test frequency and duration proportionally.

- High turnaround time for the conformance tests: For the conformance tests to be done across different ML-enabled feature configurations before deployment, even if we assume that an abridged set of conformance tests impacting the functionality alone is decided for this purpose, it could increase the turnaround time for the test significantly as the testing is done on a per device basis.

Frequent updates to the UE specific ML models supporting the ML-enabled feature configurations may significantly prolong the turnaround time and testing overhead if tested the lab.

Conformance testing for changed/updated ML functionality before deployment (Option 1) in the lab can be time consuming and it must be de-prioritized.

Whether models that have been updated post conformance testing should be kept in a device.

In this case we are of the opinion that at least one version of the ML-enabled feature configuration (including the supporting ML models), that has passed conformance test should be kept in the device. In case of a malfunction is identified during performance monitoring, then the fallback/switch should be allowed only to the functionality or legacy (non-AI/ML) feature that has passed conformance testing. This would mean that the functionality that has passed conformance testing should be always available in the UE device.

RAN4 to ensure and to clarify that at least one version of the ML-enabled feature configuration (including the supporting ML models), that has passed conformance testing shall always be available in the device.

Whether parallel operating of one model and (in-device) testing of another can be assumed.

Parallel operating of one ML-enabled feature configuration along with another ML-enabled feature configuration can be a good solution for realizing post-deployment testing of an updated ML-enabled functionality. However, this must be supported by the device which is capable of running several inferences (and the underlying ML models) in parallel. If supported, then one functionality can be active, and another can be in test-active mode (discussed in earlier section) while it is undergoing post-deployment test.

This can be realized in different ways based on how inference for the active and test-active ML-enabled functionalities can be arranged.

- Time split inference scheme for post-deployment testing

In this case the UE can do inferences across the active ML-enabled functionality and test-active ML-enabled functionality that is being tested in tandem.

- Regular inference: The major percentage of total time is used/allocated to make inference of the currently active functionality (to support the respective use-case)

- Validation Inference: The remaining/smaller percentage of the total time is allocated to make inference of the updated/new model/functionality in the device for the purpose of testing/validation in test-active state before it can be activated.

Note that in this case the model switch time is not negligible, and it is necessary to allocate model switching slot between the regular inference slot and validation inference slot.

- Parallel inference scheme for post-deployment testing

If UE is capable of parallel inference for several ML-enabled functionality at once, then the inference made by the currently active functionality will be used to support the use-case whereas the inference made by the test-active functionality can be used for the testing/validation of the updated functionality before activation.

- Note: Both the schemes of post-deployment testing described above can be used as part of Model Test Framework (proactive testing approach) as well as Model Monitoring Framework (reactive testing approach).

In order to validate the new ML-enabled feature configuration before using it in the field, the UE need to support inference either in consecutive/time splitting manner or in parallel.

RAN4 to further discuss the option of using both active as well as test-active in parallel to realize post-deployment testing of updated ML-enabled configuration in the field.

Whether signaling from a UE of changes/updates are needed.

To realize the post-deployment testing of an ML-enabled functionality in the field, it is desirable for the NW to be notified by the UE on the update of its implementation supporting an ML-enabled feature configuration. Potentially, based on the “type” of update that the ML-enabled functionality has undergone, the NW can arrange the test suite that needs to be run as part of in-field post-deployment testing of the functionality.

An illustration of the signaling flow between the UE and the NW is illustrated in Figure 3.

The trigger for the post-deployment test itself can be based on the applicability report that the UE sends to the NW (based on agreed RAN2 applicability reporting procedures). This consists of including in the applicability report additional information about the update “type” that the feature has undergone. Based on this if the NW decides that a post-deployment test is required, then it can re-configure the UE accordingly (see Option 1 or Option 2). Based on the outcome of the test(s), it can be decided if the updated functionality can be activated, or a fallback is required.

Figure 3:Typical signaling exchange between the UE and NW to indicate the update to an ML-enabled functionality

It should be noted that Option 1 is not fully supported by RAN2.

RAN4 should further discuss on the potential signaling requirements to realize the post-deployment tests in Option 1.

In order to realize post-deployment testing in Option 2, the signaling requirements can be part of the UE-side “LCM” signaling that are being discussed in RAN2.

Updates to current UE-side “LCM” signaling procedures are required to realize post-deployment testing of updated ML-enabled functionality in Option 2 as a part of performance monitoring. This must be done in alignment with RAN2 and the impacts to RAN4 should be further discussed.

Whether there is any relation to GCF procedures.

Relation to GCF should be discussed in RAN5 when RAN5 discussions start.

Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy.

For this option, we believe that a combination of Option 1 and Option 2 can also be viable way forward. The Option 1 ensures that the functionality configuration can be tested before it is activated (for inference and reporting) by the NW. And in Option 2 the performance monitoring at the NW-side is used to detect performance changes of the already activated UE-side AI/ML-enabled feature configurations. The combination of these mechanisms, supported by the signalling from the UE about the changes/updates in AI/ML-based feature, can ensure a high reliability of the post-deployment testing, hence increase the trustworthiness of the AI/ML operation in live networks.

In future releases, RAN4 can consider a coexistence of both Options 1 and 2 for the post-deployment testing.

Life-Cycle Management related aspects

Delay requirements for performance monitoring reporting

Following agreement was reached in RAN4#114bis meeting on the delay requirements for reporting/monitoring:

Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID

Regarding the Beam management use case’s performance monitoring reporting, RAN1 reached on the following agreements in RAN1#120bis meeting:

Agreement: For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB Agreement: For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support.

Regarding the CSI prediction use case’s performance monitoring reporting, following agreements were reached in RAN1#120bis meeting:

Agreement: For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric Agreement: For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement: For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Regarding the AI/ML based positioning use case’s performance monitoring reporting, following agreements were reached in RAN1#119 and RAN1#120bis meetings:

RAN1#119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B RAN1#120bis Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

Based on the above agreements in RAN1, RAN4 may define delay requirements for performance monitoring reporting from the UE for the following use cases:

  - For BM use case, Type 1 Option 2 (UE-assisted) performance monitoring in BM-Case1 and BM-Case2 with a UE-side AI/ML model

  - For AI/ML Positioning use case, Option A performance monitoring for UE-based positioning in Case 1

  - For CSI prediction use case, Type 1 and Type 3 (UE-assisted) performance monitoring in CSI prediction using UE-side model

RAN4 to consider defining performance monitoring reporting requirements for BM-Case1, BM-Case2, CSI prediction, and AI/ML Positioning Case 1.

Delay requirements for activation/deactivation/fallback

On the definition of activation and deactivation delays, following agreement was reached in RAN4#114bis meeting:

Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality

For further consistency between the two definitions agreed upon in last meeting, we propose to add the “AI/ML” term to the deactivation definition.

RAN4 to adopt the following definitions of AI/ML functionality activation and deactivation:

  - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference

  - AI/ML functionality deactivation refers to the process of network deactivating an active functionality.

Furthermore, regarding the starting and end points of deactivation delay following agreement was reached:

Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components

In our understanding, the deactivation message, which is initiated at the NW, should be also accompanied by the configuration for the new functionality, which might be an AI/ML or non-AI/ML (legacy) functionality. Thus, the time termination point of the deactivation can be traced to either 1) when the first UE inference reports for the new functionality (either AI/ML or legacy) are received at the NW side, or 2) when the UE’s acknowledgment upon reconfiguration (e.g., RRCReconfigComplete) is received at the NW side. Therefore, regarding the endpoint for deactivation, the following two options may be the candidate:

- Option 1: When the UE reports inference for the new AI/ML/non-AIML functionality

- Option 2: In response to the signalling to commence deactivation, UE may signal an ACK to the network once deactivation is complete at its end. (We need to check RAN2 signaling call flow here).

Since the report configuration may vary, that is, the reporting time interval, the Option 1 may yield significant deviations in terms of the delay requirements, therefore, the delay requirement for the deactivation (which intuitively should be accompanied by the activation of the new functionality) can be formulated as min(delay-requirement-for-option-1, delay-requirement-for-option-2).

Also note that, for option 2, the ambiguity of the UE applicability report behaviour should be discussed and resolved, that is, if the (re-)configuration for the new functionality (which is accompanied by the deactivation command) would not be applicable at the UE side, it would result in the separation of the deactivation of the old functionality and activation of the new functionality, which as stated above, should occur simultaneously.

If RAN4 agrees to define deactivation delay requirements then RAN4 to consider the endpoint of deactivation delay when UE sends an RRCReconfigurationComplete message in response to RRCReconfiguration for the configuration of new AI/ML/non-AI/ML functionality

Regarding the FFS for fallback delay, as per our understanding, fallback delay is same as deactivation delay if UE goes to non-AI/ML operation upon receiving the deactivation command from the NW. Since RAN1 is not differentiating between switching to another AI/ML functionality and to Non-AI/ML functionality. Therefore, RAN4 doesn’t need to define any requirement for fallback delay.

RAN4 should not define any requirements for the fallback delay.

In RAN4#114bis meeting, some discussions were held on the use case specific impacts from LCM requirements, particularly following agreement was reached for AI/ML based Beam Management use case:

Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting

RAN1 also reached on following agreements regarding LCM aspects of the BM use case:

RAN1#119 Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion RAN2#129 Agreements - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

According to RAN1/RAN2 agreements, for periodic CSI reporting, UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete. For semi-persistent and aperiodic CSI reporting, applicable functionality is activated following legacy CSI framework.

If RAN4 agrees to define activation delay requirement then RAN4 should consider defining activation delay requirements for periodic CSI reporting in beam management use case.

RAN4 to define activation delay requirements, especially for periodic CSI reporting where UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete.

Periodic reporting:

- Starting point: UE reports the applicable functionality via RRCReconfigurationComplete

- Ending point: UE reports inference for the activated functionality

Semi-persistent/aperiodic reporting:

- Starting point: UE receives activation command from the NW (MAC CE/DCI)

- Ending point: UE reports inference for the activated functionality

On Generalization, number of tests and non-stationary conditions.

On the generalization aspects, following agreement was reached in RAN4#114bis meeting:

Agreement: - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test.

There is a group of parameters/configurations that are not explicitly defined either by the feature/functionality nor by the Associated ID (parameters in group (3) in the list above). For example, the capability of the UE to maintain the minimal level of performance in different radio propagation conditions is the core question of generalization (and valid problem regardless of the use/non-use of ML algorithms) and should be ensured by the sufficient test/requirement coverage. We envision two open issues related to such parameters:

- What should be the number of additional test requirements per given UE AI/ML-enabled Feature configuration and selected Associated ID(s) combination?

  - This will depend, for example, whether the deployment, UE configurations or channel model will be specified by those or not.

  - On top of that, it will be necessary to study how the performance of the AI/ML-enabled Feature is sensitive to variations in those test parameters in group (3), i.e., how well the functionality generalizes to changes in those.

- Are there any potential degradations in the performance when the change of test parameters/configurations happens dynamically, i.e., in non-static way?

  - We cannot expect that RRC reconfiguration happens when additional conditions / parameters / configurations (e.g. channel conditions) change. UE should use its internal mechanisms/implementation to ensure good performance of the functionality, e.g., by changing to the proper underlying model within functionality to match the conditions.

Regarding the first issue above, we think that more clarity from other WGs and a study of generalization UE capabilities to different scenarios will be needed in RAN4. It will still take some time.

The number of additional requirements per given UE AI/ML-enabled Feature and Associated ID will depend on their concrete definitions of the functionalities/configurations in the other WGs and the generalization capabilities of the UE implementation solutions, and both require further study in a per use-case manner.

For the testing of Generalization aspects, RAN4 needs to study the variation in the AI/ML-enabled Feature performance due to the change in test conditions (e.g., propagation conditions) to conclude about the final number of requirements/tests per given AI/ML-enabled Feature and Associated ID combination.

Furthermore, on the issue of static and non-static scenarios for testing, following agreement was reached in RAN4#114bis meeting:

Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario

From our perspective, it won’t be meaningful to agree on a mixed dataset to test generalization aspects, since it will be like testing AI/ML model and not the telecom aspects of the AI/ML functionality. Therefore, a more suitable way to test generalization aspects is by testing it in different scenarios as discussed above.

RAN4 should not create a mixed dataset for generalization testing since the objective is not to test the AI/ML model.

Conclusion

In this contribution, we addressed the following topics mainly for UE sided model cases:

- Post-deployment testing

- Life-Cycle Management related aspects

- Generalization, number of tests and non-stationary conditions

Following Observations and Proposals were made in this paper:

Proposal 1: The RAN4 discussions, for both Option 1 and Option 2, to use “test-active” defined as the “functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”

Proposal 2: RAN4 should discuss on the framework related to the test-active mode of the ML-enabled feature to realize the post-deployment testing in Option 1.

Proposal 3: RAN4 should further discuss the possible in-field configurable conditions and requirements for a post-deployment test of an ML-enabled feature in Option 1.

Observation 1: RAN4 to consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the configured, active and applicable functionality/ configuration, sent from the UE, and set requirements as needed.

Proposal 4: Based on the current RAN1 and RAN2 agreements for release 19 on performance monitoring, RAN4 to consider the need and feasibility of requirements and tests to ensure the required functioning of performance monitoring procedures for the test-active functionality/ configuration, and set performance monitoring reporting requirements as needed

Proposal 5: Based on the current RAN1- RAN2 agreements for Release 19, RAN4 to not purse testing procedures for individual UE-side ML model performance testing purposes.

Observation 2: Frequent updates to the UE specific ML models supporting the ML-enabled feature configurations may significantly prolong the turnaround time and testing overhead if tested the lab.

Proposal 6:  Conformance testing for changed/updated ML functionality before deployment (Option 1) in the lab can be time consuming and it must be de-prioritized.

Proposal 7: RAN4 to ensure and to clarify that at least one version of the ML-enabled feature configuration (including the supporting ML models), that has passed conformance testing shall always be available in the device.

Observation 3: In order to validate the new ML-enabled feature configuration before using it in the field, the UE need to support inference either in consecutive/time splitting manner or in parallel.

Proposal 8: RAN4 to further discuss the option of using both active as well as test-active in parallel to realize post-deployment testing of updated ML-enabled configuration in the field.

Observation 4: It should be noted that Option 1 is not fully supported by RAN2.

Proposal 9: RAN4 should further discuss on the potential signaling requirements to realize the post-deployment tests in Option 1.

Observation 5: In order to realize post-deployment testing in Option 2, the signaling requirements can be part of the UE-side “LCM” signaling that are being discussed in RAN2.

Proposal 10: Updates to current UE-side “LCM” signaling procedures are required to realize post-deployment testing of updated ML-enabled functionality in Option 2 as a part of performance monitoring. This must be done in alignment with RAN2 and the impacts to RAN4 should be further discussed.

Proposal 11: Relation to GCF should be discussed in RAN5 when RAN5 discussions start.

Proposal 12: In future releases, RAN4 can consider a coexistence of both Options 1 and 2 for the post-deployment testing.

Proposal 13: RAN4 to consider defining performance monitoring reporting requirements for BM-Case1, BM-Case2, CSI prediction, and AI/ML Positioning Case 1.

Proposal 14: RAN4 to adopt the following definitions of AI/ML functionality activation and deactivation:

- AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference

- AI/ML functionality deactivation refers to the process of network deactivating an active functionality.

Proposal 15: If RAN4 agrees to define deactivation delay requirements then RAN4 to consider the endpoint of deactivation delay when UE sends an RRCReconfigurationComplete message in response to RRCReconfiguration for the configuration of new AI/ML/non-AI/ML functionality

Proposal 16: RAN4 should not define any requirements for the fallback delay.

Proposal 17: If RAN4 agrees to define activation delay requirement then RAN4 should consider defining activation delay requirements for periodic CSI reporting in beam management use case.

Observation 6: The number of additional requirements per given UE AI/ML-enabled Feature and Associated ID will depend on their concrete definitions of the functionalities/configurations in the other WGs and the generalization capabilities of the UE implementation solutions, and both require further study in a per use-case manner.

Proposal 18: For the testing of Generalization aspects, RAN4 needs to study the variation in the AI/ML-enabled Feature performance due to the change in test conditions (e.g., propagation conditions) to conclude about the final number of requirements/tests per given AI/ML-enabled Feature and Associated ID combination.

Proposal 19: RAN4 should not create a mixed dataset for generalization testing since the objective is not to test the AI/ML model.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.