3GPP TSG-RAN WG4 Meeting #115	R4-2507430

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	7.21.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Delay requirements for applicable functionality reporting

- AI/ML BM Failure Recovery

- AI/ML BM Pre-activation Monitoring

Discussion

Measurement delay requirements

Following agreement was reached in RAN4#114bis meeting:

Agreement (RAN4 #114bis) - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

The delay component related to CSI-RS measurement period could be addressed for measurement delay. This delay component can also be mapped into the delay due to CSI-RS measurement period component in the below table.

For NW-sided model, the UE would need to report the measured L1-RSRP for both BM-Case1 (spatial domain beam prediction) and BM-Case2 (temporal domain beam prediction) based on measurement delay conditions. For UE-sided model, the UE also needs to follow the measurement delay condition when performing beam prediction.

For delay component related to measurement delay, the CSI report configuration will associate to time requirement defined with measurement period, where the corresponding delay related to measurement should be within measurement period (TL1-RSRP_Measurement_Period_CSI-RS) defined in Clause 9.5.4.2 TS 38.133.

The below table shows the measurement period for L1-RSRP reporting in legacy.

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max( TReport, ceilM*P*N*TCSI-RS ). DRX cycle ≤ 320ms; max( TReport, ceil 1.5*M*P*N)*max⁡( TDRX, TCSI-RS). DRX cycle > 320ms; ceilM*P*N* TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. Treport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3..

For non-DRX configuration, the measurement period (TL1-RSRP_Measurement_Period_CSI-RS) in legacy is defined in the above table, where the parameters M, N and P are defined as follows:

Parameter M:

- For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise.

- For aperiodic CSI-RS resources M=1

Parameter N:

- For periodic CSI-RS and semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1.

- For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam/ N_res_per_set), where N_res_per_set is number of resources in the resource set.

- For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1.

- For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1

Parameter P:

- P=1, when CSI-RS is not overlapped with a GAP and also not overlapped with SMTC occasion.

- P= 11 - TCSI-RSTSMTC period , when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion ( TCSI-RS < TSMTC period ).

In our view, the measurement period for L1-RSRP measurement in legacy could be applied for BM-Case1. A single time instance of measurement reporting needs to be reported for BM-Case1, therefore the parameter M=1 could be the same as in legacy. Similarly, the parameters N=1 and P=1 will be the same as considered in legacy.

For BM-Case2, multiple time instances of measurement reporting need to be reported as observation window. Therefore, the current measurement period for L1-RSRP measurement in legacy could not be applied for AI/ML beam prediction for BM-Case2. Since the current measurement period in Clause 9.5.4.2 does not include when there are multiple time instances measurements corresponding to observation window. However, it might depend on UE to buffer any past measurements. RAN4 should still further discuss on the delay related to measurements especially with respect to measurement instances in observation window.

The new measurement reporting requirements will require to be addressed with inference delay to allow multiple time instances measurements for BM-Case2. The measurement period for L1-RSRP measurement (TL1-RSRP_Measurement_Period_CSI-RS) will need to be enhanced, where the parameter to define the latest measurements (parameter M) in  Clause 9.5.4.2 will need to be redefined. However, parameters N and P could remain the same as in legacy.

- RAN4 to further discuss using the legacy L1-RSRP measurements with the same values of M, N, and P for measurement delay requirements for BM-Case1.

- RAN4 to further discuss whether the measurement period for L1-RSRP measurement will need to be enhanced for BM-Case2, where the parameter M needs to be redefined.

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS and is specified in Table 8.5.6.2-2 of clause 8.5.6.2 in TS 38.133.

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD ´ P ´ N ´ PCBD) ´ TCSI-RS). DRX cycle > 320ms; Ceil(MCBD ´ P ´ N ´ PCBD) ´ TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

- However, with the addition of AI/ML BM feature, the UE is supposed to predict best beams from the Set A beams and the UE is configured to measure only a few beams (i.e. Set B). Therefore, the network will often configure only CSI-RS of the Set B beams, therefore the CSI-RS periodicity of Set A beams may be increased. If the NW uses same CSI-RS periodicity also for candidate RS resources, it may lead to a higher value of TCSI-RS used for the calculation of TEvaluate_CBD_CSI-RS. This will eventually lead to a higher value of  TEvaluate_CBD_CSI-RS, which consequently may cause a delayed beam recovery operation and may cause frequent beam failures A higher value of TCSI-RS in Table 8.5.6.2-2 may lead to a higher value of TEvaluate_CBD_CSI-RS, which consequently may be the cause of frequent beam failures.

- RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

LCM and performance monitoring related requirements

Agreement (RAN4#114bis)

- RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2:

- AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference

- functionality deactivation refers to the process of network deactivating an active functionality

Agreement (RAN4#113)

- Whether transition from AI to non-AI would be UE initiated or network initiated

  - This will be a decision for other groups

  - RAN4 to assess what RRM requirements are needed for each case.

Agreements from RAN4#113, RAN4#114bis, agreements from RAN2#129 meetings and agreements from RAN1#119

RAN2#129 Agreements

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

  - Semi-persistent reporting, activated by MAC CE/DCI

  - Aperiodic CSI reporting, activated by DCI

RAN1#119 Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

The LCM operations for AI/ML BM, e.g., activation, deactivation could be supported by CSI report configuration. These operations can be triggered, for example, based on the performance monitoring output. By reusing the existing capability framework, NW can configure CSI report configurations to enable BM-Case1 or BM-Case2 beam prediction. The CSI reports for both BM-Case1 and BM-Case2 can be based on periodic, semi-persistent, or aperiodic CSI-reporting. The latency of these operations will depend on multiple factors such as,

- Whether CSI configuration (ResourceSet) to be used is already available in the device or not,

- The type of the CSI reporting (periodic, semi-persistent, aperiodic),

- The availability of (previous) measurements that shall be used as an input to the model/functionality inference/prediction,

- Availability of the necessary model(s) in the device,

- The time it takes to substitute one model/functionality with another (the UE may have specialized AI-hardware accelerators to run the inference, which may require loading of the model),

However, it should be clearly specified how much time it takes for the UE to report the new predictions/inference results following the changes indicated by the LCM operation. Without such information NW will not be able to decide when to schedule grants for the reporting of measurements/predictions or data and taking too much time in these operations may also degrade the performance.

In LCM operation, the functionality activation operation is the first step when the UE receives from NW the necessary inference configuration needed for running the inference with the AI/ML functionality available at the UE. Then, other operations, e.g., deactivation, switching and fallback, come after functionality activation operation.

For AI/ML beam prediction reporting enabled by periodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

- delay related to RRC Reconfiguration,

- delay from UE processing time: This may be related to the size of the inference report, model-related pre-processing time or memory limitations on storing models (associated with beam prediction),

- delay due to CSI-RS measurement period.

- For deactivating via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related to RRC Reconfiguration,

  - delay from UE processing time

The time termination point pf the deactivation cam be traced either Option 1: when the first UE inference reports for the new functionality (AI/ML or legacy) to NW, where this delay is included in measurement delay or Option 2: when the UE’s acknowledgment upon reconfiguration (e.g., RRCReconfigComplete) is received at the NW.

For AI/ML beam prediction reporting enabled by semi-persistent CSI-report, the LCM-related latency requirements can be defined as below:

- For activating inference operation of AI/ML-enabled beam prediction via semi-persistent CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from activating semi-persistent CSI resource set and delay for selecting of semipersistent CSI resource set.

  - delay from UE processing time

  - delay due to CSI-RS measurement period.

- For deactivating via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from NW selecting of semi-persistent CSI resource set.

  - delay from UE processing time related

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by aperiodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related activating aperiodic CSI resource set,

  - delay from UE processing time

  - delay due to CSI-RS measurement period.

- For deactivating via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related NW selecting of aperiodic CSI resource set,

  - delay from UE processing time related

  - delay due to CSI-RS measurement.

- For UE-side model in beam management use case, the LCM operations are based on CSI report configuration, in our view, the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

- RAN4 should consider Table 3 as a baseline for further discussion on LCM operations related RRM core requirements.

Table 3: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Deactivation |
| --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay from re-configuration (RRC Reconfiguration) Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay from re-configuration (RRC Reconfiguration) |
| Aperiodic CSI reporting | Delay related activating aperiodic CSI resource set, Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay related activating aperiodic CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

    - Performance monitoring related requirements

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Agreement (RAN4#114bis)

- Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner

- FFS on which use cases the requirement will be defined for

- FFS on the details for each use case in WID

Following agreement was reached in RAN4#114bis meeting on the delay requirements for reporting/monitoring in general aspect.

Working Assumption (RAN 1 #120b)

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

Working Assumption (RAN 1 #120bis)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

For UE-sided model, the delay requirements related to performance monitoring of AI/ML enabled functionality (CSI-ReportConfig) depending on performance monitoring mechanism especially for UE-assisted performance monitoring option. The UE could calculate one of the performance metrics and report it to the NW or UE could report event to NW based on the performance metric.

- Following RAN1#119 bis, there were discussion on Type-1 Option 2 of UE-side model monitoring and the need to determine the set of samples (N instances), where N is the number of valid performance monitoring instances within performance monitoring window. In RAN1#120bis, working assumption has been agreed in which UE calculates performance metric for monitoring, where the performance metric is calculated over N valid performance monitoring instances.

Accordingly, RAN4 will need to define new delay requirements on the reporting of the performance metric(s) related to the duration of the monitoring window when the UE calculates performance monitoring metric/KPI or performs event-based performance monitoring.

- RAN4 should consider the impact in RAN4 requirements for performance monitoring when UE reports performance metric to NW while considering valid number of performance monitoring instances.

Delay requirements for applicable AI/ML functionality

The agreements following RAN2#127bis and RAN2#129bis related to applicable AI/ML functionality for BM use-case are as follows:

Agreement (RAN2#127bis)

Agreement (RAN2#129bis)

Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

RAN2 agreed as working assumption on five steps procedure, intended to cover the main signalling steps from UE capability enquire up to NW provisioning and activation of the ML functionality.

Following RAN2 agreements, the applicability of the inference related parameters should be checked by the UE and/or by the NW before the AI/ML capability/feature is enabled/activated. A configuration for ML-enabled CSI reporting, e.g., a configuration for periodic beam prediction reporting, may become inapplicable. In RAN2#128 agreement, it was agreed that during the inapplicability of functionality period, the output of beam prediction could be unreliable, the UE doesn’t autonomously deactivate the functionality. The NW is expected to deactivate an activate functionality when it receives a report from UE that the functionality is inapplicable. For periodic beam prediction reporting, the UE can report the inapplicability of inference output using UAI, while periodic beam prediction reporting is still activated and reporting. This leads to the transmission of Set A beam prediction become suspended due to inapplicable of functionality, a new UE reporting behaviour, e.g., UE could be configured to report Set B beam measurements is proposed in RAN2 after the UE reports inapplicability of beam prediction reporting configuration. In this case, beam failure would happen regarding the ambiguity of inference reports. This will also cause a delay for beam failure recovery operation if the NW uses the same CSI-RS periodicity of Set A beams for candidate RS resources.

According to RAN2, when an active beam prediction configuration becomes inapplicable, RAN4 should further study (i) delay conditions related inapplicability reporting and (ii) beam failure evaluation period due to inapplicable functionality.

AI/ML BM Failure Recovery

The applicability of the parameters still may not guarantee that accurate inference results, i.e., which may lead to performance degradation and may consequently lead to AI/ML functionality (CSI-ReportConfig) failure. However, the performance degradation and AI/ML functionality failure may be due to temporary circumstances, such as a temporary blockage obstructing signals to the UE, therefore, in this case the AI/ML functionality should not be definitely disabled/de-activated, since the blockage can move away and the signals to the UE may be resumed. In this case, there must be a recovery procedure for AI/ML functionality to recover once the conditions are changed again. On the other hand, the circumstances/conditions may have changed for indefinite period in other cases. The AI/ML functionality should be rather definitely disabled/de-activated until some other event is triggered such as, UE switch off or site change etc.

When the UE is enabled in the recovery mode, the time for recovery starts and the UE will compare the predicted beams with the measurements from the monitoring RS resources. If the prediction accuracy is higher than the predefined threshold for the recovery evaluation period, the AI/ML BM functionality (CSI-ReportConfig) will be considered recovered and hence become applicable again. Therefore, the delay requirement regarding AI/ML functionality (CSI-ReportConfig) recovery success should be further discussed in RAN4.

- RAN4 to further discuss the delay requirement for AI/ML BM functionality recovery when AI/ML BM functionality failure occurs, and UE is enabled for recovery mode.

AI/ML BM Pre-Activation Monitoring

According to RAN2 discussion, it is not agreed yet the exact procedure for the activation of the AI/ML BM functionality, while applicability of inference related parameters is checked by the UE and/or by the NW before the AI/ML BM functionality is enabled/activated. The applicability of the parameters still may not guarantee that accurate prediction results can be provided by the UE. For example, it may happen that AI/ML functionality at the UE is compatible with BS parameters, e.g., beam codebook, antenna configurations, but the prediction quality may be poor due to other conditions that are not part of the applicability parameters. However, when performance evaluation is a part of the AI/ML functionality activation, the performance metric could be calculated over multiple instances and compared to the performance evaluation threshold before it can be concluded that the performance is sufficiently high, and the functionality can be fully activated. The time that UE needs for such performance evaluation to verify the performance before AI/ML functionality is activated would be required. In such scenario, activation and evaluation periods can be defined either by configured pre-defined numbers or as a scaling factor of periodicity or interval in between the RS used for measurements, in the case of semi-persistent or periodic configuration.

- RAN4 to further discuss the delay requirement on AI/ML BM functionality when performance evaluation/monitoring is performed before or during activation.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM core requirements for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Delay requirements for applicable functionality reporting

- AI/ML BM Failure Recovery

- AI/ML BM Pre-activation Monitoring

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 to further discuss using the legacy L1-RSRP measurements with the same values of M, N, and P for measurement delay requirements for BM-Case1.

Proposal 2: RAN4 to further discuss whether the measurement period for L1-RSRP measurement will need to be enhanced for BM-Case2, where the parameter M needs to be redefined.

Proposal 3: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

Proposal 4: For UE-side model in beam management use case, the LCM operations are based on CSI report configuration, in our view, the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 5: RAN4 should consider Table below as a baseline for further discussion on LCM operations related RRM core requirements.

Table: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Deactivation |
| --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay from re-configuration (RRC Reconfiguration) Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay from re-configuration (RRC Reconfiguration) |
| Aperiodic CSI reporting | Delay related activating aperiodic CSI resource set, Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay related activating aperiodic CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

Proposal 6: RAN4 should consider the impact in RAN4 requirements for performance monitoring when UE reports performance metric to NW while considering valid number of performance monitoring instances.

Proposal 7: According to RAN2, when an active beam prediction configuration becomes inapplicable, RAN4 should further study (i) delay conditions related inapplicability reporting and (ii) beam failure evaluation period due to inapplicable functionality.

Proposal 8: RAN4 to further discuss the delay requirement for AI/ML BM functionality recovery when AI/ML BM functionality failure occurs, and UE is enabled for recovery mode.

Proposal 9: RAN4 to further discuss the delay requirement on AI/ML BM functionality when performance evaluation/monitoring is performed before or during activation.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.