# 3GPP TSG-RAN WG4 Meeting #115	R4-2507437

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 		Discussion on BS RF in-channel selectivity requirement for 7 MHz channel bandwidth

Agenda Item:	7.8.4

Document for:	Discussion

# Introduction

According to the WF in R4-2505098, one remaining open issue for BS RF was the ICS requirement.

In this contribution, we provide our view on the ICS requirement derivation for 7MHz channel bandwidth.

# Discussion

## ICS requirement

The following was captured on the ICS requirement in the topic summary in R4-2504680:

Issue 3-3-10: ICS - Proposals: Specify ICS for 7MHz channel BW according to - Option1 (Nokia). BS channel bandwidth (MHz); Subcarrier spacing (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm); Type of interfering signal. 7; 15; G-FR1-A1-7; -100.6; -79.9; DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs. - Option2 (ZTE) – new FRC for 7 MHz channel BW FRC for 7 MHz CBW; Wanted signal mean power (dBm); Interfering signal mean power (dBm); Type of interfering signal. for WA; for MR; for LA; for WA; for MR; for LA; DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs. G-FR1-A1-[X’’]; -99.6; -94.6; -91.6; -79.6; -74.6; -71.6. - Option3 (Ericsson). BS channel bandwidth (MHz); Subcarrier spacing (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm); Type of interfering signal. 5, 7; 15; G-FR1-A1-7; -100.6; -81.4; DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs. - Other option

Based on the options considered last meeting, the following was observed:

1. In Option 1 above, the interfering signal level was derived based on the assumption of 14RB bandwidth. We are guessing, that 14RB bandwidth was taken by scaling interferer bandwidths from the legacy channel bandwidths – see highlight in the extract below:

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |

Based on the above, for 7MHz, one could have come up with the interferer of 14RBs. However, it shall be noted that 14RB is not a valid number of resource blocks for PUSCH signal.

According to TS 38.211 specification, the range of RB values for the PUSCH is limited to a predefined set which is due to the DFT process principles. The bandwidth of the PUSCH in terms of resource blocks () shall fulfil the following equation:

where  is a set of non-negative integers.

For our discussion, and limiting it by the interferer RB values for 5MHz and 10MHz wanted channels (i.e., 10RB and 25RB, respectively), we can derive the range of allowed values to the following:

MRBPUSCH=[10,12,15,16,18,20,24,25]

Observation 1: 14RB is not allowed by RAN1 specification for the bandwidth of the PUSCH.

It is not advised to define RF requirement based on non-realistic system parameters. Therefore, 14RB PUSCH interferer shall be precluded.

Proposal 1: Do not consider 14RB case for ICS interferer signal, as it is not allowed as bandwidth of the PUSCH in terms of resource blocks.

With increased channel bandwidth, it is reasonable to also increase the interferer bandwidth, following the ICS requirement derivation principles in legacy specification. Therefore:

Proposal 2: For 7 MHz wanted channel bandwidth, increase ICS interferer signal bandwidth of the PUSCH (as compared to 5MHz wanted channel reference), and select as 12RB, or 15 RB.

Related interfering signal mean power values would be as below:

| NRB | Interfering signal mean power (dBm) |
| --- | --- |
| 10 | -81.4 |
| 12 | -80.6 |
| 15 | -79.6 |

In case of 12RBs for the interfering signal, the mean power increase compared to the 5MHz reference value is 0.8dB.

In case of 15RBs for the interfering signal, the mean power increase compared to the 5MHz reference value is 1.8dB.

For the time being, it is suggested to proceed with the worst case (1.8dB tightening as compared to 5MHz reference case) and select [15RB] for the ICS interferer PUSCH NRB, subject to further discussion. [12RB] can be considered as fallback solution.

Proposal 3: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [15RB].

2. Option 2 above, is already excluded as it was decided to reuse existing FRC(s) for 7MHz BS testing.

Referring to TS 38.104 Annex A.1 for the ICS FRC, one can find that FRC is explicitly defining NRB for the wanted signal.

Table A.1-1: FRC parameters for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR1-A1-1 | G-FR1-A1-2 | G-FR1-A1-3 | G-FR1-A1-4 | G-FR1-A1-5 | G-FR1-A1-6 | G-FR1-A1-7 | G-FR1-A1-8 | G-FR1-A1-9 | G-FR1-A1-10 | G-FR1-A1-11 | G-FR1-A1-20 | G-FR1-A1-21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 15 | 15 | 15 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 | 15 | 6 | 6 | 24 | 105 | 9 | 12 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 2152 | 984 | 984 | 9224 | 4352 | 2088 | 1320 | 528 | 528 | 2088 | 8968 | 808 | 1032 |
| Transport block CRC (bits) | 16 | 16 | 16 | 24 | 24 | 16 | 16 | 16 | 16 | 16 | 24 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | 24 | - | - | - | - | - | - | 24 | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 2168 | 1000 | 1000 | 4648 | 4376 | 2104 | 1336 | 544 | 544 | 2104 | 4520 | 824 | 1048 |
| Total number of bits per slot | 7200 | 3168 | 3168 | 30528 | 14688 | 6912 | 4320 | 1728 | 1728 | 6912 | 30240 | 2592 | 3456 |
| Total symbols per slot | 3600 | 1584 | 1584 | 15264 | 7344 | 3456 | 2160 | 864 | 864 | 3456 | 15120 | 1296 | 1728 |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with = 2, = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity NOTE 3: Code block size including CRC (bits) equals to in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Therefore, derivation of the ICS wanted signal level is also in-directly defined for 7MHz channel as based on 15RBs (G-FR1-A1-7 from 5MHz channel), for WA BS the wanted signal level would be -100.6 dBm.

Observation 2: Based on FRC reuse, number of RBs for the wanted signal is explicitly defined already.

With the above, an example implementation of the ICS requirement for 7MHz channel bandwidth for WA BS would look as below:

Table 1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -100.6 | WA BS: [-79.6] MR BS: [-74.6] LA BS: [-71.6] | DFT-s-OFDM NR signal, 15 kHz SCS, [15] RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -92.3 | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -101.3 | -81.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -98.8 | -78.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -92.6 | -71.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -98.2 | -78.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -92.7 | -71.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 2: Wide Area BS in-channel selectivity for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 3 |  |  | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 |  |  | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 |  |  | WA BS: [-79.6] MR BS: [-74.6] LA BS: [-71.6] | DFT-s-OFDM NR signal, 15 kHz SCS, [15] RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -124.3 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 3 |  |  | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 |  |  | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 |  |  | WA BS: [-79.6] MR BS: [-74.6] LA BS: [-71.6] | DFT-s-OFDM NR signal, 15 kHz SCS, [15] RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -130.2 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Do not consider 14RB case for ICS interferer signal, as it is not allowed as bandwidth of the PUSCH in terms of resource blocks.

Proposal 2: For 7 MHz wanted channel bandwidth, increase ICS interferer signal bandwidth of the PUSCH (as compared to 5MHz wanted channel reference), and select as 12RB, or 15 RB.

Proposal 3: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [15RB].