3GPP TSG-RAN WG4 Meeting #115 	R4-2507446

Malta, 19 – 23 May, 2025

Title: 	Tx spurious emissions co-existence limits re-evaluation

Agenda Item:	7.13.3.3

Source: 	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In Ran4#114bis the WF [1] was approved with a number of open issues and tasks identified to be further discussed in particular the aggressor BS types to be considered have been identified.

In this paper we further look at the coupling between the identified aggressors and victims using out of band assumptions for the antenna gain calculations.

# Discussion

## 2.1	Evaluation methodology

The existing co-existence (same geographical area) requirements are based on MCL assumptions and as such represent a worst case fixed condition. Looking at BS to BS and BS to UE scenarios:

### 2.1.	BS-BS

As BS are in fixed locations it the path loss between them will not change, BS are usually placed in high locations and as such it is quite likely that the PL will be free space. This at least represents a reasonable worst case

### 2.1.1	Aggressor antenna gain

The aggressor emissions are occurring out of band. The WF prioritises the 2GHz and 3.5GHz aggressors of both AAS and non-AAS type.

In the last meeting we presented in [2] some simple MCL calculations based on the aggressor antenna having the same performance out of band as it does in band. Below we further investigate the potential changes due to the out of band performance of the aggressor antenna.

#### 2.1.1.1	Non AAS Aggressor

For non-AAS we can consider that the signal applied to a passive antenna is coherent (as it’s a single signal) the radiation pattern therefore will be based on the performance of the antenna passive phase shifting networks and the radiation pattern of the individual elements and the loss and radiation efficiency of each of the antenna components (cables, phase shifters, antenna matching, etc..)

The simple element pattern of a dipole is given by:

,

Where, η is the impedance of free space, k is , I0 is the peak current, θ is the angle and L is the dipole length.

Figure 2.1-1 shows the radiation pattern of a dipole in elevation at the victim frequency of 6.4GHz when the dipole is designed for 2GHz and for 3.5GHz

Figure 2.1.1.1-1 Dipole out of band elevation gain pattern (normalised)

As the dipole effectively becomes larger the radiation pattern splits into lobes. Whilst this is a simplistic analysis (and does not importantly take into account any increase in loss or decrease in radiation efficiency) it gives a good idea of how the element radiation pattern degrades as frequency increases.

The array pattern also degrades as the frequency increases, for example: a typical passive BS antenna with 10 elements separated at 0.9λ when investigated at different frequency offsets has the following response:

Figure 2.1.1.1-2 Array response out of band

The array is effectively sub-sampled and large grating lobes appear, it can be noted that each is significantly narrower so the peak gain is not significantly lower.

Finally if we consider the whole passive antenna array with the modified element pattern ad the modified array pattern:

Figure 2.1.1.1-3 Composite array response out of band

Considering the BS to BS case where the aggressor is considered to be mechanically tilted to the edge of the cell the BS-BS angle is approx. 4deg.

Figure 2.1.1.1-4 BS to BS angle

For both the out of band cases the composite beam pattern (ignoring losses) has significantly lower directional gain than the in band case.

Observation 1: The out of band passive antenna has significantly less directional BS-BS antenna gain than the in-band case.

Considering the effects we see to both the element pattern and the array pattern it is unlikely the BS-BS antenna gain will ever be worse than the in band case (although this is not true in all directions)

Observation 2: considering the effects to the element and array patterns the BS-BS antenna gain will not be worse than in band case.

It is also worth repeating that this does not take into account the additional losses due to the cables, phase shifters and element radiation efficiency.

#### 2.1.1.2	AAS Aggressor

The non-AAS antenna assumptions were based on a typical single column passive antenna, it would be expected that an AAS will have multiple columns with the ability to steer in both dimensions.

A typical 2/3.5GHz AAS is defined in 38.803 as:

Table 2.1.1.2-1: 2/3.5 GHz AAS antenna

| elevation | No elements/sub-arrays | 4 |  |
| --- | --- | --- | --- |
|  | elements per sub-array | 3 |  |
|  | element separation | 0.7 | λ |
| Azimuth | No elements | 8 |  |
|  | element separation | 0.5 | λ |
| Directivity (zero steer) |  | 26.3 | dBi |
| Vertical Beam width |  | 6 | deg |
| Horizontal Beam width |  | 12.7 | deg |

If the aggressor is an AAS then the breakdown of the element pattern is the same but the array pattern is now dominated not by passive phase shifters (working out of band) but by the nature of the unwanted signal from the transmitter.

In most cases the out of band noise will be uncorrelated, the possible exception to this would be if the victim band is a harmonic of the aggressor. The bands selected have a small change of that occurrence.

3rd harmonic of band n1 (2GHz) is 6330 to 6510 MHz

2nd harmonic of band n48 (3.5GHz) is 7100 to 4700 MHz

Both these overlap with n104 which is 6425 to 7125MHz

Given that the victim band in this case is a long way out of band it will be in the stop band of any transmit filter. The stop bands of multiple filters are unlikely to have correlated phase responses however so it is still very likely that the unwanted signals will be decorrelated. However if we look at the worst case and the unwanted emissions are coherent harmonics of the wanted signal then they will still form a beam albeit the array will still be sparsely sampled and element pattern will be affected as in the non-AAS case. The beam pattern therefore will be affected in a similar way to the non-AAS antenna although in this case the vertical spacing will be smaller as suits an AAS so the effect on the array pattern may not be as extreme.

The other difference with an AAS aggressor is that the beam will be steered rather than be a fixed pattern. In such cases we can consider if we are to consider a worst case MCL type assumption or a statistical assumption.

If a beam is formed and steered to the edge of the cell then this will give a similar scenario to the non-AAS case however the AAS antenna assumption is different.

Observation 3: worst case for a coherent AAS harmonic emission a coherent beam is steered towards the edge of the cell.

If we consider the power distribution when the beam is being steered it is a well-known result that the average power distribution of AS BS with randomly located UE’s that the power distribution looks like the element pattern. As such we can assume that the power distribution of the harmonic will also look like the out of band element pattern. This is generally what the non-correlated power distribution will look like also.

Looking at the element patterns in figure 2.1-1 the biggest risk is the 2GHz band where the 3rd harmonic response has a lobe in the centre of the response. This lobe however is of significantly lower gain than the array and also of the in-band element pattern

Observation 4: The average power distribution will take the shape of the element pattern

### 2.1.2	Victim antenna gain

In this case the victim is an AAS operating at 6.4GHz and the antenna gain will be dependent on the victim network UE location. The most vulnerable links will be those operating at the edge of the cell which is also potentially the worst direction to be pointing at the aggressor BS.

A typical sub-array AAS antenna operating at this frequency is the 24 (3 element sub-arrays) by 16 antenna shown in table 2.1-1.

Table 2.1.2-1: 6.4GHz antenna

| elevation | No elements/sub-arrays | 8 |  |
| --- | --- | --- | --- |
|  | elements per sub-array | 3 |  |
|  | element separation | 0.7 | λ |
| Azimuth | No elements | 16 |  |
|  | element separation | 0.5 | λ |
| Directivity (zero steer) |  | 32.4 | dBi |
| Vertical Beam width |  | 3 | deg |
| Horizontal Beam width |  | 6.4 | deg |

As in this case we are considering the in band gain of an AAS there is potentially a statistical element to the gain assumptions.

Worst case we can consider that the victim network UE is at the edge of the cell and that the receiver beam is pointed at it. This will be a similar situation to figure 2.1-4.

If we consider a statistical approach then once again we can be relatively confident that the average radiation pattern will match the element/sub-array pattern.

### 2.1.3	Calculations

The 2GHz system with the potential 3rd harmonic in the victim band is clearly the worst case as it has antenna gain in the interference direction so we will concentrate on this case.

Based on the observations and approximations and the antenna gain values in the annex we have:

Table 2.1.3-1: Non-AAS out of band antenna gain assumptions

|  | Tx 2GHz non AAS @ 6.4GHz |  | Tx 3.5GHz non AAS @ 6.4GHz |  | Rx - 6.4GHz AAS in-band |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Correlated | Non-correlated | Correlated | Non-correlated | Correlated | Non-correlated |  |
| el directivity | -2.08 | 1.85 | -11.8 | -20 | 2.69 | 6.36 | dBi |
| az directivity | 4.59 | 4.59 | 4.59 | 4.59 | 16.26 | 3.63 | dBi |
| Loss | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | dB |
| Gain | 1.01 | 4.94 | -8.71 | -16.91 | 17.45 | 8.49 | dBi |

Table 2.1.3-2: AAS out of band antenna gain assumptions

|  | Tx 2GHz AAS @ 6.4GHz |  | Tx 3.5GHz AAS @ 6.4GHz |  | Rx - 6.4GHz AAS in-band |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Correlated | Non-correlated | Correlated | Non-correlated | Correlated | Non-correlated |  |
| el directivity | 2.38 | 7.22 | -16.3 | -16.3 | 2.69 | 6.36 | dBi |
| az directivity | 13.32 | 3.51 | 12.79 | 3.51 | 16.26 | 3.63 | dBi |
| Loss | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | dB |
| Gain | 14.2 | 9.23 | -5.01 | -14.29 | 17.45 | 8.49 | dBi |

If we then apply these to the MCL calculations

Table 2.1.3-3: 2GHz aggressor MCL budget AAS and non-AAS

|  | 2 GHz Non AAS aggressor |  | 2 GHz AAS aggressor |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | Correlated | Non-Correlated | Correlated | Non-Correlated |  |
| freq | 6.4 | 6.4 | 6.4 | 6.4 | GHz |
| d | 288 | 288 | 288 | 288 | m |
| FSPL | 97.8 | 97.8 | 97.8 | 97.8 | dB |
| G_TX | 1.01 | 4.94 | 11.44 | 5.05 | dBi |
| G_Rx | 17.45 | 8.49 | 17.45 | 8.49 | dBi |
| MCL | 79.3 | 84.3 | 68.9 | 84.2 | dB |

Table 2.1.3-4: 3.5GHz aggressor MCL budget AAS and non-AAS

|  | 3.5 GHz Non AAS aggressor |  | 3.5 GHz AAS aggressor |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | Correlated | Non-Correlated | Correlated | Non-Correlated |  |  |
| freq | 6.4 | 6.4 | 6.4 | 6.4 | GHz |  |
| d | 288 | 288 | 288 | 288 | m |  |
| FSPL | 97.8 | 97.8 | 97.8 | 97.8 | dB |  |
| G_TX | -8.71 | -16.91 | -5.01 | -14.29 | dBi |  |
| G_Rx | 17.45 | 8.49 | 17.45 | 8.49 | dBi |  |
| MCL | 89.0 | 106.2 | 85.3 | 103.6 | dB |  |

The worst case is for the AAS correlated set of assumptions for the 2HGHz aggressor (note this assumes the receiver beam is pointing directly at the aggressor BS) where the MCL is 69dB (only 2dB more than the original assumption).

These numbers are quite similar, taking the correlated value we have an interferer at 6.4GHz of:

Pco-existence_UL=-174+60+6+69-7=-46dBm/MHz

(note BS NF is 6dB @ 6.4GHz)

This is potentially only a 3dB relaxation over the current requirement. Once again it should be noted this is assuming worst case correlated conditions and does not include any potential additional loss in the Tx antenna due to the frequency offset (cables, phase shifters, radiation efficiency etc).

Observation 5: with absolute worst case assumptions the maximum allowable interferer to maintain BS-BS co-existence performance with an AAS aggressor is -46dBm

## 2.2	BS-UE

The background for the current BS to UE requirement is not quite as clear as the BS-BS however based on information in 25.942 and other sources we can summarise that it is based on the MCL between the BS and the closest UE for a medium range (Micro) BS) with a desensitisation of the UE of 3dB.

Pco-existence_DL=-174+60+9+53-0=-52dBm/MHz

The antenna gains for the BS Tx and UE Rx are included in the MCL figure along with the path loss.

In the AAS BS class definitions conducted systems (non-AAS and AAS type 1-H) use an MCL value to describe the minimum coupling between the BS and the UE, this includes the antenna gain. For OTA systems where the BS antenna is included in the system a minimum distance is used.

In both cases the antenna gain is assumed to be zero. The UE gain is generally assumed to be omni directional and hence is zero. The BS case is a little more complicated;

For the wide area case the minimum distance is 35m which is at an angle of 34deg from the BS tower (25m tower with 1.5m UE), assuming a 4deg tilt to the edge of the cell then this is an offset of 30 deg. At this angle we are way off the main beam of the BS antenna radiation pattern and the gain is likely less than zero.

For the medium range case the BS antenna gain is assumed lower and the mounting location is lower a street level location is assumed to be at 15m (25.942 sub-clause 4.1.1.2), in this case the angle is 70deg (66 deg assuming the 4deg tilt) so once again way off the main beam and likely to be significantly less than zero. In both cases 0dB BS antenna gain is a reasonable worst case simplification.

So the minimum UE distances are equivalent to the MCL as a free space path loss.

Wide area minimum distance = 35m (23m on hypotenuse, FSPL@2GHz = 70dB)

Medium range minimum distance = 5m (FSPL@2GHz = 52.5dB)

Using the same scenarios with a 6.4GHz victim UE we can assume:

The victim UE operating at 6.4GHz will have an in-band antenna gain assumption of 0dB

The FSPL at 6.4GHz will be:

FSPLWA = 81dB

FSPLMR = 62.5dB

Looking at the aggressor BS out of band antenna gain patterns, as this is for BS to UE and hence not a permanent interference scenario (as both the victim UE and the aggressor UE and hence the BS beam direction move) the average beam pattern (or the sub-array pattern) is more useful.

Comparing the in-band case (where a 0dB assumption had previously been made) with the 2GHz and 3.5GHz out of band cases we have the following radiation patterns.

Figure 2.2-1: Spurious antenna radiation patterns for 2GHz and 3.5GHz AAS

It can be seen from the in-band result that the 0dB assumption of BS gain under is much higher than the pattern suggests is necessary with the exact value being closer to -10dB. The markers in figure 2.1-5 for the spurious responses of the 2GHz and 3.5GHz BS antennas are shown at the nearest peak in the response to ensure a worst case is identified.

The worst case BS antenna gain to the closest UEs is hence:

Spurious antenna gain at 6.4GHz for 2GHz BS = 0.3+ 4.2 = 4.5dB

Spurious antenna gain at 6.4GHz for 3.5GHz BS = 5.5 + 4.2 = 9.7dB

Considering only the worst case band of 3.5GHz the MCL is now:

MCL= -GBS + FSPLMR - GUE = -9.7 + 62.5 - 0 = 52.8 dB

This is almost exact the same MCL figure a currently assumed.

Observation 6: The worst case MCL for the 6.4GHz victim with a 3.5GHz aggressor is 53dB

Observation 7: The MCL is the same as the existing requirement.

## 2.3	Discussion on results

In the analysis carried out we have used simulated antenna radiation patterns to estimate gain at spurious frequencies in the somewhat arbitrary directions dictated by the defined scenarios, small changes in assumptions have the potential to change the results, however we have the rather perhaps surprising result that when considering worst case scenarios both the BS to BS MCL (and hence maximum allowable interferer level) and the BS-UE MCL are very similar to the existing requirements.

However it is perhaps not so surprising when we consider the scenarios.

- As frequency increases the free space path loss increases by 6dB an octave, at 6.4GHz the FSPL is 10dB higher than at 2GHz.

- We aim to keep cell sizes constant as frequency increases for practical deployment reasons.

- To account for higher path loss with higher frequency and the same cell size we compensate with high antenna gain.

- If implementation losses and radiation efficiency are ignored (as we have done here) and the worst case correlated or non-corelated response identified then that higher gain exists in the spurious responses of the antennas also.

- As with the in-band case the increased path loss and the increased antenna gain effectively cancel each other out.

Whilst this reasoning is somewhat simplistic it explains why the more detailed analysis comes up with the result that the existing MCL values are the same for the higher FR1 bands.

Observation 8: The similar MCL values can be simply explained by the increase in FSPL being cancelled out by the increase in antenna gain.

It must also be pointed out that the original requirement was based on a fixed aggressor antenna radiation pattern as such interference would be present at all times, so assuming worst case antennas gains was more valid for AAS systems when we consider the worst-case correlated gains these occur only at the extreme of the CDF curve. The difference is particularly relevant when considering the victim AAS BS, the correlated gain used in the calculations only occurs if the UE in the wanted network aligns with the aggressor BS direction, something that only occurs occasionally. Comparing non-AAS methodology with AAS methodology therefore requires some assumptions to be made and it could be argued the worst-case assumptions we have used are to strict. However the worst-case analysis show at least the current requirement levels are safe.

Observation 9: Due to the statistical nature of the AAS beam pattern direct comparison with non-AAS MCL requires some assumptions. Worst case assumptions used are potentially to strict.

It is also the case that in reality that the out of band antenna gain will drop off significantly due to factors not considered here such as losses in the physical antenna and the radiation efficiency of the antenna elements. Further analysis of these effect may yield some additional margin but they tend to be very implementation specific and difficult to agree on.

Once again the results here very much indicate a worst case and show that the existing values represent a safe requirement.

Observation 10: Analysis does not include losses due to increased loss and decreased radiation efficiency and as such show the existing requirement represents a safe worst case.

Considering the closeness of the MCL values identified and the potential complexity of implementing a frequency (aggressor and victim) based requirement for co-existence emissions levels maintaining the existing requirements is preferred.

Proposal 1: Maintain the existing co-existence in same geographical area emissions levels.

# Summary

We have examined the spurious responses of AAS and non-AAS 2 and 3GHz aggressor systems at a victim frequency of 6.4GHz and made the following observations:

BS-BS Non-AAS aggressor

Observation 1: The out of band passive antenna has significantly less directional BS-BS antenna gain than the in-band case.

Observation 2: considering the effects to the element and array patterns the BS-BS antenna gain will not be worse than in band case.

BS-BS AAS aggressor

Observation 3: worst case for a coherent AAS harmonic emission a coherent beam is steered towards the edge of the cell.

Observation 4: The average power distribution will take the shape of the element pattern

Observation 5: with absolute worst case assumptions the maximum allowable interferer to maintain BS-BS co-existence performance with an AAS aggressor is -46dBm

BS-UE

Observation 6: The worst case MCL for the 6.4GHz victim with a 3.5GHz aggressor is 53dB

Observation 7: The MCL is the same as the existing requirement.

It has been discussed that

Observation 8: The similar MCL values can be simply explained by the increase in FSPL being cancelled out by the increase in antenna gain.

The results here very much indicate a worst case and show that the existing values represent a safe requirement.

Observation 9: Due to the statistical nature of the AAS beam pattern direct comparison with non-AAS MCL requires some assumptions. Worst case assumptions used are potentially to strict.

Observation 10: Analysis doe not include losses due to increased loss and decreased radiation efficiency and as such show the existing requirement represents a safe worst case.

Hence

Proposal 1: Maintain the existing co-existence in same geographical area emissions levels.

# References

[1]	R4-2504769 Way Forward for Tx spurious emissions co-existence limits re-evaluation Huawei Technologies Co., Ltd.

[2]	R4-2504298 Transmitter co-existence spurious emission requirements	 Huawei

# ANNEX – Antenna gain plots

The following assumptions are made from the estimated antenna array responses for corelated and uncorrelated patterns in-band and out of band

## Non-AAS single column antenna

10 element 0.9λ vertical spacing

As the model uses a dipole for the element it does not cover what happens in azimuth, a patch antenna would most likely split in the same way the diploe elevation pattern does but as a worst case the element pattern in azimuth has been kept the same as the in band pattern.

## AAS antenna for 2 and 3.5GHz

8x8 no sub arrays, vertical spacing 0.8λ horizontal spacing 0.5λ.

## AAS antenna for 6.4GHz

24x16 element antenna with sub arrays as described in table 2.1-1.

As this is the victim antenna only the in band response is of interest.

## Summary

Taking the marker points from all the responses we have:

|  | Azimuth @ 30deg |  | Elevation @ 4deg |  | Unit | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Correlated | Non-correlated | Correlated | Non-correlated |  |  |
| Non AAS antenna |  |  |  |  |  |  |
| in band | 4.59 | 4.59 | 5.6 | 3.74 | dBi |  |
| 2GHz antenna at 6.4GHz | 4.59 | 4.59 | -2.08 | 1.85 | dBi | correlated value from peak of 1st sidelobe |
| 3.5GHz antenna at 6.4GHz | 4.59 | 4.59 | -11.8 | -20 | dBi | estimate - gain is very low. |
| 2GHz and 3GHZ AAS antenna |  |  |  |  |  |  |
| in band | 13.19 | 3.51 | 7.02 | 6.76 | dBi |  |
| 2GHz antenna at 6.4GHz | 13.32 | 3.51 | 2.38 | 7.22 | dBi | correlated value from peak of 1st sidelobe |
| 3.5GHz antenna at 6.4GHz | 12.79 | 3.51 | -16.3* | -16.3 | dBi | *estimate - gain is very low use non-correlated value |
| 6.4GHZ AAS antenna |  |  |  |  |  |  |
| in band | 16.26 | 3.63 | 2.69 | 6.36 | dBi |  |