3GPP TSG-RAN4 Meeting #115	R4-2507467

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0397 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (AAS_BS_LTE_UTRA-Perf) CR to TS 37.145-2: removal of outstanding FFS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Multiple outstanding FFS are removed, as not allowed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multiple outstanding FFS are removed. Missing Test Requirement derivations are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FFS would remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.2, 6.8.4, 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 4.15.2	Co-location test antenna

#### 4.15.2.1	General

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

A CLTA is a practical antenna which can be used to test conformance to the co-location requirements.

#### 4.15.2.2	Co-location test antenna characteristics

A co-location test antenna is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. A CLTA shall comply to the requirements specified in Table 4.15.2.2-1. Translation of the requirements to other test antennas is not precuded but suitable translations between the co-location reference antenna and test antennas must be provided to demonstrate that the method is within the specified MU.

The CLTAs are suitable for testing AAS BSs implemented with a planar antenna array. The method for testing AAS BS with other antenna array implementations are not covered by the present release of this specification.

------------------------------ Next modified section ------------------------------

### 6.8.4	Method of test

#### 6.8.4.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	M; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see clause 4.12.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

Directions to be tested for:

------------------------------ Next modified section ------------------------------

## 7.7	OTA Receiver spurious emissions

### 7.7.1	Definition and applicability

The receiver spurious emission requirement is the power of the emissions radiated from the antenna array from a receiver unit. For an OTA AAS BS operating in FDD, OTA RX spurious emissions requirement do not apply as they are superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

NOTE:	The OTA receiver spurious emission requirement applicability for the AAS BS with the RX-only capabilities are not covered by the present release of this specification..

For an OTA AAS BS operating in TDD, the OTA receiver spurious emissions requirement applies during the transmitter OFF period only.

For RX only multi-band RIB, the RX spurious emissions requirements are subject to exclusion zones in each supported operating band.

------------------------------ Next modified section ------------------------------

Table C.2-1: Derivation of Test Requirements (OTA transmitter tests)

| Test | Minimum Requirement in TS 37.105 [6] | Test Tolerance (TT) | Test Requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 37.105 [6], clause 9.2 | 1.1 dB, f ≤ 3.0 GHz 1.3 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.2 Radiated transmit power (extreme conditions) | See TS 37.105 [6], clause 9.2 | 2.5 dB, f ≤ 3.0 GHz 2.6 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3.2 OTA Maximum output power | See TS 37.105 [6], clause 9.3.2 | 1.4 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 | Formula: Upper limit + TT, Lower limit – TT |
| 6.3.3 OTA E-UTRA DL RS power | See TS 37.105 [6], clause 9.3.3 | 1.3 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4.2 OTA UTRA Inner loop power control in the downlink | See TS 37.105 [6], clause 9.4.2 | 0.1 dB | Formula: Upper limit + TT, Lower limit – TT |
| 6.4.3 OTA Power control dynamic range | See TS 37.105 [6], clause 9.4.3 | 1.1 dB | Formula: Upper limit - TT, Lower limit + TT |
| 6.4.4 OTA Total power dynamic range | See TS 37.105 [6], clause 9.4.4 | 0.3 dB UTRA 0.4 dB E-UTRA & NR | Formula: limit - TT |
| 6.4.5 OTA IPDL time mask | See TS 37.105 [6], clause 9.4.5 | 0.7 dB | Formula: limit - TT |
| 6.5 OTA Transmit ON/OFF power | See TS 37.105 [6], clause 9.5 | 3.4 dB, f ≤ 3.0 GHz 3.6 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: limit - TT |
| 6.6.2 OTA Frequency Error | See TS 37.105 [6] clause 9.6.2 | 12 Hz | Limit + TT |
| 6.6.3 OTA Time alignment error | See TS 37.105 [6], clause 9.6.3 | 25 ns | Limit + TT |
| 6.6.4 OTA Modulation quality | See TS 37.105 [6], clause 9.6.3 | 1 % | Limit + TT |
| 6.7.2 OTA occupied bandwidth | See TS 37.105 [6], clause 9.7.2 | 0 Hz | Limit |
| 6.7.3 OTA Adjacent Channel Leakage power Ratio | See TS 37.105 [6], clause 9.7.3 | ACLR / CACLR 1.0 dB, f ≤ 3.0 GHz 1.2 dB, 3.0 GHz < f ≤ 4.2 Absolute limit 0 dB | Limit + TT |
| 6.7.4 OTA Spectrum emission mask | See TS 37.105 [6], clause 9.6.3 | 1.8 dB, f ≤ 3.0 GHz 2.0 dB, 3.0 GHz < f ≤ 4.2 GHz | Limit + TT |
| 6.7.5 OTA Operating band unwanted emission | See TS 37.105 [6], clause 9.6.3 | close to carrier (<10 MHz) 1.8 dB, f ≤ 3.0 GHz 2.0 dB, 3.0 GHz < f ≤ 4.2 GHz far from carrier ( ≥10 MHz) 0dB Additional limits for bands n50, n51, n74, n75, n76: 0 dB | Limit + TT |
| 6.7.6.2 OTA Transmitter spurious emissions, Mandatory Requirements | See TS 37.105 [6], clause 9.7.6.2.1, 9.7.6.3.1 and 9.7.6.4.1 | 0dB | Limit |
| 6.7.6.3 Transmitter spurious emissions, Protection of BS receiver | See TS 37.105 [6], clause 9.7.6.2.2, 9.7.6.3.2 and 9.7.6.4.2 | 3.1 dB, f ≤ 3.0 GHz 3.3 dB, 3.0 GHz < f ≤ 4.2 GHz | Limit + TT |
| 6.7.6.4 Transmitter spurious emissions, Additional spurious emission requirements | See TS 37.105 [6], clause 9.7.6.2.3, 9.7.6.3.3 and 9.7.6.4.3 | 2.6 dB, f ≤ 3.0 GHz 3.0 dB, 3.0 GHz < f ≤ 4.2 GHz 3.5 dB, 4.2 GHz < f ≤ 6.0 GHz For co-existence with PHS and public safety bands. 0 dB Additional limits for bands n50, n51, n74, n75, n76: 0 dB | Limit + TT |
| 6.7.6.5 Transmitter spurious emissions, Co-location | See TS 37.105 [6], clause 9.7.6.2.4, 9.7.6.3.4 and 9.7.6.4.4 | 3.1 dB, f ≤ 3.0 GHz 3.3 dB, 3.0 GHz < f ≤ 4.2 GHz 3.4 dB, 4.2 GHz < f ≤ 6.0 GHz | Limit + TT |
| 6.8 OTA Transmitter intermodulation | See TS 37.105 [6] | 0dB | Limit |

----------------------------- End of modified section -------------------------