3GPP TSG-RAN WG4 Meeting #115                                                               R4-2507486

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:	On the modification of existing RX requirements for SBFD-capable BS

Agenda item:			7.25.3.3

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. And in the RAN4#111, #112 and #112bis, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8] [10] [12] and [14]. In this contribution, accordingly, we would like to provide our analysis and viewpoints on the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing RX requirement for SBFD

### 2.1 OTA sensitivity

As the outcome from RAN4 study item, regarding reference sensitivity requirement for SBFD-capable BS, due to the self-interference caused internally to receiver side, RAN4 reached the following consensus [2]:

- For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed. - Otherwise, OTA sensitivity requirement could be derived based on the following equation: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase. - The following aspects need more discussion during a WI phase - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

Based on the agreements achieved in RAN4#112 [8] and RAN4#113 [12], OTA sensitivity is agreed to be defined with 0.7dB degradation on top of existing minimum requirements with only self-interference:

Agreement from RAN4#112: - OTA sensitivity is measured with only self-interference. FFS on the OTA sensitivity degradation within [0.5~1.0] dB. Agreement from RAN4#113: - 0.7 dB degradation on top of existing minimum requirements for OTA sensitivity.

Based on the above agreements, the following text proposal is given for the group to discuss how the 0.7dB degradation to OTA sensitivity requirement should be implemented in TS38.104. In our accompanying contribution on the general aspects of BS RF requirement, the OTA sensitivity requirement shall be provided in clause 12.7.2.

Proposal 1: The following text proposal is adopted for OTA sensitivity requirement:

=================== Start of Text Proposal ===================

## 12.6.2	OTA sensitivity for SBFD

### 12.6.2.1	BS type 1-H and BS type 1-O

#### 12.6.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

#### 12.6.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and BS channel bandwidth in clause 7.2 shall be interpreted as UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

=================== End of Text Proposal ===================

### 2.2 Dynamic range

Dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth. In this condition a throughput requirement shall be met for a specified reference measurement channel. For the requirement of dynamic range, the following agreements is achieved in the study phase [2]:

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase.

In RAN4#114, the following agreement is achieved for dynamic range requirement:

WF: - The dynamic range requirements of FR1 can reuse existing requirement. - For FR2, no dynamic range requirement is needed.

By taking the above agreement into account, the following text proposal can be provided for dynamic range requirement:

Proposal 2: The following text proposal is adopted for dynamic range requirement (by taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.3.3	Dynamic range

#### 12.3.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The same requirement as clause 7.3.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be interpreted as SBFD UL subband bandwidth.

=================== End of Text Proposal ===================

### 2.3 ACS requirement

For the requirement of in-band selectivity (more specifically, adjacent channel selectivity (ACS)) and in-band blocking, the following agreements is achieved in the study phase [2]:

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

In last RAN4 meeting (RAN4#114bis), there is still deadlock for this topic, with at least the following four options on the table:

- FFS on this issue - Option 1: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD. - Option 2: Prioritize discussing the scenarios under non-colocation case first. - The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked. - Option 3: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level. - Option 4: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD. - Derive ACS requirement level based on the simulation results using 5% GS in SI. - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS

Based on our understanding, for ACS requirement, by adopting the 9dB offset to the in-band blocking interference level is reasonable proposal, while the frequency configuration of the NR interference signal shall be reused. Furthermore, it should be noted that the offset shall be 12dB for band n104.

Proposal 3: In general, revised Option 1 shall be adopted:

- Option 1a: The requirement of ACS shall be applicable to Rel-19 SBFD-capable BS.

  - the interference level keeps 12dB offset for n104 and 9 dB offset for other bands to the general in-band blocking interference level defined for SBFD

  - no change on the ACS interferer frequency offset values specified in Table 7.4.1.2-2 for conducted requirement and Table 10.5.1.2-2 for OTA requirement.

### 2.4 In-band blocking requirement

For in-band blocking requirement, in on our analysis given in [R4-2411639], the levels of adjacent channel gNB DL power level for different scenario have been provided. It can be observed that at least for Scenario 6 (FR2 UMa-UMa), 99% of the levels of adjacent channel gNB DL power is smaller than the existing in-band blocking requirement. Therefore, we suggest the group to conclude with the following proposal firstly:

Proposal 4: For FR2 WA BS capable of SBFD operation, the existing in-band blocking requirement shall be reused.

For the in-band blocking requirement for FR1 WA BS (potentially also for FR1 MR BS), it is still FFS to have a unified or separate blocking requirement, while another option is from us, i.e., manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting). Since the discussion for TX ACLR and OBUE requirements could be separated into (1) when coexisting with legacy TDD system in adjacent channel and (2) when coexisting with new SBFD system in adjacent channel, and we think it is not appropriate to preclude the option of separate requirements if the TX requirement for coexisting with new SBFD system in the adjacent channel is under consideration. Because if this new tightened requirement is introduced, the CLI from adjacent channel could be greatly mitigated. Since this issue has been discussed for several meetings, we think the Option 3, i.e, manufacture declared level of interference in the specified range from the system-level evaluation could be a reasonable compromised method.

Proposal 5: RAN4 consider the following compromise solution to resolve the deadlock for in-band blocking requirement:

- Two sets of requirements defined, by considering 100% and 20% grid shifts to derive in-band blocking level and depends on manufacture-declaration for which requirement is supported by gNB product.

### 2.5 In-channel selectivity

For in-channel selectivity requirement, the following agreements is achieved in the study phase [2]:

Regarding the receiver in-channel selectivity requirement, the requirement shall be studied based on that the wanted signal and UL interfering signal shall be located in the configured UL subband, and the wanted signal and interfering signal levels could be further studied in the WI phase.

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. It should be noted that ICS requirement is introduced to consider the interference at the receiving bandwidth. After SBFD feature is introduced, the receiver functionality and performance is guaranteed by the OTA sensitivity performance in which the subband DL self-interference is present and by in-band blocking requirement in which the subband DL interference from neighboring cells are present. Therefore we see no needs to consider SBFD DL subband in the in-channel selectivity testing.

Proposal 6: The existing in-channel selectivity (ICS) requirement shall be applicable to SBFD-capable BS, while the receiver functionality/performance is guaranteed by other requirements/tests for the presence of interference from DL subband.

# 3. Conclusion

In this contribution, we provided the viewpoints on the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Proposal 1: The following text proposal is adopted for OTA sensitivity requirement:

=================== Start of Text Proposal ===================

## 12.6.2	OTA sensitivity for SBFD

### 12.6.2.1	BS type 1-H and BS type 1-O

#### 12.6.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

#### 12.6.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and BS channel bandwidth in clause 7.2 shall be interpreted as UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

=================== End of Text Proposal ===================

Proposal 2: The following text proposal is adopted for dynamic range requirement (by taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.3.3	Dynamic range

#### 12.3.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The same requirement as clause 7.3.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be interpreted as SBFD UL subband bandwidth.

=================== End of Text Proposal ===================

Proposal 3: In general, revised Option 1 shall be adopted:

- Option 1a: The requirement of ACS shall be applicable to Rel-19 SBFD-capable BS.

  - the interference level keeps 12dB offset for n104 and 9 dB offset for other bands to the general in-band blocking interference level defined for SBFD

  - no change on the ACS interferer frequency offset values specified in Table 7.4.1.2-2 for conducted requirement and Table 10.5.1.2-2 for OTA requirement.

Proposal 4: For FR2 WA BS capable of SBFD operation, the existing in-band blocking requirement shall be reused.

Proposal 5: RAN4 consider the following compromise solution to resolve the deadlock for in-band blocking requirement:

- Two sets of requirements defined, by considering 100% and 20% grid shifts to derive in-band blocking level and depends on manufacture-declaration for which requirement is supported by gNB product.

Proposal 6: The existing in-channel selectivity (ICS) requirement shall be applicable to SBFD-capable BS, while the receiver functionality/performance is guaranteed by other requirements/tests for the presence of interference from DL subband.

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411639, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.

[9] R4-2416500, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#112bis.

[10] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#112bis.

[11] R4-2418100, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#113.

[12] R4-2419908, “Way Forward for [113][307] NR_duplex_evo_BSRF”, Huawei, RAN4#113.

[13] R4-2501970, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#114.

[14] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114.

[15] R4-2504221, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#114bis.

[16] R4-2504752, “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114bis.