3GPP TSG-RAN-WG4 Meeting #115 	R4-2507488

# Saint Julian's, Malta, 19th – 23rd May, 2025

# Title:	Input on making 3 MHz channel bandwidth mandatory in Release 19

# Source:	Rakuten Mobile Inc , T-Mobile US , Southern Co,Anterix

# Agenda Item:	6.12.2

# Document for:	Approval

# Introduction

At RAN4#114bis, a Way Forward on handling optional channel bandwidth across releases [1] was approved. This contribution provides Rakuten Mobiles input on WF1 from [1].

# Discussion

[1] included the following agreement:

Agreement 1:

- Change across Releases from “optional support” to “mandatory support” is evaluated on a case-by-case basis. Interested companies are invited to share their views on the following options for the following CBW:

- 3MHz CBW:

  - option 1: remains optional across Releases for all bands except for band n106 where mandatory support is kept across Releases,

  - option 2: changes to mandatory support in Release 19, but this CBW is not accounted for MSD analyses.

The supporting companies propose Option 2: “Changes to mandatory support in Rel-19 but this CBW is not accounted for MSD analysis” based on the following rationale:

- Continuity with the LTE ecosystem:  Many operators rely on 2×3 MHz allocations for LTE. Mandatory 3 MHz CBW in NR enables direct spectrum migration without requiring replanning, consistent with the precedent set for Band n106.

- Support for RedCap and narrowband use cases: 3 MHz CBW is well-matched to the requirements of Reduced Capability (RedCap) UEs and narrowband services, both expected to expand in the Rel-18 to Rel-20 timeframe.

- Efficient use of spectrum in CA/DC configurations: Enabling 3 MHz in aggregation scenarios such as n3+n28 and n28+n77 enhances spectral flexibility, particularly for rural and deep-indoor coverage.

- Harmonization across low bands: Aligning 3 MHz-capable bands (n28,n26, n85, n106) simplifies RF design, and supports a broader, cost-efficient device ecosystem.

The proposal requires no new RF configurations, introduces no testing burden, and aligns with an already validated approach used for Band n106.

This change will ensure all NR UEs supporting Band n28 ,n26 and n84 in Rel-19 are capable of operating at 3 MHz, aligning support expectations across the ecosystem.

# Conclusion

Proposal 1:   Change the 3 MHz CBW for Band n28, n26 and n84 from optional to mandatory in Release 19. This CBW to be excluded from MSD interference impact analysis.

# Reference

[1] R4-2505214 – “WF on handling optional channel bandwidth across releases”
[2] R4-2506184 – “Input on Optional vs. Mandatory Channel BWs”