# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507489

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			TP to TR38.774: BS RF requirement overview for LP-WUS

Agenda item:		7.28.4

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact for LP-WUS.

# Discussion

In WF[1], for power boosting, the WF is quoted as below:

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS

- WF

  - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth.

  - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB.

    - X can be different for different channel bandwidth.

    - FFS on the value of X and use 3 as starting point.

We think to make the power boosting useful, the X = 3 dB seems reasonable.

As it is declaration based, the power boosting level and CBW associated with power boosting should be also declarable. For example, the declaration text in 38.141-1 could be specified as below:

| D.x | LP-WUS/LP-SS power dynamic range | If the BS supports LP-WUS/LP-SS operation, manufacturer can declare the minimum [+3] dB of LP-WUS/LP-SS power boosting level it could support together with the channel bandwidth. The LP-WUS/LP-SS power boosting is defined as follows: The LP-WUS/LP-SS power boosting is the difference between the average power of LP-WUS/LP-SS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS/LP-SS and the NR carrier containing the LP-WUS/LP-SS REs). |
| --- | --- | --- |

The above declaration text based on WF to be agreed in conformance test.

Issue 1-3: Whether to define EVM requirements for LP-WUS

- Options

  - Option 1: No need to define EVM requirements for LP-WUS. (Huawei)

    - Because simulation results show that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirements do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. (Nokia)

    - Because frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR. (ZTE)

  - Option 2: EVM requirements for LP-WUS is needed. (CMCC)

    - Option 2-1 (CATT):

      - RAN4 to specify EVM requirement only for “ON” chips of OOK symbols.

      - RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence.

      - RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%.

      - RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence.

    - Option 2-2 (Ericsson):

      - Define the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS as 8%.

      - Wait further RAN1 progress on PAPR discussion.

- WF

  - Further check in next meeting whether EVM for LP-WUS needs to be specified.

According the current WID objective, the below objective still in line with the spirit of the study phase and keep the RF impact as min as possible.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

The 64QAM modulation is mandatory tested in 38.141-1 and therefore, the LP-WUS signal quality test can be dependent on the PDSCH 64QAM and there is no need to introduce additional test. In the core specification, the LP-WUS signal quality requirement should be specified as this is new modulation scheme in Rel-19. The suggested wording in clause 6.5.2.2 as below:

The OOK4 signal quality test can be dependent on the 64QAM EVM test.

#### 6.5.2.2	Minimum Requirement for BS type 1-C and BS type 1-H

For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2.

Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for LP-WUS | Required EVM |
| --- | --- |
| OOK-4 | 8 % |

Some company last meeting claims the PAPR may increase for combined WUS and NR signal and want to introduce the test for this case. As the balanced LP-SS is agreed in RAN1, we donot see the PAPR issue anymore.

Agreement

For LP-SS with M=4, supports set1 (balanced).

No PAPR issue when testing the combined NR and WUS signal.

TP starts here:

### 7.2.4	Transmitted signal quality

It is agreed that for overlaid OFDM sequences is designed for LP-WUS and the time sequence for overlaid OFDM symbol is ZC sequence. This applies both for OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal and this is illustrated in Figure 7.2.4-1. After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7.2.4.2.

Figure 7.2.4-1: OOK-4 signal processing and subcarrier mapping before IFFT

Figure 7.2.4-2: ZC sequence in frequency domain after DFT processing before IFFT

The simulation results on the WUR performance degradation due to the EVM impairment from different companies are listed in Table 7.2.4-1.

Table 7.2.4-1: The WUR performance degradation with different EVM of LP-WUS

| Companies |  | LP-WUS EVM |  | OOK WUR degradation (dB) |  | OFDM WUR degradation (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Ericsson | 12,5% (16QAM) | 0.6 | 0.6 |
|  | 8% (64QAM) | 0,2 | 0.2 |

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Proposal-1: The above declaration text based on WF to be agreed in conformance test.

Proposal-2:The OOK4 signal quality test can be dependent on the 64QAM EVM test.

Proposal-3:No PAPR issue when testing the combined NR and WUS signal.

# References

- R4-2504744, WF on NR LP-WUS BS RF, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing

- 38.104

- R4-2502092, TP to TR38.774: BS RF requirement overview for LP-WUS, Ericsson