# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507497

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			A-IoT device requirement overview

Agenda item:		7.26.3.2

Document for:		Discussion

# Introduction

In this paper, we present our view on the A-IoT device RF requirements.

# RF requirement for A-IoT UE

## Tx requirement

In WF [3], the TX requirement relating to backscatter transmission is quoted below:

Issue 3-1-1: Transmit output power

Agreement:

- Performance metric of transmit output power:

  - Backscattering loss

- Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power

  - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

- For the Input CW Tx power to define requirements:

  - FFS on the whether to use one value or a range of CW Tx power

  - FFS on the value(s) of CW Tx power

- Consider following aspects to determine backscattering loss to define requirements:

  - modulation loss

  - antenna efficiency

  - backscattering circuitry loss

  - impairment margin

  - minimum distance to the reader

  - differential radar cross section in two different modulation states

  - differential reflection coefficient in two different modulation states

  - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself

  - Other factors not excluded

- .

The backscatter loss is to be decided from last meeting WF. Though it can be derived from design considering either the differential radar cross section or differential reflection coefficient in two different modulate states. It can also derive the backscattering loss from the system level considering either the min ,max or other distance between device to BS. It can be understood that a max backscattering loss should be required at the min distance between device to BS if one expects the device shall work at a distance >= this min distance. Similarly a max backscattering loss should be required at the maximum distance between device to BS if one expects the device shall work at a distance <= max distance.  It is not clear whether it can allow the device 1 fulfil the reader range of min distance or max distance or a range in between. For example, in a case where the device attached in a box passing through the gate where reader and CWT are installed, a min distance is good enough to be specified for device reading range. In another case where the device is paged at a maximum distance, the backscatter loss should be specified according to this maximum distance. In this paper, different aspects are considered to derive the backscatter loss.

The required maximum backscattering loss may be decided by either min or max distance depending on use case.

For min distance from BS to device, the issue 3-5-1 gives a 1m as guidance. For maximum distance, 15m can be used as this is maximum range distance for device in TR 38.769.

Issue 3-5-1: Maximum input power

Agreement:

- Define maximum input level, consider following options:

- Option 1: using [1m] distance assumption from reader to device

- Option 2: using square law appliance range

- Option 3: using dynamic range of device receiver

Min distance between device to BS is 1 m and max distance is 15m.

No matter which distance is used, the methodology is the same to calculate the allowed maximum backscatter loss. The coexisting assumption from study item on the CWT node and A-IoT BS location is used in this case. As illustrated in Figure 1, the device can be any location between dmin and dmax and CW node coverage is provided by either CWT#1 and CWT#2. It should be noted that with the maximum distance of 15m, the cell radius will be expanded in green or blue area.

Figure 1: The CWT node location and BS location in coexisting study

Figure 2: The geometry relation for device coverage

To simply the calculation, a 2-D geometry is used as illustrated in Figure 2, where

- d1 is the distance between CWT and BS, d1= 20m in TR 38.769

- d2 is the min or max distance between device to BS, d2 = 1m for min distance and d2 = 15 m for max distance

- PR_CW is the received conducted CW power used in BS REFSENS test.

- Pbackscatter is the received conducted backscatter signal from device

- PD_R_CW is the device received CW power at device antenna

- PT_CW is the conducted transmitted CW power at CWT

- The CWT node and BS installed height is 8 m, referring to coexisting study in TR 38.769

Firstly, the CWT node transmitted power can be decided once the maximum allowed CW received power for BS REFSENS test is agreed. As this number to be discussed, for illustrated purpose, we set the CWT node power to 23 dBm instead which is used in coexisting study.

PT_CW = 23 dBm.

When d2 = 1m, dCWT_Device= 20.6m and dDevice_BS= 8.1 m.

Using the free space path loss (Tx/Rx antenna gain ignored) :

FSPL= 20log10 (d) + 20log10 (f) + 20log10 (d) +20log10 (4pi/C)

The received CW power at device antenna:

PD_R_CW = PT_CW – PL CWT_Device = 23 – 57.8 = -34.8 dBm				(1)

If the backscattered signal power received at BS equals the BS REFSENS, this should be min backscatter signal which can be detected by BS:

Pbackscatter = REFSENS

As the SNR simulation is ongoing on D2R and there is no agreement on D2R channel bandwidth. Here we assume the f=1/Tb, which is the signal bandwidth. Taking Tb= 266.67 us, this means the signal bandwidth is 3.7 kHz. Also assume SNR= 10 dB, the REFSENS is – 174 + 10log10(3700) + 5 + 10 = -123 dBm.

Therefore,

Pbackscatter = -123 dBm

The backscattered signal power at device antenna is

Pbackscatter  + PL Device_BS = - 123 + 49.5 = - 73.5 dBm				(2)

Using the received CW power at device antenna in equation (1) and generated backscatter signal power at device antenna in equation  (2), the backscatter loss can be calculated:

LBackscatter_loss_1 = PD_R_CW – (Pbackscatter  + PL Device_BS) =- 34.8 + 73.5 = 38.7 dB

Similarly, for the maximum distance of 15m, the backscatter_loss can be calculated (When d2 = 15m, dCWT_Device= 8.5m and dDevice_BS= 17 m.

LBackscatter_loss_2 = PD_R_CW – (Pbackscatter  + PL Device_BS)= - 27.1 – (- 123 +56.1)=39.8 dB.

In addition, the weakest backscatter signal will occur in the middle of the range. This is illustrated in Figure 3. Figure 1

Figure 3: Link budget for ISD of 20m between CWT and BS

Due to this, the d = 10 m (ISD/2) also to be checked.

Similarly, for the maximum distance of 15m, the backscatter_loss can be calculated (When d2 = 10m, dCWT_Device= 12m and dDevice_BS= 12 m.)

LBackscatter_loss_2 = PD_R_CW – (Pbackscatter  + PL Device_BS)= - 30 – (- 123 +53)=40 dB.

From the above discussion, it can be observed that backscatter loss can be derived with a certain deployment scenario where the CWT node position is fixed. The CWT power can be set with the maximum tolerated CW received power at BS. The min backscatter signal received at BS is BS REFSENS and the backscatter loss can be derived once the max distance of device to the BS is decided. Be aware that the maximum reader distance may not be “worst” case as the CWT node is outside topology so a higher CW received power at device antenna may compensate the path loss to BS. In above calculation, the backscatter losses calculated at min distance (1m) and max distance (15m) are very close because of this.

The backscatter loss can be derived when BS REFSENS is agreed.

The backscatter loss could similar at both min and max distance due to that the higher received CW power compensate the path loss to BS .

- As the BS REFSENS still under discussion and above calculation is using 2-D geometry so it can be further calibrated once the numbers used in above calculation is agreed.  What can be discussed further is that how to treat the backscatter loss calculated. As this is backscatter loss that device must meet otherwise no backscatter signal can be detected in the certain deployment scenario. It can be viewed 10 percentile value when CDF curve of the backscatter loss is tested.

Use the max backscatter loss in the 10 percentile point requirement for OTA test.

In another companion paper for CW node RF analysis, the phase noise impact is identified. For purposes of discussion, the noise power (R=1) measured in the BS receiver using a phase noise profile from TR 38.803 is illustrated in Figure 4. To illustrate the relation between noise power due to phase noise impact and the backscatter loss, the SNR at BS receiver is illustrated in Figure 5. It can be observed that backscatter loss play big role on the reader range. Especially for R=1 case, where the range is between 1 m to 2 m if the backscatter loss would be 30 dB. The phase noise from the CW node becomes a limiting factor on the design of the backscatter loss. We suggest using the 15 dB and it seems in this case, with the help of the other Tc (so R> 1), the reader range of 15 m could be achieved.

Phase noise is a system limiting factor on design of the backscatter loss

Backscatter loss should be better than 15 dB to achieve 15 m with R other than 1.

Figure 4: noise power within different signal BW (1/Tb) with R=1

Figure 5: Received SNR with different backscatter loss and frequency shift

Referring the backscatter loss simulation in [2], this maximum backscatter loss should not be an issue from design considering the dipole antenna type. The backscatter loss relates to the antenna gain and differential reflection coefficient or differential radar cross section in two different modulation states when device modulate the backscattering signal. In OTA test discussion, non RF reflective material is assumed so the backscatter loss will not be correlated to the incidence angel of the CW signal..

Issue 4-2-4: tag mounting material

Agreement:

- The device needs to mount on the non-RF-reflective and non-RF-conductive material

Another issue is the test condition of the sidelobe where the wanted signal should be detected. As the OOK signal waveform will not be detected in time domain due to the presence of the CW signal, a filter will be needed on the designed frequency offset to CW frequency where the data is modulated. RAN1 still discuss the down-selection of the R and Tb values.

Add the test condition to identify the frequency of the test signal , i.e frequency offset of filter center to the frequency of the CW signal.

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
|  | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Based on analysis above, the backscatter loss for OTA requirement could be defined as similar as min EIRP of spherical coverage in NR FR2, but with a different metric, for example:

The minimum backscatter loss at the Xth percentile of the distribution derived from radiated power measured in the sidelobe of backscattering signal over the [full sphere] around the UE is specified in Table Y below. The side lobe power is measured with a filter and the filter offset between center of filter frequency and frequency of CW signal is the data rate multiplying with the R factor configured for backscatter signal.  The requirement is verified with the test metric of backscatter loss (Link=Spherical coverage grid, Meas=Link angle) and derived with equation below

Backscattering loss= Input CW Tx power at device antenna-Backscattering power at device antenna					(3)

Table Y: spherical coverage for backscatter loss

| Operating band | Min minimum backscatter loss at [10] %-tile CDF (dB) |
| --- | --- |
| N8 | FFS |

The side condition are listed in table below

Table Y: test condition for backscatter loss

| Test parameter | value |
| --- | --- |
| Data rate (bps) | 1/Tb |
| R factor | X |
| CW frequency (MHz) | Y |

Specify the spherical coverage requirement for max backscatter loss associated with Xth percentile.

Issue 3-2-1: SFO requirement

Agreement:

- 10% SFO is assumed as the minimum performance for devices.

- FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

Issue 3-2-2: Modulation quality requirements

Agreement:

- Define modulation quality requirements for both BPSK and OOK.

- Consider to define same modulation quality metric for both BPSK and OOK if possible.

- Reference measurement process needs to be discussed

For modulation signal quality requirement, the SNR degradation of a signal can be contributed by intrinsic noise from the hardware (e.g. the intermodulation product from non-linear PA) and EVM can be measured for legacy system. For the backscatter signal modulation, modulation is done by switching across different load impedance and therefore, a different metric may be relevant to modulate signal quality. In modulation efficiency requirement discussion, the designated sideband power is measured and backscatter loss is derived using equation (3). It can be discussed that also the backscattered signal should be decodable so any imperfection hidden in design could be detected. In this sense, the BLER could be used as one metric to test the signal quality and this could be tested in the direction where minimum backscatter loss is measured (beam locked mode). Such test should be complementary to the backscatter loss test as it only relates to power but not monitoring the signal quality.

It should be noted that with the phase noise impact from CW node and also the leakage power from CW, the noise floor raise may be substantial. In this case, the noise raise by test environment may mask the device internal noise which should be tested in the modulation quality requirements. If this would be the case, the modulation quality cannot be tested. Therefore, we suggest that discussing the test feasibility before specifying this requirement.

BLER can be tested for the backscattered modulation quality as a directional requirement.

Test feasibility should be discussed before specifying the modulation signal quality requirements.

Issue 3-2-3: Occupied bandwidth

Agreement:

- Further discuss whether to define the occupied bandwidth for device 1.

In our view, the SFO (sampling frequency offset) which is quite large (e.g 10%), this may shift the center frequency of the first sideband but there is no frequency error requirement in device 1 and occupied bandwidth can capture this as a “distortion” on the modulated signal.  This test can be directional and only test the maximum modulation efficiency direction.

Specify the occupied bandwidth test to test the SFO imperfection.

Issue 3-2-4: Emission requirements

Agreement:

- Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point.

- Further discuss whether to define TRP or EIRP requirement for unwanted emission.

In 38.101-2, the SEM and spurious is measured in TRP. The spurious requirement is regulatory requirement so the measurement should be aligned with 38.101-2 for OTA test.

The TRP metric should be used for SEM and Spurious requirement.

## Rx requirements

For device 1 which harvests the RF energy, it may be useful to discuss whether to have EH sensitivity as one requirement. As the energy harvesting sensitivity may be different with the REFSENS, for D1T1-B, the device can be illuminated with CW or other RF signal but still have a different REFSENS specified and as such, the OTA test can be set up with the knowledge both EH sensitivity and REFSENS.

The EH sensitivity and REFSENS both should be known for the RF test.

According to the coexisting assumption in SI phase, the CW node is co-located with reader, implying the EH sensitivity also play role in the reader coverage and therefore, RAN4 needs to discuss whether an EH sensitivity should be specified. Our opinion is that if coexisting simulation is not changed, EH sensitivity should be specified otherwise the system may not work as expected. Another alternative for EH is the manufacture declare the EH but then needs further discussion in test feasibility to see if at least some range is needed to limit the test condition impact.

Energy harvesting sensitivity relevant to the reader range for D1T1-B deployment

Specify the REFSENS and declaration based on EH sensitivity if there is no impact on test feasibility design.

For the REFSENS requirement, the options are listed below from WF [1]:

Issue 3-4-1: Reference sensitivity

Agreement:

- Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

- At least to define maximum gain direction sensitivity requirements

- FFS on how to different sensitivity requirements on different orientations

For option 2, for a ISD of 20 meter case between CW node and BS, REFSENS is good enough with -26 dBm as illustrated in Figure 4. Any number higher than this is not necessary. It can even be relaxed to -20 dBm if BS transmits 38 dBm power.

Depending the maximum BS transmission power, REFSENS can be either -26 dBm or -20 dBm.

However, such REFSENS requirement is quite relaxed. Manufacture should declare a better REFSENS than this and network can use lower transmit power to save energy. The REFSENS is conducted requirement and EIS is the OTA requirement. If the dipole antenna type is used in the device, the total isotropic sensitivity (TIS) or the averaging the EIS in one direction ( elevation and azimuth angle) may be worth to consider as the antenna gain difference for a donut-shaped antenna pattern is maximum 2.15 dBi.

Manufacture should declare a better REFSENS than min REFSENS derived with link budget.

Averaging EIS can be specified instead of the maximum EIS if a dipole antenna is used in device.

If the device would design different type antenna other than dipole, maybe RAN4 can continue discuss if a simpler requirement would be fine.

Figure 4: The received power at device with different BS output power and ISD between CW node and BS

Issue 3-5-1: Maximum input power

Agreement:

- Define maximum input level, consider following options:

- Option 1: using [1m] distance assumption from reader to device

- Option 2: using square law appliance range

- Option 3: using dynamic range of device receiver

For the maximum input power, legacy requirement relates to the saturation of the receiver RF chain. For device 1, the diode will be used and it is a question if the square-law relation between input power and output voltage can be kept within a pre-defined range. As the square law is assumed in the ED modeling in LLS, if the component deviates from this behavior beyond certain input power, it should be specified as a requirement. Figure 5 gives one example of one diode from one manufacture and shows the square law only applies within a certain range of input power. RAN4 can start to collect this parameter before specifying the maximum input power.

The maximum input power can be also specified from the min distance calculation. With the maximum BS transmission power of 38 dBm and 1 meter minimum distance between BS and device, the maximum input power device should tolerate is 38 – 31.5 =6.5 dBm.

6.5 dBm should be specified as maximum input power.

Figure 5: detector output characteristic

# Conclusions

In this contribution, we present our overview on the A-IoT device RF with below proposal and observations:

Observation 1 The required maximum backscattering loss may be decided by either min or max distance depending on use case.

Observation 2 Min distance between device to BS is 1 m and max distance is 15m.

Observation 3 The backscatter loss can be derived when BS REFSENS is agreed.

Observation 4 The backscatter loss could similar at both min and max distance due to that the higher received CW power compensate the path loss to BS .

Proposal-1: Use the max backscatter loss in the 10 percentile point requirement for OTA test.

Proposal-2: Backscatter loss should be better than 15 dB to achieve 15 m with R other than 1.

Proposal-3: Add the test condition to identify the frequency of the test signal , i.e frequency offset of filter center to the frequency of the CW signal.

Proposal-4: Specify the spherical coverage requirement for max backscatter loss associated with Xth percentile.

Proposal-5: BLER can be tested for the backscattered modulation quality as a directional requirement.

Proposal-6: Test feasibility should be discussed before specifying the modulation signal quality requirements.

Proposal-7: Specify the occupied bandwidth test to test the SFO imperfection.

Proposal-8: The TRP metric should be used for SEM and Spurious requirement.

Observation 6 The EH sensitivity and REFSENS both should be known for the RF test.

Observation 7 Energy harvesting sensitivity relevant to the reader range for D1T1-B deployment

Proposal-9: Specify the REFSENS and declaration based on EH sensitivity if there is no impact on test feasibility design.

Proposal-10: Depending the maximum BS transmission power, REFSENS can be either -26 dBm or -20 dBm.

Proposal-11: Manufacture should declare a better REFSENS than min REFSENS derived with link budget.

Proposal-12: Averaging EIS can be specified instead of the maximum EIS if a dipole antenna is used in device.

Proposal-13: 6.5 dBm should be specified as maximum input power.

# References

- R4-2505230, WF on A-IOT device requirements, Huawei

- R4-2504399, A-IoT device requirement overview, Ericsson