3GPP TSG-RAN WG4 Meeting #115															R4-2507527

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.7.3

Source:	Samsung

Title:	Views on UE RF aspects for LB CA via switching

Document for:	Discussion

1	Introduction

RAN4 has discussed a Rel-19 Work Item (WI) on low band (LB) carrier aggregation (CA) via switching, which was initiated to address a market demand for LB utilizations [1]. During the past meetings, RAN4 approved WFs [2, 3] and an initial agreement on switching period values was already sent to RAN1 [4] to help their discussion on the semi static switching pattern for LB CA switching, considering the short period.

Building on the previous discussion, in this contribution, we would like to continue providing our views focusing on the UE RF aspects taking into account the WID assumption and discussion status across WGs.

2	Discussion

2.1	Work scope

First of all, RAN4 discussed and reached some agreements on the Rel-19 scope as captured in the WF to focus on the scenario with the switching between two low bands in Rel-19 WI as below [2]:

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

With respect to the two bands mentioned in the WF above, they should be considered as CA_n5A-n29A, which serves as a single example band combination for this WI, as noted in [1]. Then, the switching scenario between LB-LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, which are also related to following deployment constraints captured in [1].

- The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

Figure 1 shows the switching scenario of Rel-19 with its assumptions and constraints, which should be taken into account for the WI discussion in RAN4.

.

Figure 1: Switching scenario for Rel-19 (Case 1 and Case 2)

Observation 1:	Switching scenario between LB-LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, e.g., CA_n5A-n29A, which serves as a single example band combination for this WI.

For higher-order combination than two low bands, although some companies propose to consider additional UE aspects including higher number of Rx chain, or LB CA switching capability in Rel-19, it can be discussed in the future releases but not in Rel-19 since it is obviously out of scope for this WI based on the last sub-bullet on the WI scope in [2] and WID assumption [1]. RAN1 also made an agreement that Rel-19 supports only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between two low NR bands with FDD (PCell) and SDL (SCell), but both would not be SCell with additional band (PCell) as follows [5]:

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers

Observation 2:	RAN1 also made an agreement that Rel-19 supports only semi-static configuration of switching pattern to a UE between two low NR bands with FDD (PCell) and SDL (SCell), but both would not be SCell with additional band (PCell).

Therefore, our previous agreement should be kept, which is that the combination of those two low bands with other band should not be discussed in Rel-19 but could be further discussed in the future release up to RAN plenary discussion. This is also considered by the feature completion right on schedule. The WI does not need to discuss or restrict any other aspects beyond the combination of two low bands in Rel-19.

Observation 3:	Higher-order combinations than two low bands are out of the scope of the WID of Rel-19 LB CA via switching .

2.2	Switching period

For switching period values, as sent an LS to RAN1 [3], two candidate values were agreed based on potential UE architectures for the LB-LB switching and previous uplink Tx switching discussions. However, given that the switching delay period value in RAN4 should be based on potential UE architectures for low bands, and also on the UE capability which is agreed to have per band pair per band combination among candidate values, it would be better to have additional periods between 35us and 140us. Considering the symbol length-based switching period, 70us is a good candidate for 15kHz SCS together with 35us and 140us.

Observation 4:	Switching period values should consider UE architecture for the switching between two bands.

Observation 5:	UE capability is agreed to have per band pair per band combination among candidate values.

Proposal 1:		Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us.

2.3	Time mask

Regarding the time mask for switching, not a few switching mask proposals have been submitted to capture the switching period and location for both Tx and Rx switching between LB FDD and LB SDL bands. During the last meeting, RAN4 has agreed and considered several options as follows [3]:

Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded

As described above, for Tx time mask, it was agreed to define the on-off time mask requirements between FDD UL and SDL. In our view, the Tx on-off time mask for the LB-LB switching could be based on the legacy on-off time mask as much as possible which is defined in clause 6.3.3.2 of TS 38.101-1. Similarly, it also needs to discuss Rx switching requirements to reflect where the Rx switching period occurs between FDD DL and SDL. However, as discussed in the last meeting, the switching location is necessary to consider many aspects, e.g., PDCCH reception and FDD (PCell) impact, which is primarily concerned with the physical-layer channels or switching period pattern at least. RAN1 has agreed that the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier, for switching from SDL carrier to FDD carrier as proposed in [5]. For the other switching from FDD carrier to SDL carrier, RAN1 also has the similar options until their last meeting as below [6]. Therefore, RAN4 time mask including switching location for both Tx and Rx can be determined later after RAN1 makes a clear decision on the latter case, switching from FDD carrier to SDL carrier.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier

Observation 6:	Tx on-off time mask for the LB-LB switching should be based on the legacy on-off time mask as much as possible which is defined in clause 6.3.3.2 of TS 38.101-1.

Observation 7:	Rx switching requirements are also required to reflect where the Rx switching period occurs between FDD DL and SDL.

Observation 8:	RAN1 has agreed that the switching gap location for switching from SDL carrier to FDD carrier, but still open for the other switching from FDD carrier to SDL carrier.

Proposal 2:		RAN4 time mask including switching location for both Tx and Rx the switching location can be determined later for both Tx and Rx after RAN1 makes a clear decision on the latter case, switching from FDD carrier to SDL carrier.

2.4	Testability

On top of switching period and location from UE Rx side, the testability of Rx switching could be another issue if it could be captured in TS 38.101-1 clause 7. Since the switching to/from SDL band does not have any Tx, it would be not the case to measure the switching operation unlike other RF requirements on top of Tx on-off time mask requirements.

One candidate method for the evaluation would be to decouple Tx and Rx requirements. For example, Tx requirements can be defined as time mask for evaluation as the legacy Tx switching. Meanwhile, RRM requirements can be used to verify the interruptions for both DL and UL, which can be tested such as L1/L3/SCell activations if they are defined by RRM specification as discussed in [7] below.

Issue 1-3: SDL SCell related requirement Issue 1-3-1: Activated SDL SCell L3 measurement and corresponding interruption Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-3: SDL SCell activation/deactivation Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-4: BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-5: others for SDL SCell Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

Observation 9:	Rx switching time mask requirement can be defined in UE RF specification clause 7.

Observation 10:	RRM requirements can be used to verify the interruptions for both DL and UL, which can be tested such as L1/L3/SCell activations.

Proposal 3:		It is recommended for RAN4 to evaluate Rx switching by decoupling Tx and Rx requirements.

3	Conclusion

In this contribution, we provide our views focusing on the UE RF aspects taking into account the WID assumption and discussion status across WGs. Following observations and proposals were derived.

Observation 1:	Switching scenario between LB-LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, e.g., CA_n5A-n29A, which serves as a single example band combination for this WI.

Observation 2:	RAN1 also made an agreement that Rel-19 supports only semi-static configuration of switching pattern to a UE between two low NR bands with FDD (PCell) and SDL (SCell), but both would not be SCell with additional band (PCell).

Observation 3:	Higher-order combinations than two low bands are out of the scope of the WID of Rel-19 LB CA via switching .

Observation 4:	Switching period values should consider UE architecture for the switching between two bands.

Observation 5:	UE capability is agreed to have per band pair per band combination among candidate values.

Proposal 1:		Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us.

Observation 6:	Tx on-off time mask for the LB-LB switching should be based on the legacy on-off time mask as much as possible which is defined in clause 6.3.3.2 of TS 38.101-1.

Observation 7:	Rx switching requirements are also required to reflect where the Rx switching period occurs between FDD DL and SDL.

Observation 8:	RAN1 has agreed that the switching gap location for switching from SDL carrier to FDD carrier, but still open for the other switching from FDD carrier to SDL carrier.

Proposal 2:		RAN4 time mask including switching location for both Tx and Rx the switching location can be determined later for both Tx and Rx after RAN1 makes a clear decision on the latter case, switching from FDD carrier to SDL carrier.

Observation 9:	Rx switching time mask requirement can be defined in UE RF specification clause 7.

Observation 10:	RRM requirements can be used to verify the interruptions for both DL and UL, which can be tested such as L1/L3/SCell activations.

Proposal 3:		It is recommended for RAN4 to evaluate Rx switching by decoupling Tx and Rx requirements.

Reference

[1] 	RP-243317,	New WID on low band carrier aggregation via switching, RAN#106

[2]	R4-2503016,	WF on low-low band switching, RAN4#114

[3]	R4-2505223,	WF on UE RF requirements for LB CA via switching, RAN4#114-bis

[4]	R4-2502877,	LS on switching periods for low-low band switching, RAN4#114

[5]	R4-2504388,	UE RF requirements for LB CA via switching, RAN4#114-bis

[6]	Chair’s notes of RAN1#120b meeting

[7]	R4-2504895,	WF on RRM requirements for NR_LBCA_Sw, RAN4#114-bis