3GPP TSG-RAN WG4 Meeting # 115															R4-2507529

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.1.2.3.4

Source:	Samsung

Title:	Views on SRS IL imbalance issue

Document for:	Discussion

1	Introduction

As one of the objectives, ”6Rx for handheld and FWA UE” under Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1], the issue of insertion loss imbalance across SRS ports has been discussed to study whether the corresponding solution is required or not. However, despite RAN4 spending considerable time trying to find a solution for the issue, the same question and outcome have been repeated over the last year. In this contribution, we would like to continue sharing our views based on the latest discussion.

2 Discussion

As noted in the latest WF [2], like previous ones, RAN4 tried to have a single solution based on UE self-compensation. It was agreed to perform UE self-compensation of SRS IL up to its configured maximum output power for SRS transmissions without RAN1/4 specification impacts.

Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications)

Observation 1:	RAN4 agreed to perform UE self-compensation of SRS IL up to its configured maximum output power for SRS transmissions without RAN1/4 specification impacts.

On top of the agreement on the principal solution above, RAN4 also have two additional solutions, which can be discussed by this meeting.

Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

First of all, in our view, Issue 4-2-2 should be for the topic on whether/how to improve UE self-compensation method such as UE capability introduction to indicate the self-compensation for the network as Solution 1. However, it seems Solution 2 to extend Type 3 SRS PHR reporting aims at a different target to change Type 3 PHR triggering condition, which is nothing to do with the UE self-compensation, but has RAN1 specification impact, which is totally up to RAN1 discussion. It is also not aligned with the main agreement that no RAN1/4 specification impact is expected in the previous issue. In that sense, Solution 2 should be excluded from RAN4 discussion.

Observation 2:	RAN4 needs to discuss the topic on whether/how to improve UE self-compensation method such as UE capability introduction to indicate the self-compensation for the network as Solution 1.

Observation 3:	Solution 2 to change Type 3 PHR triggering condition is nothing to do with the UE self-compensation, but has RAN1 specification impact, which is totally up to RAN1 discussion.

Proposal 1:		Solution 2 should be excluded from RAN4 discussion to follow the agreement in Observation 1.

Even for Solution 1, it still remains unclear how the network can get the benefit from the UE indication even if the UE can do it for the network since they anyway need to perform SRS IL self-compensation by themself without any help from the network. Then, it would give the same question about the purpose of providing assistance to the network on SRS IL compensation, e.g., UE capability indication, if network cannot make any change of the SRS imbalance issue. Therefore, in our view, the additional UE capability indication for the network with UE self-compensation would not help improving the downlink channel estimation from the legacy SRS IL imbalance issue.

Observation 4:	Solution 1 still remains unclear how the network can get the benefit from the UE indication.

Observation 5: 	Additional UE capability indication for the network with UE self-compensation would not help improving the downlink channel estimation from the legacy SRS IL imbalance issue.

Nevertheless, in order to move forward our discussion in a productive way, we could see that an optional UE capability with single bit indication can be introduced for the compromise if it still helps the network performance practically. The new capability should indicate whether UE can perform SRS IL self-compensation for the network as a static solution. By doing so, the SRS IL imbalance solution does not have any impacts to RAN1/4 specifications, nor need further discussion for this WI. Additional solution to resolve fundamental issues of SRS beyond UE self-compensation, i.e., PCMAX or PHR for SRS, should be handled in the future radio discussion.

Observation 6:		An optional UE capability with single bit indication can be introduced for the compromise if it still helps the network performance practically.

Proposal 2:		Additional solution to resolve fundamental issues of SRS, i.e., PCMAX or PHR for SRS, should be handled in the future radio discussion.

3	Conclusion

In this contribution, we continued to provide our analysis and view on the proposed solutions. Following observations and proposals are provided to be considered by this meeting.

Observation 1:	RAN4 agreed to perform UE self-compensation of SRS IL up to its configured maximum output power for SRS transmissions without RAN1/4 specification impacts.

Observation 2:	RAN4 needs to discuss the topic on whether/how to improve UE self-compensation method such as UE capability introduction to indicate the self-compensation for the network as Solution 1.

Observation 3:	Solution 2 to change Type 3 PHR triggering condition is nothing to do with the UE self-compensation, but has RAN1 specification impact, which is totally up to RAN1 discussion.

Proposal 1:		Solution 2 should be excluded from RAN4 discussion to follow the agreement in Observation 1.

Observation 4:	Solution 1 still remains unclear how the network can get the benefit from the UE indication.

Observation 5: 	Additional UE capability indication for the network with UE self-compensation would not help improving the downlink channel estimation from the legacy SRS IL imbalance issue.

Observation 6:		An optional UE capability with single bit indication can be introduced for the compromise if it still helps the network performance practically.

Proposal 2:		Additional solution to resolve fundamental issues of SRS, i.e., PCMAX or PHR for SRS, should be handled in the future radio discussion.

Reference

[1] RP-240828	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,	RAN#103

[2]	R4-2505101	WF on 6Rx RF requirements and SRS IL imbalance,							RAN4#114-bis