3GPP TSG-RAN WG4 Meeting #115	R4-2507534

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	On General aspects in AIML Mobility

Agenda item:	7.23.2

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis in the scope of Rel-19 SI ‎[1]. The RAN4#115 meeting will discuss RAN4's general aspects and impact on RRM measurement prediction, measurement event prediction, and AI/ML mobility testability and interoperability. This contribution provides our views on RAN4 general aspects.

Discussion

Priorities of different scenarios

Regarding prioritization of different scenarios, the following prioritization was agreed upon during RAN2#126, as outlined in Table 1.

Table 1: Prioritization of evaluation scenarios (Table 5.2.1-1 in TR 38.744 draft)

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

From the above listed scenarios, Scenario 2, 3 and 4 have been prioritized as high priority in RAN2. Thus, RAN4 should focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3, and #4 (i.e., FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction and FR2 to FR2 intra-frequency temporal domain case A).

RAN4 to focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3 and #4 (i.e., FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction and FR2 to FR2 intra-frequency temporal domain case A).

Impacts of measurement errors

In the SI phase, the potential performance gain using AIML-based mobility has been evaluated with an assumption on no measurement error based on RAN2 metrics such as a mean square error. However, in the WI phase, RAN4 should evaluate the performance of the AIML mobility with having an assumption on the measurement error to understand the impact on the prediction accuracy.

Therefore, the RAN4#113 meeting has discussed the impact of measurement errors for AI/ML Mobility evaluation and identified a number of options on measurement errors.

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. Way Forward: Many companies are supporting Option 4. Companies are encouraged to bring proposals on what input RAN4 should provide to RAN2 to consider impact of error on measurements used for prediction on the accuracy of the predicted measurement/event

Also, in RAN4#114, it has been agreed that RF and baseband errors on the measurements would have an impact on the measurement prediction accuracy.

Issue 2-1-4: Impacts of measurement error on prediction accuracy Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

In mobility scenarios, it is important that measurement errors will occur when utilizing data as input or ground truth for AI/ML models. These errors can significantly affect network performance by resulting in inaccurate predictions, which in turn may impact mobility management, handover optimization, and RRM. Therefore, it is essential for RAN4 to have a common understanding on the cause and modeling of the measurement error. Thus, the following WF was recommended in RAN4#114bis meeting:

WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

As stated in the WF, it is imperative to assume the proper measurement errors in simulations. Therefore, during the Study Item period, RAN4 should collect all analysis on potential modeling methods and at least capture the collected analysis in the TR.

In Study Item, RAN4 to collect analysis on potential modeling methods and at least capture the collected analysis in the TR.

To define the measurement errors in the inputs of AI/ML models, there exist RF and baseband measurement errors. The models of RF and baseband measurement error can be the same or different.

Also, it is widely assumed to use a uniform or a Gaussian distribution as a model of a measurement error. The uniform or Gaussian distribution modeling can be readily applicable to RF and baseband error. Note that any other modeling can be further considered in the Work Item phase.

RAN4 to capture the following error modeling principles in the TR:

- RF and baseband measurement error modeling may be considered.

- A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

- Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied in RAN4 working group for this use case.

The following Observations and Proposals were made:

- RAN4 to focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3 and #4 (i.e., FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction and FR2 to FR2 intra-frequency temporal domain case A).

In Study Item, RAN4 to collect analysis on potential modeling methods and at least capture the collected analysis in the TR.

RAN4 to capture the following error modeling principles in the TR:

- RF and baseband measurement error modeling may be considered.

- A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

- Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

- References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19).