3GPP TSG-RAN WG4 Meeting #115	R4-2507535

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Testability and Interoperability Issues for AIML Mobility

Agenda item:	7.23.4

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis in the scope of Rel-19 SI ‎[1]. In RAN4#115 meeting, following topics are to be discussed.

- RAN4 impacts for RRM measurement prediction

- RAN4 impacts for measurement event prediction

- RAN4 General aspects

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution, we provide our views and discuss different aspects of testability and interoperability issues in RAN4 for AI/ML Mobility.

Discussion

This document addresses the Testability and interoperability issues related to AI/ML mobility SI in RAN4 based on the agreements and the recommended way forward of the issues from the RAN4#114bis meeting. Specifically, we cover following topics:

- Testing setup

- Multiple cells in testing setup, and

- Prediction consistency in time domain.

Testing setup

Following is the RAN4#114 agreement for AIML Beam management use case:

Agreement: - Use CDL-based channel model as starting point

The legacy test is based on AWGN channel model which is a deterministic channel. However, for AIML Mobility and Beam management use cases, the fading channel is preferred to emulate variations of the radio channel for evaluating the prediction accuracy of the models/ functionalities.

RAN4#114 agreement for Beam management indicates the CDL channel model to be considered as the starting point for FR2. Hence the CDL channel model should be considered as a baseline for AIML Mobility for both FR1 and FR2.

RAN4 should consider CDL channel model for FR1 and FR2 as a starting point based on the RAN4 Beam management agreement.

One of the considerations for RAN4 in establishing RRM requirements and defining test cases for measurement prediction is the effect of L3 filtering and the number of beams utilized within test environments. Currently, a majority of legacy test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero). Additionally, to streamline test configurations, only a limited number of beams are typically simulated in the test setup.

Figure 1: 3GPP TS 38.300 Measurement Model

As illustrated in Figure 1, when L3 filtering is absent, the L1 and L3 beam measurement levels at point A1 align. Furthermore, for L3 cell-level RSRP predictions at Point C, the predicted values may correspond to the L1 and L3 beam-level predictions when a single beam is employed for prediction purposes. Furthermore, in some of the sub-use cases (for instance Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)) cell level measurements are predicted based on beam level measurements. Therefore, even if RAN4 agrees to consider only cell level measurements, a multi beam test setup would still be required to test such a use case.

- Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

- Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

Test verification procedure: The purpose of the test is the verification of mobility accuracy KPIs (Absolute RSRP accuracy and Relative RSRP accuracy) within specified limits (preselected within the testing framework). It is important to mention that TE can use either both metrics or one of them for the evaluation.

Test parameters: For the temporal domain measurement prediction, a single cell is required in the test (however at least two cells may be required for relative RSRP calculation considering beams from different cells). The test parameters are to be selected accounting for (not limited to) the frequency, Bandwidth, transmission power and antenna configuration.

In addition, parameters related to AI/ML model should be specified, namely:

- Observation Window (OW)

- Prediction Window (PW)

- Delay (𝛿)

- Measurement reduction rate in temporal domain (MRRT)

Ground truth extraction: This testing procedure targets AI/ML mobility use case with a focus on FR2-to-FR2 intra-frequency temporal domain case A and FR1-to-FR1 temporal domain case B. Therefore, it is needed to configure the UE channel variations during a predefined time duration, accounting for a given UE speed value.

At each time iteration, the UE radio conditions with regards to the serving cell are set up. This allows to assess RSRP value at each iteration. The TE should configure the DUT to report Predicted and Measured (Ground Truth) RSRP.

To get ground truth (GT) for FR1 at the test equipment (TE), following agreement has been made in RAN4#114 meeting:

- Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Furthermore, following agreement was reached in RAN4#114bis meeting;

Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

In Alt 2, there is a high possibility that a UE may cheat, e.g., it reports predicted value as GT.

UE cheating can be identified based on reported profile, e.g., RSRP strength, transmitting power. Taking RSRP strength as an example, TE can draw the profile of RSRP strength against the distance through emulation. Here distance is one example, attenuation and/or transmit power can also be considered. Also, TE draws a second profile based on the GT reported by the UE (the reported values could be predicted values (ML output) or GT; however, TE sees them as GT as it does not know whether UE is cheating or not). Consequently, TE compares the two profiles. If there is significant difference between the two profiles, it is concluded that UE is cheating. There may be more ways to deterministically vary RSRP such as adding attenuation in case of conductive testing, or by reducing transmit power of the probes in case of OTA.

UE cheating should be identified through comparing various profiles, e.g., RSRP strength. TE shall draw RSRP strength against the distance and compare it with the profile reported by the UE. If there is significant difference between the two profiles, then it is concluded that UE is cheating.

Another approach to identify UE cheating is through comparison of reported RSRP values from different UEs. The underlying assumption in this case is that UEs are collocated. 	For example, in the case of two UEs, TE compares whether RSRP1=RSRP2+δ, where δ is a tolerance threshold as it is possible that two RSRP values have minor difference (due to measurement/prediction noise). Here RSRP1 and RSRP2 denote the values of first and second UE, respectively. UE is detected as cheating when one UE reports suspicious value, i.e., different than second UE. The same rule is applicable to >2 UEs.

In certain conditions, UE cheating should be identified through comparing the profiles, e.g., RSRP strength of one UE against the other UEs. UE is detected as cheating when one UE reports suspicious value, i.e., different than other UEs.

Impacts on Testability aspects for Time domain Case A

An example of Case A is depicted in Figure 2. Briefly, in this case, there is no measurement reduction, and the focus is instead to optimize mobility performance based on predictions.

Figure 2: Time domain case A RRM measurement prediction.

The delay parameter, δ, must be configured, which is based on the DUT (UE) capability to predict and measure concurrently or sequentially. However, when the UE does the measurement and prediction sequentially then there will be delay between the time measurement and prediction are reported. Hence, the gNB must account for those delays. In contrast, when the UE can do prediction and measurement concurrently then delay might be small to gather the values at the TE. However, in the latter case, there can be UE processing delays, which we leave to UE implementation details.

UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities.

TE must account for the delays in acquiring prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

Impacts on Testability aspects for Time domain Case B

In temporal domain case B, a measurement reduction is realized following the parameter MRRT (Measurement reduction rate in temporal domain). However, for a given selected MRRT value (e.g. 50%), different possible skipping patterns can be applied as shown in the example below.

Figure 3.  Example of intra-frequency temporal domain case B with MRRT of 50%

For the testing, the DUT needs to match a skipping pattern for a given configured MRRT value. However, so far, RAN2 does not have yet any agreement related to skipping pattern indication (if it is part of the ML model configuration or not).

- At each iteration, the DUT should measure only at the identified slots following the skipping pattern within the current OW.

- The collected measurement samples are then input to AIML model to output predicted samples in the PW, reported to the TE.

- Additionally, DUT can be configured to measure and report the ground truth measurements within PW.

- At the TE, error (E) is calculated based on the selected evaluation metric (EM).

  - Generally, when E≤τ then it is assumed that test has passed. However, there can be a requirement that error for first EM, E1 (for absolute RSRP accuracy), and error for second EM, E2 (fore relative RSRP accuracy), both must satisfy the condition,  τ, or one of them at least. Further, there can be separate threshold values, τ1, τ2 for E1 and E2, resp.

  - Test procedure can also be looped over multiple iterations following the selected skipping pattern, where ‘Max’ value can be predefined at the TE.

    - When both E1 and E2 are configured by TE, TE calculates E, and then conditions E1 and E2 can be evaluated independently. The test results with respect to E1 and E2 are stored in the TE. The same test can be repeated until the iteration reaches to Max.

    - In addition to the aforementioned testing procedure, if the ratio of tests passed to tests failed has reached to a certain threshold, the testing can be deemed ended. For example, the ratio can be 0.9 or 90% of the tests passed out of the total number of tests.

As RAN2 does not have agreement on the way the skipping pattern information is identified by the UE, two options may be considered:

- Skipping pattern to be part of the ML inference configuration which means that this information is available along with OW and PW parameters

- Skipping pattern is not part of the ML inference configuration: UE may select autonomously the skipping pattern for a given configured MRRT value. However, in this case, RAN4 should agree on test configuration to explicitly indicate to DUT on the skipping pattern to use and identify the measurement slots.

For testing time domain prediction case B, RAN4 to follow RAN2 agreements regarding skipping pattern indication either part of inference configuration or test configuration.

As the number of tests can increase considerably with the different possible configuration settings (MRRT with possible skipping patterns as well as OW and PW parameters), a possible optimization of the testing setup can be realized through parallel processing of the testing. In fact, using the same basis of measurements, DUT can apply different selections on the data to match different MRRT & skipping pattern configurations. Therefore, the different extracted data are used to test simultaneously different configurations (with their corresponding ML Models).

Figure 4.  Parallel processing of testing setup

For testing time domain prediction case B, RAN4 to consider parallel processing of testing procedure to evaluate different configurations (mainly MRRT and related skipping patterns).

Multiple cells in the testing setup

In RAN2#128 meeting, following were the agreements related to cluster approach:

Agreements/Observations to be captured in TR - The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases. - For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping patter affects the performance. - For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs

In RAN4#114bis meeting, following were the agreements for the multiple cells for the test setup:

Issue 4-1-3: Multiple cells in the testing setup Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

For inter-frequency inter-cell scenario (i.e., scenario 3), RAN2 agreements indicate that the measurements from more than one cell are required as input to the ML model/ functionality for the cluster approach. This indicates that at least two cells are required for input and at least one target cell is required for the output of the ML model/ functionality during the test.  Hence RAN4 should consider more than two cells for the inter frequency inter cell measurement prediction in the test setup.

RAN4 should consider more than two cells for inter-frequency inter-cell scenario (i.e., scenario 3) in the test setup based on the RAN2 agreements.

Based on RAN2 agreement, it is known that at least two cells should be considered as input to the ML model/ functionality for the cluster approach. The two input cells should be intra frequency cells that are not co-located. The target cell for which the prediction will be performed should be an inter-frequency cell and should be co-located with one of the input cells.

For the cluster approach, at least two input cells are required that should be intra-frequency, non co-located cells and the target cell should be inter-frequency cell and co-located with one of the input cells.

Prediction consistency in time domain

Issue 4-1-2: Prediction consistency in time domain Agreement: - To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories. - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation RAN4 to prioritize discussion on FR1

The variations in network configurations across different cells/sites and the dynamic radio conditions may be challenging for the ML model/ functionality to maintain the desired prediction accuracy. The ML model/ functionality could perform well with certain characteristics of the cell and may not be the same with other cells. Another aspect is that the moving UE trajectory will cause the time-varying received power and SINR which may also impact the prediction accuracy.

RAN4 should study how to emulate the randomness and variations of the radio channel to evaluate the prediction accuracy of the ML models/ functionalities. This requires consistency in few basic parameters between the training and testing. To achieve this, RAN4 should define the baseline NW (i.e. cell) configurations, the ML measurement/ event prediction configurations and the deployment scenarios (i.e. UMi and UMa) as a starting point for testing.

Furthermore, RAN4 should study how to model the different time-varying or non-static characteristics per cell/site considering the UE trajectory. One possible method is to consider the time varying RSRP values for different UE trajectories like UE moving from cell center to cell edge or vice versa, UE moving around cell center etc., for evaluation.

RAN4 should study and define the test configuration including AI/ML functionality configurations and the UE trajectories to be considered during training and testing for maintaining the prediction consistency in time domain scenarios.

Conclusion

In this paper, we have shared our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. The aspects covered in this paper include testing setup, multiple cells in the testing setup, and prediction consistency in time domain. Thus, the following Observations and Proposals were made.

- RAN4 should consider CDL channel model for FR1 and FR2 as a starting point based on the RAN4 Beam management agreement.

- Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

UE cheating should be identified through comparing various profiles, e.g., RSRP strength. TE shall draw RSRP strength against the distance and compare it with the profile reported by the UE. If there is significant difference between the two profiles, then it is concluded that UE is cheating.

In certain conditions, UE cheating should be identified through comparing the profiles, e.g., RSRP strength of one UE against the other UEs. UE is detected as cheating when one UE reports suspicious value, i.e., different than other UEs.

UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities.

TE must account for the delays in acquiring prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

For testing time domain prediction case B, RAN4 to follow RAN2 agreements regarding skipping pattern indication either part of inference configuration or test configuration.

For testing time domain prediction case B, RAN4 to consider parallel processing of testing procedure to evaluate different configurations (mainly MRRT and related skipping patterns).

RAN4 should consider more than two cells for inter-frequency inter-cell scenario (i.e., scenario 3) in the test setup based on the RAN2 agreements.

For the cluster approach, at least two input cells are required that should be intra-frequency, non co-located cells and the target cell should be inter-frequency cell and co-located with one of the input cells.

RAN4 should study and define the test configuration including AI/ML functionality configurations and the UE trajectories to be considered during training and testing for maintaining the prediction consistency in time domain scenarios.

- References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19).