3GPP TSG-RAN WG4 Meeting #115	R4-2507638

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.25.3.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#114bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the existing BS RF TX requirements for SBFD operation.

# Discussion

## Deployment scenarios for FR1 SBFD macro BS

- Way forward - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope. - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include: - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.: - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios - Option 2: Co-existence simulation and potentially analysis of co-located scenarios - Expected spectrum allocation and sub-band sizes - Expected feasible filtering performances considering the above two aspects

We don’t see the new SBFD-SBFD co-existence is in the scope of Rel-19. RAN4 is already started to provide draftCRs for the requirements of SBFD-capable BS and there really is no time to widen the scope any further. There have been several meetings without conclusion of RX in-band blocking requirements and if the scope is widened any further, we are risking the finalization of the WI. The core part of the WI should be finished in RAN4#116 meeting and there is a mountain of work and numerous agreements needed before that. The described scenario where two operators SBFD capable BS’s would coexist, is a new scenario and a rare one. It has not been discussed in the study item phase and should not be included here. There are numerous aspects that would require solving, which some are listed in the WF. One crucial thing not to be forgotten here is that SBFD-SBFD deployment scenario will need all the operators to deploy the SBFD network and agreement will be needed from all operators operating in the band. Full coordination is needed between operators for the deployment of networks. If two SBFD networks would coexist with additional emission requirements, this would mean that TDD networks would not be possible to deploy. For FR1 frequencies at the moment, only the current n104 band is suitable for this. This in effect would mean reserving n104 only to SBFD use. In our view the customer specific proprietary requirement can be used in such case where two SBFD systems are co-existing and reasonable co-location is needed. If there is a requirement from regulators for SBFD on adjacent operators, RAN4 can have an enhancement to SBFD on a dedicated WI to define additional emissions and blocking requirements for this scenario.

SBFD-SBFD co-ex case should not be in the scope of Rel-19.

## Joint measurement for normal DL symbols/slots and SBFD symbols/slots

The following was agreed in RAN4#114bis:

- Way forward - Further discuss on the following aspects in the performance WI - Whether the following two slots configured with flexible symbols could be used for EVM measurement. - Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or - Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols. - Separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. - Use the following text proposal as starting point. - --------------------------------------------------------Start of Text Proposal ------------------------------------------------------- # B.7 Averaged EVM EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured. For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step. - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements: - Thus is calculated using in the expressions above and is calculated using in the calculation. - Thus we get: EVM=maxEVMframe,l,EVMframe,h For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×NslotNdlTDD and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2 - ---------------------------------------------------------End of Text Proposal---------------------------------------------------------.

It has been agreed that averaged EVM will be measured separately for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. This is captured well in the TP in the way forward. In case the slot has a combination of SBFD symbols, and flexible or UL symbols, the EVM shall be measured on the SBFD symbols only.

EVM is calculated on SBFD symbols only. Symbols used for guard period and uplink symbols are not included in EVM calculation.

EVM calculation in Annex B.7 can be updated as proposed in the way forward.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our proposals are listed below.

- SBFD-SBFD co-ex case should not be in the scope of Rel-19.

EVM is calculated on SBFD symbols only. Symbols used for guard period and uplink symbols are not included in EVM calculation.

EVM calculation in Annex B.7 can be updated as proposed in the way forward.

# References

- R4-2504752 “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025