3GPP TSG-RAN WG4 Meeting #115	R4-2507648

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	5.25.4

Source:	Ericsson

Title:	Discussion on the LS for updating the definition of 8Rx receivers

Document for:	Discussion

# 1	Introduction

Following the agreement of the definition of MMSE-IRC receiver capability for 8Rx UE below [1], the corresponding CR has been drafted to make updates in TS38.101-4.

Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors.

However, the corresponding definition in RAN2 specification: TS38.306 should be updated as well. In this contribution, we try to provide suggested modification for the spec.

# Discussion

RAN4 has agreed to redefine the simplified and baseline receiver definitions in TS38.101-4 section 3.1 and TS38.306 section 5.2 to the following:

- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors.

Following the agreements above, we propose the changes below to be implemented in TS38.306. The corresponding LS to RAN2 has been added into the section: Annex

Definitions for feature. 8Rx receiver - Baseline 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1..

Proposal 1: Update the receiver definition in TS38.306 as follows:

Definitions for feature. 8Rx receiver - Baseline 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1..

# 3	Summary

In this contribution, we discussed and provided our suggestions on how to update the receiver definition in TS38.306 for 8Rx receivers. Our proposal has been captured as the following:

Proposal 1: Update the receiver definition in TS38.306 as follows:

Definitions for feature. 8Rx receiver - Baseline 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1..

# References

[1]	R4-2504694: Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #114bis, Wuhan, China

# Annex

We provided the Draft LS as follows:

3GPP TSG-RAN WG4 Meeting #xxx                                                                         R4-250xxxx

xxx, xxx, xx – xx xxxxxx, 2025

Title:	LS on updating the UE capability for 8Rx receivers

Response to:

Release:	Release 19

Work Item:	NR_ENDC_RF_FR1_enh2-Perf

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	xxx

#### E-mail Address:	xxx

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1. Overall Description:

RAN4 has agreed to redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:

- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors.

RAN4 respectfully asks RAN2 to update the existing UE capability: ‘SU-MIMO 8Rx receiver’ based on the agreements above.

## 5.2	UE receiver features

Definitions for feature

8Rx receiver - Baseline 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified 8Rx MMSE-IRC receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

# 2. Actions:

To RAN2 group:

RAN4 respectfully asks RAN2 to take above into account.

# 3. Date of Next RAN4 Meetings:

RAN4#116	from 2025-08-25	to 2025-08-29		Bangalore, ID

RAN4#116bis	from 2025-010-13	to 2025-10-17		Prague, CZ