3GPP TSG-RAN4 Meeting #115	        R4-2507653

St Julians, Malta, 19th May – 23rd May 2025

Source: 	Qualcomm Inc.

Title: 	PC2 UE : Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth

Agenda Item:	7.8.2

Document for:	Discussion

WI Code:	NR_FR1_7MHz_BW-Core

# Introduction

There’s an ongoing discussion within RAN4 to support PC2 power levels in band n26. In this proposal, we present a simulation study performed in band n26 for a power class 2 UE highlighting A-MPR behavior for 7 MHz channel bandwidth.

# Discussion

7 MHz channel bandwidth is being added in bands n5 and n26 [1], out of which for band n26 has multiple signalling labels defined as shown below. It is assumed there’s no impact on A-MPR due to NS_100 but it will be verified.

| NR band | Value of additionalSpectrumEmission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| n26 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |

Out of the remaining four network signalling labels applicable to band n26, NS_13 and NS_14 are applied for channel bandwidths up to maximum of 5 MHz and minimum of 10 MHz respectively and are currently being discussed in RAN4. In this proposal, A-MPR study based only on network signalling labels NS_12 is presented.

Additional requirements to be met by the UE when NS_12 is indicated in the cell are tabulated below.

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 7 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# Simulation Study

For analyzing the impact on Tx performance, we simulated the impact on achievable output power. A power class 2 (PC2) transmitter was calibrated to just meet the ACLR with DFT-s-OFDM waveform with Start_RB = 0 and L_CRB = 100 and MPR = 1 dB. Other parameters in the simulation set up are as follows :

- Channel Bandwidth = 7 MHz

- SCS = 15 kHz

- Maximum # of RBs = 35

- Band of Operation : n26 (Uplink : 814 – 849 MHz)

- LO leakage = 28 dBc

- Image = 28 dBc

- CIM3 = 60 dBc

- Network Signalling Labels : NS_12 and NS_15

- Waveforms : CP-OFDM and DFT-s-OFDM

- Modulation : QPSK

Preliminary simulated results are presented below, further corroboration with bench measurements will be performed.

NS_12 :

Carrier frequency of 817.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. The results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Figure 1: A-MPR behavior for NS_12 @ Fc = 817.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Based on these initial results Table 1 and Table 2 are updated along with the chosen carrier frequency. Further study is required to complete the analysis over the range of carrier frequencies and other sets of waveforms as highlighted in the tables. There’s a possibility of defining new A-MPR tables for PC2 supporting band n26 but for now PC3 A-MPR tables are used as a reference.

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 5 dB and ≤ 7 dB respectively.

Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 5 dB and ≤ 7 dB respectively.

Proposal 2: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_12.

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7 MHz | Fc = 817.5 MHz | TBD | TBD | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

Proposal 3: Further study is required over carrier frequencies and sets of waveforms to complete the A-MPR analysis for NS_100, NS_13, NS_14 and NS_15 signalling labels.

# Conclusions

In this contribution A-MPR analysis with network signalling label NS_12 is presented in band n26 for 7 MHz channel bandwidth for a PC2 UE. Following observations and proposals are made:

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 5 dB and ≤ 7 dB respectively.

Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 5 dB and ≤ 7 dB respectively.

Proposal 2: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_12.

Proposal 3: Further study is required over carrier frequencies and sets of waveforms to complete the A-MPR analysis for NS_100, NS_13, NS_14 and NS_15 signalling labels.

# References

[1]	RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5