3GPP TSG-WG4 Meeting #115	R4-2507654

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2836 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [HPUE_NR_FR1_FDD_R18-Core] CR to TS38.101-1-i90: Correction to PC2 intra-band contiguous CA REFSENS - CatA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc., T-Mobile USA. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18-Core |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PC2 intra-band contiguous CA REFSENS requirements are specified as applicable to two uplink carriers in Table 7.3A.2.1-3. This is wrong. These are one uplink carrier requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes the title of Table 7.3A.2.1-3 to “Power class 2 intra-band contiguous CA reference sensitivity with one uplink carrier”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2 intra-band contiguous CA REFSENS requirements are erroneous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.3A.2	Reference sensitivity power level for CA

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. For power class 3, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 7.3A.2.1-1a.

For aggregation of two or more downlink FDD carriers with two uplink carriers, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1 and the power class 3 reference sensitivity power level increased by ΔRIBC. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-1: Power class 3 intra-band contiguous CA  reference sensitivity with two uplink carriers

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3B5 | 15/15 | 20MHz + 20MHz | 25 (RBSTART = 0) | 25 (RBSTART = 81) | 25.1 | 12.4 | FDD |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n5B5 | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 44.6 | 23.0 | FDD |
| CA_n7B | 15/15 | 10MHz + 40MHz | 9 (RBstart = 26) | 36 (RBstart = 180) | 34 | 25 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 5: Applicable only to BCS 1 and to BCS 4 and 5. |  |  |  |  |  |  |  |

Table 7.3A.2.1-1a: Power class 3 intra-band contiguous CA reference sensitivity
with one uplink carrier

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B1 | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 3.8 | FDD |
| NOTE 1: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |

Table 7.3A.2.1-2: Void

For power class 2, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 7.3A.2.1-3. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-3: Power class 2 intra-band contiguous CA reference sensitivity
with one uplink carrier

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | SCC ΔRIBC1 (dB) | SCC ΔRIBC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30 MHz + 5 MHz | 20 (RBstart = 0) | 5.6 3 | 7.73 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with 1Tx. NOTE 2: Applicable to UE supporting PC2 with 2Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

<END OF CHANGES>