3GPP TSG-RAN WG4 Meeting # 115 	    R4-2507664

Malta, MT, 19 – 24 May 2025

Agenda Item:	7.21.6

Source: 	Ericsson

Title:	On performance requirements for AI/ML based positioning

Document for:	Approval

Introduction

This contribution discusses issues related to the use case 1 in WID for AI/ML for air interface work item. In particular the FFS items from previous meeting are addressed and report mapping tables for use case 3b are proposed.

Discussion

Use case 1

In RAN4#112bis meeting it was agreed to bring analysis on feasibility of defining requirements for positioning use case 1. This issue has then been discussed in RAN4#113, RAN4#114, and RAN4#114bis meetings but is yet to be concluded.

Agreement from RAN4#112bis [1]

- Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible.

The agreement from RAN4#112bis meeting is generic, and is valid for both core requirement, and performance requirement discussions. Since, defining the performance requirement for use case 1 is more challenging than defining the core requirement, in this contribution feasibility issues related to defining the performance requirement for use case 1 are elaborated.

Use case 1 in the WID for AI/ML based positioning refers to a scenario where the AI/ML model to support the positioning functionality is deployed at the UE [2]. The output of the AI/ML model is the location of the UE as shown in Figure 1. This use case is no different than the UE-based positioning supported by the NR specifications other than the fact that the measurements performed by the UE on PRS resources is used by the UE as an input to the AI/ML model to infer its location in contrast to the legacy UE-based positioning method where the measurements performed by the UE on the PRS resources are used by the UE to estimate its location. For legacy UE-based positioning, performance requirement in terms of the accuracy of the location estimated by the UE is not specified in RAN4 specification, even though the reporting of estimated location from the UE to the network is supported by TS 37.355.

Observation 1: Performance requirement in terms of the accuracy of the location estimated by the UE is not specified for non-AI/ML UE based positioning supported by the NR specifications.

Figure 1. Use case 1 for AI/ML based positioning.

Accuracy of the estimated UE position, among others, highly depends on the accuracy of the measurement used to estimate the UE position, and the scenario in which the UE is located while performing the measurements to estimate its location. To give a better overview, simulation results showing the accuracy of UE positioning in urban macro (UMa), urban micro (UMi), indoor open office (IOO) and indoor factory (InF) scenarios are presented. Parameters specific to each of these scenarios are taken from TR 38.857 [3]. As the accuracy of the positioning measurements depend on the PRS processing capability of the UE, PRS bandwidth in all simulations is fixed to 100MHz. Further to this, the simulation results discussed in this paper do not use AI/ML models to infer UE location and the results are presented for DL-TDoA based method in FR1.

Figure 2 shows the comparison of positioning accuracies in UMa, UMi, and IOO scenarios. As can be observed the positioning accuracy is different in different scenarios, where the positioning accuracy is highest in IOO and lowest in UMa. Based on this evaluation, it is fair to conclude that the achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be observed in different scenarios.

Figure 2. Positioning accuracy in urban macro, urban micro, and indoor open office scenarios in FR1.

Figure 3 and Figure 4 show positioning accuracies in indoor factory (InF) scenarios. Deployment considered for InF simulations is shown in Figure 5. Results in Figure 3 are for InF dense high (InF-DH) scenario that characterizes presence of dense clutters in InF environment. Results in Figure 4 are for InF sparse high (InF-SH) scenario that characterizes an InF environment with significantly less dense clutters compared to InF-DH scenario. Therefore, the number of LoS links between the UE performing measurements on PRS resources transmitted by 18 TRPs that are deployed as shown in Figure 5 is significantly high in InF-SH in comparison to InF-DH scenario. Due to the availability of large number of LoS links most of the positioning measurements are not impacted by the NLoS excess delay and therefore higher positioning accuracy is achieved in InF-SH scenario in comparison to the InF-DH scenario. Another important observation that can be made in the results for InF scenarios is the impact of geometric dilution of precision (GDOP). To highlight the impact of GDOP, positioning accuracy of UEs that are at the center of the InF deployments while performing the measurements are denoted by the curve for UE(s) in convex hull in Figure 2 and Figure 3. Positioning accuracy of UEs in the center of the deployment is higher than the positioning accuracy of the UEs that are not at the center of the deployment.

Observation 2: Achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be achieved in different scenarios.

Figure 3. Positioning accuracy in indoor factory-dense high scenario in FR1.

Figure 4.  Positioning accuracy in indoor factory-sparse high scenario in FR1.

InF-DH: W = 120m, L = 60m, and D = 20m.

InF-SH: W = 300m, L = 150m, and D = 50m.

Figure 5.  Indoor factory (InF) deployment. Black dots denote location of deployed TRPs.

During the Rel. 18 study item on AI/ML based positioning, achievable positioning accuracy of direct AI/ML based methods were studied in InF scenarios. Based on the results submitted by the participating companies, it was observed that the direct AI/ML positioning can significantly improve the positioning accuracy in comparison to the existing RAT-based positioning methods when the generalization aspects are not considered [3]. Generalization capability of direct AI/ML positioning was evaluated when the model is trained in one InF scenario and tested in different InF scenario, where InF scenarios considered for evaluation were different in terms of the clutter parameters and the density of the clutters present in the InF environment [3]. This signifies that the observation 2 also holds true for the direct AI/ML positioning considered in use case 1. We therefore do not see a feasibility of defining a general requirement in terms of the accuracy of estimated UE position to be met by UE in any scenario for case 1. Furthermore, it is also our observation that for UE based positioning, UE can also use measurements from sensors such as WLAN, Bluetooth, A-GNSS etc. to derive its location. UE can in principle combine measurements from different sensors and achieve positioning accuracy requirements mandated by TS 22.261 to support use cases requiring different service levels [4]. Defining requirements for this type of scenario is however out of the RAN4 scope.

As highlighted above, the performance requirement for use case 1 is scenario dependent. From the simulation point of view, it may be possible to identify one of the scenarios, run the simulations, and discuss the accuracy requirement based on the simulation results. Since testing for use case 1 should be conducted in the scenario for which the requirements are defined for, producing a test setup that mimics, for example InF or IOO or UMi or UMa, scenario with lot more cells than in the legacy setup is hard to be achieved in a lab.

Observation 3: Producing a test setup that mimics for example InF or IOO or UMi or UMa with lot more cells than in the legacy setup is hard to be achieved in a lab.

Proposal 1: RAN4 to conclude that defining requirement in terms of the accuracy of estimated UE position for case 1 to be met in any scenario is not feasible.

Use case 3b

In RAN4#112 meeting it was agreed to postpone the discussion on the metric to be reported by the gNB in RAN4 until an agreement on gNB reporting metric has been agreed in RAN1. In previous meetings, RAN1 has discussed gNB reporting metric and reached to the conclusion that for use case 3b, gNB can report path-based or sample-based measurements to the LMF. The agreements from RAN1 meetings are copied here for reference purpose.

Agreement from RAN4#112 [9]:

- Postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

RAN1 agreements on gNB reporting for use case 3b.

Agreement from RAN1#116 [10]

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information.

- paired timing information and power information.

Agreement from RAN1#119 [11]

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt´ values of the estimated channel response in time domain. The Nt´ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt´ values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt´ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt´, k to gNB via NRPPa. Candidate set values:

  - Nt´<=24. FFS: Nt´ values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt´, Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement from RAN1#120 [12]

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting:

- k = {0...5}

Agreement from RAN1#120 [12]

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt´ values” in RAN1#119 agreement is resolved by supporting:

- Nt´ = {8, 16, 24}

From the agreements from RAN1#119 and RAN1#120 the following observations on measurement reporting from gNB for use case 3b can be made:

Observation 4: Depending on the request from LMF, gNB can report path-based measurements to the LMF. For such measurement reporting, no enhancement to Rel. 18 reporting has been introduced.


Observation 5: Depending on the request from LMF, gNB can report sample-based measurements to the LMF. For such measurement reporting, report mapping table needs to be specified in RAN4 specification.

Observation 6: For sample-based reporting, gNB may either report timing measurement or timing and power measurements associated to Nt´ samples of the estimated channel response.

Observation 7: Reporting granularities for sample-based timing measurement reporting is same as the reporting granularities supported for legacy measurements UL-RToA and gNB Rx-Tx time difference measurement.

Based on the RAN1 agreements, report mapping table for the timing measurements associated to the Nt´ samples of the estimated channel response can at least be concluded now in RAN4. Report mapping for the power measurements associated to the samples of the estimated channel response for which the timing measurements are reported can be discussed later after the issue has been concluded in RAN1.

In RAN1#116bis meeting, RAN1 agreed to reuse reference time defined for the existing UL RTOA measurement. The reporting range for UL RTOA can therefore be reused for sample-based measurement reporting for use case 3b. The measurement report mapping tables for the RAN1 agreed reporting granularity factors are proposed in Table 1, Table 2, Table 3, Table 4, Table 5, and Table 6.

Agreement from RAN1#116bis [13]

- For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215.

- FFS: whether it is applicable when Case 3b is used to support multi-RTT.

Proposal 2: Reporting range for sample-based timing measurements is from -985024´Tc to 985024´Tc.

Table 1. Measurement report mapping for k=0.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_0000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_0000001 | -985024 £ SAMPLE_TIME < -985023 | Tc |
| SAMPLE_TIME_0000002 | -985023 £ SAMPLE_TIME < -985022 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_0985023 | -2 £ SAMPLE_TIME < -1 | Tc |
| SAMPLE_TIME_0985024 | -1 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_0985025 | 0 < SAMPLE_TIME £ 1 | Tc |
| SAMPLE_TIME_0985026 | 1 < SAMPLE_TIME £ 2 | Tc |
| SAMPLE_TIME_0985027 | 2 < SAMPLE_TIME £ 3 | Tc |
| … | … | … |
| SAMPLE_TIME_1970048 | 985023 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_1970049 | 985024 < SAMPLE_TIME | Tc |

Table 2. Measurement report mapping for k=1.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985022 | Tc |
| SAMPLE_TIME_000002 | -985022 £ SAMPLE_TIME < -985020 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_492511 | -4 £ SAMPLE_TIME < -2 | Tc |
| SAMPLE_TIME_492512 | -2 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_492513 | 0 < SAMPLE_TIME £ 2 | Tc |
| SAMPLE_TIME_492514 | 2 < SAMPLE_TIME £ 4 | Tc |
| SAMPLE_TIME_492515 | 4 < SAMPLE_TIME £ 6 | Tc |
| … | … | … |
| SAMPLE_TIME_985024 | 985022 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_985025 | 985024 < SAMPLE_TIME | Tc |

Table 3. Measurement report mapping for k=2.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985020 | Tc |
| SAMPLE_TIME_000002 | -985020 £ SAMPLE_TIME < -985018 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_246255 | -8 £ SAMPLE_TIME < -4 | Tc |
| SAMPLE_TIME_246256 | -4 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_246257 | 0 < SAMPLE_TIME £ 4 | Tc |
| SAMPLE_TIME_246258 | 4 < SAMPLE_TIME £ 8 | Tc |
| SAMPLE_TIME_246259 | 8 < SAMPLE_TIME £ 12 | Tc |
| … | … | … |
| SAMPLE_TIME_492512 | 985020 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_492513 | 985024 < SAMPLE_TIME | Tc |

Table 4. Measurement report mapping for k=3.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985016 | Tc |
| SAMPLE_TIME_000002 | -985016 £ SAMPLE_TIME < -985008 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_123127 | -16 £ SAMPLE_TIME < -8 | Tc |
| SAMPLE_TIME_123128 | -8 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_123129 | 0 < SAMPLE_TIME £ 8 | Tc |
| SAMPLE_TIME_123130 | 8 < SAMPLE_TIME £ 16 | Tc |
| SAMPLE_TIME_123131 | 16 < SAMPLE_TIME £ 24 | Tc |
| … | … | … |
| SAMPLE_TIME_246256 | 985016 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_246257 | 985024 < SAMPLE_TIME | Tc |

Table 5. Measurement report mapping for k=4.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985008 | Tc |
| SAMPLE_TIME_000002 | -985008 £ SAMPLE_TIME < -984992 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_061563 | -32 £ SAMPLE_TIME < -16 | Tc |
| SAMPLE_TIME_061564 | -16 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_061565 | 0 < SAMPLE_TIME £ 16 | Tc |
| SAMPLE_TIME_061566 | 16 < SAMPLE_TIME £ 32 | Tc |
| SAMPLE_TIME_061567 | 32 < SAMPLE_TIME £ 48 | Tc |
| … | … | … |
| SAMPLE_TIME_123128 | 985008 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_123129 | 985024 < SAMPLE_TIME | Tc |

Table 6. Measurement report mapping for k=5.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_00000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_00001 | -985024 £ SAMPLE_TIME < -984992 | Tc |
| SAMPLE_TIME_00002 | -984992 £ SAMPLE_TIME < -984960 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_30781 | -64 £ SAMPLE_TIME < -32 | Tc |
| SAMPLE_TIME_30782 | -32 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_30783 | 0 < SAMPLE_TIME £ 32 | Tc |
| SAMPLE_TIME_30784 | 32 < SAMPLE_TIME £ 64 | Tc |
| SAMPLE_TIME_30785 | 64 < SAMPLE_TIME £ 96 | Tc |
| … | … | … |
| SAMPLE_TIME_61564 | 984992 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_61565 | 985024 < SAMPLE_TIME | Tc |

Proposal 3: Table 1 is used to define measurement report mapping table for sample-based timing measurement reporting for reporting granularity corresponding to k = 0.

Proposal 4: Table 2 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 1.

Proposal 5: Table 3 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 2.

Proposal 6: Table 4 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 3.

Proposal 7: Table 5 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 4.

Proposal 8: Table 6 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 5.

Summary

In this contribution, FFS items from previous meeting are addressed and report mapping tables for use case 3b are proposed. The observations and proposals below summarize the discussions presented in the paper.

Observation 1: Performance requirement in terms of the accuracy of the location estimated by the UE is not specified for non-AI/ML UE based positioning supported by the NR specifications.

Observation 2: Achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be achieved in different scenarios.

Observation 3: Producing a test setup that mimics for example InF or IOO or UMi or UMa with lot more cells than in the legacy setup is hard to be achieved in a lab.

Proposal 1: RAN4 to conclude that defining requirement in terms of the accuracy of estimated UE position for case 1 to be met in any scenario is not feasible.

Observation 4: Depending on the request from LMF, gNB can report path-based measurements to the LMF. For such measurement reporting, no enhancement to Rel. 18 reporting has been introduced.

Observation 5: Depending on the request from LMF, gNB can report sample-based measurements to the LMF. For such measurement reporting, report mapping table needs to be specified in RAN4 specification.

Observation 6: For sample-based reporting, gNB may either report timing measurement or timing and power measurements associated to Nt´ samples of the estimated channel response.

Observation 7: Reporting granularities for sample-based timing measurement reporting is same as the reporting granularities supported for legacy measurements UL-RToA and gNB Rx-Tx time difference measurement.

Proposal 2: Reporting range for sample-based timing measurements is from -985024´Tc to 985024´Tc.

Proposal 3: Table 1 is used to define measurement report mapping table for sample-based timing measurement reporting for reporting granularity corresponding to k = 0.

Table 1. Measurement report mapping for k=0.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_0000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_0000001 | -985024 £ SAMPLE_TIME < -985023 | Tc |
| SAMPLE_TIME_0000002 | -985023 £ SAMPLE_TIME < -985022 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_0985023 | -2 £ SAMPLE_TIME < -1 | Tc |
| SAMPLE_TIME_0985024 | -1 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_0985025 | 0 < SAMPLE_TIME £ 1 | Tc |
| SAMPLE_TIME_0985026 | 1 < SAMPLE_TIME £ 2 | Tc |
| SAMPLE_TIME_0985027 | 2 < SAMPLE_TIME £ 3 | Tc |
| … | … | … |
| SAMPLE_TIME_1970048 | 985023 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_1970049 | 985024 < SAMPLE_TIME | Tc |

Proposal 4: Table 2 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 1.

Table 2. Measurement report mapping for k=1.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985022 | Tc |
| SAMPLE_TIME_000002 | -985022 £ SAMPLE_TIME < -985020 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_492511 | -4 £ SAMPLE_TIME < -2 | Tc |
| SAMPLE_TIME_492512 | -2 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_492513 | 0 < SAMPLE_TIME £ 2 | Tc |
| SAMPLE_TIME_492514 | 2 < SAMPLE_TIME £ 4 | Tc |
| SAMPLE_TIME_492515 | 4 < SAMPLE_TIME £ 6 | Tc |
| … | … | … |
| SAMPLE_TIME_985024 | 985022 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_985025 | 985024 < SAMPLE_TIME | Tc |

Proposal 5: Table 3 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 2.

Table 3. Measurement report mapping for k=2.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985020 | Tc |
| SAMPLE_TIME_000002 | -985020 £ SAMPLE_TIME < -985018 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_246255 | -8 £ SAMPLE_TIME < -4 | Tc |
| SAMPLE_TIME_246256 | -4 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_246257 | 0 < SAMPLE_TIME £ 4 | Tc |
| SAMPLE_TIME_246258 | 4 < SAMPLE_TIME £ 8 | Tc |
| SAMPLE_TIME_246259 | 8 < SAMPLE_TIME £ 12 | Tc |
| … | … | … |
| SAMPLE_TIME_492512 | 985020 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_492513 | 985024 < SAMPLE_TIME | Tc |

Proposal 6: Table 4 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 3.

Table 4. Measurement report mapping for k=3.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985016 | Tc |
| SAMPLE_TIME_000002 | -985016 £ SAMPLE_TIME < -985008 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_123127 | -16 £ SAMPLE_TIME < -8 | Tc |
| SAMPLE_TIME_123128 | -8 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_123129 | 0 < SAMPLE_TIME £ 8 | Tc |
| SAMPLE_TIME_123130 | 8 < SAMPLE_TIME £ 16 | Tc |
| SAMPLE_TIME_123131 | 16 < SAMPLE_TIME £ 24 | Tc |
| … | … | … |
| SAMPLE_TIME_246256 | 985016 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_246257 | 985024 < SAMPLE_TIME | Tc |

Proposal 7: Table 5 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 4.

Table 5. Measurement report mapping for k=4.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_000000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_000001 | -985024 £ SAMPLE_TIME < -985008 | Tc |
| SAMPLE_TIME_000002 | -985008 £ SAMPLE_TIME < -984992 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_061563 | -32 £ SAMPLE_TIME < -16 | Tc |
| SAMPLE_TIME_061564 | -16 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_061565 | 0 < SAMPLE_TIME £ 16 | Tc |
| SAMPLE_TIME_061566 | 16 < SAMPLE_TIME £ 32 | Tc |
| SAMPLE_TIME_061567 | 32 < SAMPLE_TIME £ 48 | Tc |
| … | … | … |
| SAMPLE_TIME_123128 | 985008 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_123129 | 985024 < SAMPLE_TIME | Tc |

Proposal 8: Table 6 is used to define measurement report mapping for sample-based timing measurement reporting for reporting granularity corresponding to k = 5.

Table 6. Measurement report mapping for k=5.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SAMPLE_TIME_00000 | -985024 > SAMPLE_TIME | Tc |
| SAMPLE_TIME_00001 | -985024 £ SAMPLE_TIME < -984992 | Tc |
| SAMPLE_TIME_00002 | -984992 £ SAMPLE_TIME < -984960 | Tc |
| ¼ | ¼ | … |
| SAMPLE_TIME_30781 | -64 £ SAMPLE_TIME < -32 | Tc |
| SAMPLE_TIME_30782 | -32 £ SAMPLE_TIME £ 0 | Tc |
| SAMPLE_TIME_30783 | 0 < SAMPLE_TIME £ 32 | Tc |
| SAMPLE_TIME_30784 | 32 < SAMPLE_TIME £ 64 | Tc |
| SAMPLE_TIME_30785 | 64 < SAMPLE_TIME £ 96 | Tc |
| … | … | … |
| SAMPLE_TIME_61564 | 984992 < SAMPLE_TIME £ 985024 | Tc |
| SAMPLE_TIME_61565 | 985024 < SAMPLE_TIME | Tc |

References

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated.

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

- TR 38.857, Study on NR Positioning Enhancements (Release 17), 3GPP.

- TR 38.847, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 3GPP.

- TS 22.261, Service requirements for the 5G system (Release 18), 3GPP.

- R4-2502856, WF on the requirements for AI/ML air interface, Qualcomm Incorporated.

- R4-2403712, WF on AI/ML, Qualcomm Incorporated.

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated.

- R4-2414323, WF on AI/ML, Qualcomm Incorporated.

- R1-2401825, Summary #4 of specification support for positioning accuracy enhancement, Moderator(Ericsson).

- R1-2410718, Summary #5 of specification support for positioning accuracy enhancement, Moderator (Ericsson).

- R1-2501414, Summary #5 of specification support for positioning accuracy enhancement, Moderator (Ericsson).

- R1-2403740, Summary #6 of specification support for positioning accuracy enhancement, Moderator (Ericsson).