3GPP TSG-RAN WG4 Meeting #115 	    R4-2507667

Malta, MT, 19 – 24 May 2025

Agenda Item:	7.23.3.1

Source: 	Ericsson

Title:	On requirements for RRM measurement prediction for AI/ML based mobility

Document for:	Approval

Introduction

This contribution discusses issues related to the RRM measurement prediction for AI/ML aided mobility. The issues related to consideration for temporal prediction, definition of relative RSRP accuracy, and RLF prediction are particularly discussed.

Discussion

Temporal Prediction

Two cases, as elaborated in Figure 1 and Figure 2, are being considered in RAN2 evaluations for the temporal prediction of RRM measurements. In both cases, measurements performed on a cell is used by the UE to predict the RRM measurements for the future time instances for the same cell.

Temporal prediction case-A:

In temporal prediction case-A, UE performs measurement within the observation window (OW) and uses these measurements to perform prediction of measurements valid for the prediction window (PW). An OW consists of one or more measurement occasions and within a PW there can be one or more than one prediction occasions. A high-level overview of OW, measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 2. The prediction occasions or PW starts after the completion of the measurement occasions or OW in temporal prediction case-A.

Figure 1.  Example elaboration of temporal prediction case-A.

Temporal prediction case-B:

Like the temporal prediction case-A, an OW consists of one or more measurement occasions and within a PW there can be one or more prediction occasions in temporal prediction case-B. The UE is expected to perform measurements in measurement occasions and predict the measurements valid for the prediction occasions inside the prediction window. Unlike temporal prediction case-A, within the OW the UE can skip performing measurements on some of the measurement occasions in temporal prediction case-B. A high-level overview of OW, measurement occasions within OW, skipped measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 3. Like temporal prediction case-A, the prediction occasions or PW starts after the completion of the measurement occasions or OW in temporal prediction case-B.

Figure 2. Example elaboration of temporal prediction case-B.

Simulation results submitted by the participating companies for the intra-frequency intra-cell scenarios were analyzed in RAN2#127bis and RAN2#128 meetings and the following were concluded.

Agreements from RAN2#127bis [1]

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors.

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy. RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy.

Agreements from RAN2#128 [2]

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet.

- We can capture in the TR how the skipping pattern affects the performance.

From the RAN2 agreements above, it can concluded that the accuracy of the predicted measurements for a fixed measurement reduction rate in temporal domain (MRRT) in temporal prediction case-B depends on the measurement skipping pattern governed by the UE within an OW.

Observation 1: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW.

In RAN4#114 meeting it was agreed to consider the impact of UE speed, MRRT, OW, PW and the number of measurements performed during the OW on the prediction accuracy requirements for temporal predictions. The agreement however does not consider the impact of measurement skipping pattern on the prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT. This issue shall be considered for further study in RAN4.

Agreement from RAN4#114 [3]

- RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

Proposal 1: RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B.

Relative RSRP accuracy definition

Definition of relative RSRP accuracy has been discussed in the last couple of RAN4 meetings. The latest agreement on the definition lists some FFS items that need to be resolved.

Agreement from RAN4#114bis [4]:

- For intra-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - FFS if this definition will be needed.

- For inter-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

    - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

    - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

- FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

The first thing we would like to highlight is that the agreements reached in the last meeting for the definition of relative accuracy for intra-frequency and inter-frequency L3-RSRP measurement needs to be reformulated to clarify that the requirements under discussion are the relative accuracy requirements and not the requirements for predicted relative L3-RSRP measurements. In practice the UE predicts measurements for cells in same carrier frequency or in different frequencies. From the requirement point of view the relative accuracy of the predicted measurements needs to be within a specified threshold. This is not quite well reflected in the agreements reached in the last meeting. Therefore, we would like to propose reformulation of the term “Accuracy of predicted relative L3-RSRP” to “Relative accuracy of  predicted L3-RSRP measurements”.

Proposal 2: Reformulate the term “Accuracy of predicted relative L3-RSRP” in the agreements reached in RAN4#114bis to “Relative accuracy of  predicted L3-RSRP measurements”.

In addition, we also foresee a need for the relative accuracy between the measured cell and predicted cell is within an accuracy threshold for intra-frequency L3-RSRP particularly for the case where one of the cells in the formula refers to the reported measurement from a measured cell that is used to predict L3-RSRP measurement of the inferred cell in the same carrier frequency. This is needed to also ensure predicted measurement have the same level of bias (if present) between the measured and predicted measurements. A similar need should also be mandated for the UE to meet for inter-frequency L3-RSRP measurement. The square bracket in the agreement for the definition of relative accuracy of predicted L3-RSRP for inter-frequency scenario should be removed for the case where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

Proposal 3: Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency.

Proposal 4: Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

The definition for beam level intra-frequency L3-RSRP relative accuracy was agreed for FFS in the last meeting. In our understanding the relative accuracy for beam level L3-RSRP measurements from the same cell is not relevant for L3-based mobility and is therefore not needed.

Proposal 5: Definition for relative accuracy of predicted L3-RSRP measurements in intra-frequency scenarios where beam 1 and beam 2 are from the same cell is not needed.

The agreement from the last meeting in our understanding does not address the temporal prediction cases. For temporal domain predictions the relative accuracy requirement should be based on two measurements measured or predicted from the same cell or beam. The relative accuracy definition of predicted L3-RSRP measurement should therefore be extended to also capture the temporal domain prediction scenarios.

Proposal 6: Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as:

- Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2).

- Reported RSRP2 can be predicted or measured.

- T1 and T2 refer to time instances for temporal domain prediction.

RLF Prediction

In RAN4#112bis meeting it was agreed to start the discussion on RLF prediction use case from RAN4#114bis meeting. RLF prediction was discussed in RAN2#129 meeting where it was agreed to not study the RLF prediction as a part of Rel. 19 study item. RAN4 discussions should therefore also be aligned with the RAN2 agreement such that the impact on requirement for RLF prediction is not discussed in RAN4 in Rel. 19 study item.

Agreement from RAN4#112bis [5]

- RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2.

- The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025).

- The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

Agreement from RAN2#129 [6]

- RLF prediction will not be studied in Rel-19.

- Further RRM prediction simulations are not expected.  For next specification general impact study phase, we will consider spatial, temporal, and frequency domain cell level prediction.   FFS if L3 beam level needs to be considered.  For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

- Continue with generalization across cell configurations for RRM prediction

- Continue with measurement events

Observation 2: RLF prediction is out of Rel. 19 study item scope.

Summary

In this contribution issues related to consideration for temporal prediction, definition of relative RSRP accuracy, and RLF prediction are particularly discussed. The observation and proposals below summarize the discussion presented in this paper.

Observation 1: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW.

Proposal 1: RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B.

Proposal 2: Reformulate the term “Accuracy of predicted relative L3-RSRP” in the agreements reached in RAN4#114bis to “Relative accuracy of  predicted L3-RSRP measurements”.

Proposal 3: Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency.

Proposal 4: Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

Proposal 5: Definition for relative accuracy of predicted L3-RSRP measurements in intra-frequency scenarios where beam 1 and beam 2 are from the same cell is not needed.

Proposal 6: Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as:

- Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2).

- Reported RSRP2 can be predicted or measured.

- T1 and T2 refer to time instances for temporal domain prediction.

Observation 2: RLF prediction is out of Rel. 19 study item scope.

References

- Chair’s note from RAN2#127bis, RAN2 Chair (InterDigital).

- Chair’s note from RAN2#128, RAN2 Chair (InterDigital).

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2504901, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

- Chair’s note from RAN2#129, RAN2 Chair (InterDigital).