# 3GPP TSG-RAN WG4 Meeting #115		R4-2507671

St. Julian’s, Malta, 19th to 23rd May 2025

Agenda Item:	7.10.3.1

Source:	Ericsson

Title:	On NR NTN and IoT NTN HPUE co-existence

Document for:	Approval

# 1	Introduction

This contribution presents the updated results for NB-IoT NTN HPUE with 1.5 km distance assumptions and views on NR NTN PC1 ACLR discussion.

# 2	Discussion

In the following sections, we present our updated simulation results for NB-IoT NTN HPUE with 1.5 km isolation distance assumption.

## 2.1	Case 4 – TN UL as victim and NB-IoT NTN UL as aggressor

##### NB-IoT NTN; TN BS type – AAS; Deployment Scenario – Urban (With 1.5 km isolation distance)

| GEO | Power Class |  | Required ACIR (dB) | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 16 | 12 | 9 | 6.8 | 4.9 |  |  |  |
|  |  |  | 5%-tile | 20 | 13 | 5.4 | 4.3 | 2.5 |  |  |  |
|  | PC1 | Throughput Loss | Average |  |  | 18 | 14 | 11 | 8 | 5.8 | 4.4 |
|  |  |  | 5%-tile |  |  | 24 | 17 | 8.7 | 4.8 | 3 |  |
| LEO1200 | PC2 | Throughput Loss | Average |  |  | 8 | 6 | 4 | 3 | 2 | 1.5 |
|  |  |  | 5%-tile |  |  | 19 | 10 | 3.2 | 3 | 2.8 | 2.8 |
|  | PC1 | Throughput Loss | Average |  |  |  |  | 6.5 | 5 | 3.5 | 3 |
|  |  |  | 5%-tile |  |  |  |  | 18 | 11 | 7.2 | 3 |

Consider to capture Ericsson’s updated simulation results in the TR 38.863 and simulation results collection document for NB-IoT NTN HPUE with 1.5 km isolation distance assumption.

## 2.2	NR NTN HPUE PC1 ACLR

In the previous way forward [1], it was tentatively agreed to have NR NTN HPUE ACLR as 34 dBc. However, a note was added to further evaluate the throughput degradation on the TN side with 34 dB ACLR.

The performance metric as stated in TR 38.863 is highlighted below –

“For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%”

The ACLR relaxation from 37 dB to 34 dB allows more adjacent channel leakage, increasing interference from NTN UEs to nearby TN cells. With further analysis to study the 5%-ile throughput degradation at 34 dBc, it is observed in our simulations that the degradation is slightly higher than 5% (~7-8%) towards the TN network with zero-kilometer isolation distance assumption i.e. when the NTN UEs are at the edge of the TN clusters.

The 5%-ile throughtput degradation is slightly higher than 5% (~7-8%) towards the TN network with zero km isolation distance assumption i.e. when the NTN UEs are at the edge of the TN clusters.

Introducing an isolation distance could mitigate this effect. However, the relaxation of ACLR may significantly ease the design of PC1 UEs by lowering complexity and cost, which could justify this tradeoff. This performance impact and the associated design benefit should be clearly captured and evaluated in the TR.

The relaxation of ACLR may significantly ease the design of PC1 UEs by lowering complexity and cost, which could justify this tradeoff. This performance impact and the associated design benefit should be clearly captured and evaluated in the TR.

Consider capturing the finding – throughput degradation vs ACLR relaxation along with the justification, in the TR 38.863 in Table 6b.5.1-2 with the following: “Note 3 – It is observed in some company co-existence results that the 5%-ile throughput degradation is slightly higher than 5% with zero km isolation distance assumption. It was however agreed to progress with this ACLR as it faciliates UE RF implementation.”

# Conclusion

In the previous sections we made the following observations:

Observation 1	The 5%-ile throughtput degradation is slightly higher than 5% (~7-8%) towards the TN network with zero km isolation distance assumption i.e. when the NTN UEs are at the edge of the TN clusters.

Observation 2	The relaxation of ACLR may significantly ease the design of PC1 UEs by lowering complexity and cost, which could justify this tradeoff. This performance impact and the associated design benefit should be clearly captured and evaluated in the TR.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider to capture Ericsson’s updated simulation results in the TR 38.863 and simulation results collection document for NB-IoT NTN HPUE with 1.5 km isolation distance assumption.

Proposal 2	Consider capturing the finding – throughput degradation vs ACLR relaxation along with the justification, in the TR 38.863 in Table 6b.5.1-2 with the following: “Note 3 – It is observed in some company co-existence results that the 5%-ile throughput degradation is slightly higher than 5% with zero km isolation distance assumption. It was however agreed to progress with this ACLR as it faciliates UE RF implementation.”

# References

[1]	R4-2505113, “WF on HPUE RF requirements for NR NTN”, Moderator