# 3GPP TSG-RAN WG4 Meeting #115	R4-2507683

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	7.25.3.2

Document for:	Approval

1 Introduction

During RAN4#114bis, BS RF requirements for SBFD have been discussed. Following conclusions for existing TX requirements [7], we would like to share our views regarding the open issues. Furthermore, we provide text proposal for the CR drafting for the core specification.

2 Discussion

2.1 Summary of the status on the TX requirements

The status on existing TX RF requirements is summarized in the following table.

| ## Requirements | ## Agreements | ## Completed or open | ## Still open for single operator case? | ## Still open for inter-operator case? |
| --- | --- | --- | --- | --- |
| ## Base Station output power and radiated transmit power | ## R4-2409958 o PSD scaling for normal and SBFD slots/symbols: § No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols § Vendors can declare different TX power values for normal and SBFD slots/symbols | ## Completed | ## N | ## N |
| ## Output power dynamics | ## R4-2413515 • Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement. R4-2419908 l The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured. | ## Completed | ## N | ## N |
| ## Transmit ON/OFF power | ## R4-2413515 • Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol. | ## Completed | ## N | ## N |
| ## Transmitted signal quality | ## R4-2416585 • Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. • Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. R4-2502380 n Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity. | ## Completed | ## N | ## N |
| ## BS OBW | ## Rel-18 SI conclusion in 38.858 the existing OBW requirement shall be applied for the whole BS channel bandwidth in SBFD symbols/slots instead of DL sub-band. | ## Completed | ## N | ## N |
| ## ACLR requirement | ## R4-2413515 • When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. | ## Open | ## N | ## Y |
| ## OBUE requirement | ## R4-2413515 • When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. | ## Open | ## N | ## Y |
| ## transmitter spurious emission | ## Rel-18 SI conclusion in 38.858 all the existing requirements shall also be applied to SBFD-capable BS in SBFD symbols/slots. The requirement of protection of the BS receiver of own or different BS is not applicable for TDD operation. | ## Completed | ## N | ## N |
| ## inter-band co-location and co-existence requirement | ## Rel-18 SI conclusion in 38.858 it was agreed not to update on existing inter-band co-location requirements and manufacturer will declare whether support co-location requirements in SBFD symbols/slots. | ## Completed | ## N | ## N |
| ## Transmitter intermodulation | ## R4-2409958 The transmitter intermodulation requirement is applicable in SBFD slots/symbols: § Follow the existing requirement § No receiver requirement is specified. | ## Completed | ## N | ## N |

It can be found that for single operator case, the requirements are all completed. And the controversial issue is the additional unwanted emission, which is only for inter-operator case. Hence, we propose to start drafting specification for single operator case and continue discussion on inter-operator case.

Proposal 1: for existing TX requirements, it is proposed to start drafting specification for single operator case and continue discussion on inter-operator case.

2.2 Unwanted emissions

Per WF in [11], the following way forward was reached.

  - Issue 1-1: Deployment scenarios for FR1 SBFD macro BS

- Way forward

  - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope.

  - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include:

    - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.:

      - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios

      - Option 2: Co-existence simulation and potentially analysis of co-located scenarios

    - Expected spectrum allocation and sub-band sizes

    - Expected feasible filtering performances considering the above two aspects

In this contribution we discuss the TX part on how to derive the additional emission requirements.

Option 1: Potential expected coupling loss and receiver desensitization.

The discussion can be found in our previous paper [5]. One example is to use the assumption for existing co-existence requirement. It assumes 67 dB MCL and 7dB lower than noise floor, which is derived for 3G system at an intermediate case scenario (577/2 m base station shifting, 2GHz UMTS band) in TR25.942. These requirements are defined as the same for all existing FR1 bands. It can be anticipated that at higher frequency band, e.g. n104 upper 6GHz, similar MCL assumption cover much more scenarios.

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused, if 67 dB MCL and 0.8 dB desensitization is adopted.

Option 2: RAN4 co-ex simulation

Still based on RAN4 co-ex simulation methodology, we use the same simulation assumptions as in Rel-18 SBFD SI and focus on SBFD-SBFD deployment. The only difference is we traverse the ACIR in order to get a desired BS-BS ACIR level.

Following figure demonstrates how UL throughput loss @5%-tile, i.e. cell edge, could be improved with an increasing ACIR. And the results in the tables can be found in [x]

The grid shift is set to 10%. Around 36 dB improvement on ACIR (comparing to the existing 42.5dB ACIR determined by 45dB ACLR and 46dB ACS) can be enough to guarantee <5% UL throughput loss.

The grid shift is set to 20%. Around 28 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

The grid shift is set to 50%. Around 16 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

Depending on the adopted grid shift, the protection level is in the range of -31 dBm/MHz ~ -51 dBm/MHz.

For example, as discussed in [7], it is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for adjacent channel co-existence cases. Using 5% throughput loss as the criterion, around 28 dB improvement are needed, which results a protection level to -43 dBm/MHz (-15 dBm/MHz -28 dB).

Observation 2: Depending on the adopted grid shift, based on the co-existence simulation, the protection level is in the range of -31 dBm/MHz ~ -51 dBm/MHz.

Proposal 2: Depending on the adopted grid shift or the MCL assumption, the additional unwanted emission can be defined.

| Grid Shift | Minimum requirement (dBm/MHz) | MCL assumption |
| --- | --- | --- |
| 50% | -31 | 85 |
| 20% | -43 | 73 |
| 10% | -51 | 65 |

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | [-43~-51] dBm | 1MHz |

==== End of Text proposal ====

Expected spectrum allocation and sub-band sizes

The issue was discussed in R4-2413283. As shown in Table 2.2-1, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. In existing FR1 networks, usually operators hold less than or equal to 400 MHz. For the band n104 which has 700 MHz band width, considering it is allocated to 2 ~ 3 operators, each may hold 200 MHz, 300 MHz or 400 MHz. Table 2.2-1 shows the available guard band for different operator’s spectrum.

Table 2.2-1: available guard band for different operator’s spectrum

| Operator's spectrum | Achieved guard band (MHz) |  | min Ratio of transition guard/sub frequency band | Comments |
| --- | --- | --- | --- | --- |
|  | 2 operator case | 3 operator case (in middle) |  |  |
| 100 MHz | 80 | 40 | 40% | Configuration 1: 40-20-40 Configuration 2: 80-20 |
| 200 MHz | 180 | 80 | 30% | Configuration 1: SBFD 20-80+ TDD 100 |
| 300 MHz | 260 | 100 | 33.30% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100 |
| 400 MHz | 360 | N/A | 90% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100+ TDD 100 |

Expected feasible filtering performances considering the above two aspects

Based on the discussion above on the expected spectrum allocation. To consider the operator’s spectrum is larger than 100 MHz and the implementation aspects. 80 MHz offset could be used as starting point. The feasible attenuation for different cases are shown in Table 2.2-2.

Table 2.2-1: Filtering performance evaluation

| Operator's spectrum | Filtering performance at 80 MHz Freq offset to the operators’ spectrum edge |
| --- | --- |
| 100MHz | 35 dB attenuation |
| 200MHz | 45 dB attenuation |
| 300MHz | 45 dB attenuation |
| 400MHz | 45 dB attenuation |

Analysis of co-located scenarios

As analysed in [] R4-2413283, we did some evaluation on the coupling loss. SA to SA coupling loss for n104 AAS is measured and the average value is shown in the Table 2.2-1. It can be found that ~ 65 dB MCL can cover most of deployments scenarios since the measured cases can present a certain worse case. And the CDF of SA to SA coupling for Case a-2 is shown in Figure 2.2-1. The SA to SA coupling loss is around 52~ 82 dB.

- Horizontal 0°

- Vertical 0°

- Horizontal 180° 0°

Table 2.2-1: SA to SA coupling for co-located scenarios

| Case | Average SA2SA coupling loss (dB) |  |
| --- | --- | --- |
|  | in chamber | in- field |
| Case a-1) Horizontal 0° spacing 50 cm | 68 | 64 |
| Case a-2) Horizontal 0° spacing 10 cm | 65 | 62 |
| Case b) Vertical spacing 70 cm | 77 | 68 |
| Case c) Horizontal 180° spacing 1 m | 81 | 73 |

Figure 2.2-1: CDF of SA to SA coupling for Case a-2

In contribution R4-2502160, some simulation results are presented for A-to-A and A-to-A coupling loss. For band n104, the A-to-A coupling loss is around -77±25 dB and the A-to-SA coupling loss is around -70±20 dB.

From the discussion above, the 100% CDF of the coupling loss is around 50 dB. However, considering the robust of the massive MIMO architecture, it is not necessary to adopt 100% CDF. ~55 dB can be considered as the worst case for band n104. Meanwhile it should be noted the evolution of OTA co-location is being discussed in Rel-19 BS RF evolution WI. The MCL assumption of co-location can be revised based on the outcome from Rel-19 BS RF evolution WI.

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD.

Proposal 1: for existing TX requirements, it is proposed to start drafting specification for single operator case and continue discussion on inter-operator case.

Proposal 2: Depending on the adopted grid shift or the MCL assumption, the additional unwanted emission can be defined.

| Grid Shift | Minimum requirement (dBm/MHz) | MCL assumption |
| --- | --- | --- |
| 50% | -31 | 85 |
| 20% | -43 | 73 |
| 10% | -51 | 65 |

References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2419244, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2501856, On modification of existing RX RF requirements for SBFD

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF, Huawei

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung

- R4-2504300, Deployment scenarios for FR1 SBFD macro BS, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

- R4-2504752, Way Forward for [114bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon