3GPP TSG-RAN WG4 Meeting #115 	R4-2507684

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing RX RF requirements for SBFD

Agenda Item:	7.25.3.3

Document for:	Approval

1 Introduction

During RAN4#114bis, BS RF requirements for SBFD have been discussed [11]. In this contribution, we would like to share our views regarding the open issues.

2 Discussion

2.1 Summary of the status on the RX requirements

The status on existing RX RF requirements is summarized in the following table.

| RX Requirements | Agreements | Completed or open | Still open for single operator case? | Still open for inter-operator case? |
| --- | --- | --- | --- | --- |
| Reference sensitivity level and OTA sensitivity | R4-2419908 l 0.7 dB degradation on top of existing minimum requirements for OTA sensitivity. | Completed | N | N |
| Dynamic range | n R4-2502380 The dynamic range requirements of FR1 can reuse existing requirement. n For FR2, no dynamic range requirement is needed. | Completed | N | N |
| ACS | R4-2416585 • The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. | Open | N | Y |
| In-band blocking | R4-2413515 Agreement: • On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference. • RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements. • For co-existence study, n The reference point for the power level which should be before array gain R4-2416585 • For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. | Open | N | Y |
| Out-of-band blocking | Rel-18 SI conclusion in 38.858 the existing OOBB requirement is still applicable for SBFD-capable BS except for OTA sensitivity degradation with the power level of wanted signal taken into account. | Completed | N | N |
| Receiver spurious emissions | Rel-18 SI conclusion in 38.858 apart from existing requirements for normal reception on UL symbols/slots, it’s not necessary to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols/slots. | Completed | N | N |
| Receiver intermodulation |  | Open | N | Y |

It can be found that for single operator case, the requirements are all completed. And the controversial issues including in-band blocking and its related requirements are only for inter-operator case. Hence, we propose to start drafting specification for single operator case and continue discussion on inter-operator case.

Proposal 1: for existing RX requirements, it is proposed to start drafting specification for single operator case and continue discussion on inter-operator case.

In the following clause 2.2~2.4 we provide discussion on the remaining issue for inter-operator case.

2.2 ACS requirement

Per WF in [6],

WF:

- FFS on the followings:

  - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

For ACS and in-band blocking, both requirements are defined out of the BS channel bandwidth instead of uplink subband bandwidth. ACS and in-band blocking are defined with different interferer frequency offsets at adjacent channel(s). Since the blocker are coming from the adjacent BS of another operator and BS always transmits at its whole channel bandwidth, the channel bandwidth of 100 MHz is proposed for interference. And value of the interferer level is 9 dB offset. Hence, we propose to consider the two requirements together.

Proposal 2: it is proposed to consider the ACS and in-band blocking requirements together. The interference level keeps 9 dB offset to the general in-band blocking interference level.

2.3 In-band blocking

Per WF in [11],

## Issue 2-2: In-band blocking for FR1 WA BS - FFS on this issue - Option 1: Define the in-band blocking requirement with the same grid-shift [TBD] for both SBFD-TDD adjacent operator scenario and SBFD-SBFD adjacent operator scenario. Additional emission requirement is included in this release - Option 2: Define the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19. If a decision is made to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario

Firstly for SBFD-TDD adjacent operator scenario, due to the principle that no change on the ACLR/emission from legacy TDD BS, improving the blocking capability of SBFD only would not improve the co-existence performance further. Assuming the actual ACLR/emission is ideally 10 dB lower than that in the specification, higher than -27 dBm (-43 dBm-10-6) may not be helpful, which is already lower than the simulation results with 100% grid shift. Hence from our results with 100% grid shift, it is 17 dB higher than existing in-band blocking limits. Furthermore, the co-existence shows that only 100% grid shift is feasible for SBFD-TDD scenario, we should define requirements based on the corresponding grid shift.

Proposal 3: As a general requirement, it is proposed to adopt 100% grid shift to derive the in-band blocking level for SBFD-TDD adjacent operator scenario.

Proposal 4: it is proposed to adopt -25 dBm as the in-band blocking level for SBFD-TDD adjacent operator scenario.

Secondly, we provide the study on SBFD-SBFD adjacent operator scenario.

In our view eventually, the same grid shift assumption should be adopted for both TX and RX requirements derivation since both requirements are defined for SBFD BS and the same performance shall be guaranteed under the same deployment.

As those requirements are actually the guarantee of SBFD network performance, we feel the need to collect some measurements based on real deployment in order to cross-check the reliability of RAN4 co-ex simulation. To be specific, we extract the site location from a region where typical co-existence among operators with active NR traffic can be identified. Note that the concerning deployment is dense urban macro in C band, co-location case is also included.

According to the CDF of site distance, following characteristics for such real deployment can be observed:

- Site distance is smaller than 10m @11%-tile of the CDF

- Site distance is smaller than 50m @45%-tile of the CDF

While in RAN4 co-ex simulation, 10% grid shift (site distance sets to 500m) means that the adjacent BS is only 28.8m away, the distance would increase to 57.6m for 20% grid shift and to 144m for 50% grid shift.

Based on base station locations in one example region, the CDF of in-band blocking is simulated and the results are shown below. It is should be noted the statistics the co-location cases is not excluded. If it does the number will further lower.

And our RAN4 simulation results are also put here for comparasion. From the simulation results, for 10% grid shift case, -5.1 dBm blocking level is consistent with ~55 dB MCL, which is the worse case of the co-location coupling loss [12]. It is obvious that using grid shift less than 20% as RAN4 co-ex assumption will lead to an over stringent requirement.

Table 2.1-1: blocking level @ 99%CDF

| Blocking level(dBm) |  |
| --- | --- |
| Grid-shift | 99% CDF |
| 10% | -5.1 |
| 20% | -12.4 |
| 50% | -22.4 |
| 100% | -25.8 |

Observation 1: using grid shift less than 20% as simulation assumption will result over stringent requirement.

For some scenarios which could lead smaller site distance than 20% grid shift, we think it can be addressed by co-location limits.

Observation 2: For some scenarios which could lead smaller site distance than 20% grid shift, we think it can be addressed by co-location limits.

Proposal 5: It is proposed to adopt 20% grid shift to be used for RX requirements definition for SBFD-SBFD adjacent operator scenario.

In addition, from the co-existence study, for 20% grid shift, the ACIR need to improve by 28 dB to achieve good co-existence, which means -43+28= -15 dBm will be needed.

From the simulation result of RAN4 simulations and the simulation based on the BS locations of a real deployment at C band, it is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

Then we return to the two options discussed in last meeting.

For option 1 the complete proposal is

  - Using 20% grid shift, to define in-band blocking interferer signal power to -15 dBm/100 MHz for FR1 WA BS and additional emission requirement is included in this release. For some scenarios which could lead smaller site distance than 20% grid shift, it will be addressed by co-location limits.

And for option 2, we believe it is not reasonable to adopt grid-shift 20-50% for SBFD-TDD adjacent operator scenario as discussed in [x]. For TDD-SBFD scenario, co-existence in only feasible for 100% grid-shift. However, we also agree that it is meanness to limit the deployment scenarios with 100% grid-shift only. We suggest to continue the effort on option 1.

Proposal 6: Using 20% grid shift, to define in-band blocking interferer signal power to -15 dBm/100 MHz for FR1 WA BS and additional emission requirement is included in this release. For some scenarios which could lead smaller site distance than 20% grid shift, it will be addressed by co-location limits.

2.4 Receiver intermodulation

Per WF in [4],

Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal. Necessity of New RX intermodulation requirement with 2 interfering signals WF: - FFS on the following: - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

For general in-band blocking case as discussed in clause 2.2, the stronger intermodulation product from closer in-band blocking blocker and self sub-band DL TX is already covered. For example, the interfering signal for existing intermodulation is -52 dBm, while the in-band blocking blocker is -25 dBm or -15 dBm, and the self-interference from DL sub-band is ~ -27 dBm. The power level for either in-band blocking or self-interference are much higher than existing -52 dBm. And both interference are on during the in-band blocking test. Hence there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s).

Proposal 7: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

2.5 Text Proposals for RX

According to the work split, we provide the text proposal for clause 12.3.2, 12.3.3, 12.5.2, 12.5.3 and 12.5.4

Proposal 8: the text proposal for clause 12.3.2, 12.3.3, 12.5.2, 12.5.3 and 12.5.4 is approved for SBFD.

=================== Start of Text Proposal ===================

12.3	Conducted receiver characteristics for SBFD-capable BS

12.3.1	General

12.3.2	Reference sensitivity level

12.3.2.1	General

The reference sensitivity power level is the minimum mean power received at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation at which a throughput requirement shall be met for a specified reference measurement channel.

12.3.2.1	Minimum requirements for BS type 1-H

The same requirement as clause 7.2.2 is applicable to reference sensitivity power level for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.3.3	Dynamic range

#### 12.3.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The same requirement as clause 7.3.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.5.2	OTA sensitivity for SBFD

12.5.2.1	BS type 1-H and BS type 1-O

12.5.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

12.5.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

12.5.3	OTA reference sensitivity level

12.5.3.1	General

The general description of OTA reference sensitivity requirement as clause 10.3.1 shall also be applied for SBFD operation.

12.5.3.2	Minimum requirement for BS type 1-O

The OTA reference sensitivity requirement as specified in clause 10.3.2 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.5.3.3	Minimum requirement for BS type 2-O

The OTA reference sensitivity requirement as specified in clause 10.3.3 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

12.5.4	OTA dynamic range

#### 12.5.4.1	General

The general description of OTA dynamic range requirement as clause 10.4.1 shall also be applied for SBFD operation.

#### 12.5.4.2	Minimum requirement for BS type 1-H

The same requirement as clause 10.4.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

=================== End of Text Proposal ===================

3 Conclusion

In this contribution we discussed on the BS RX requirement for SBFD. We have the following observations and proposals:

Proposal 1: for existing RX requirements, it is proposed to start drafting specification for single operator case and continue discussion on inter-operator case.

Proposal 2: it is proposed to consider the ACS and in-band blocking requirements together. The interference level keeps 9 dB offset to the general in-band blocking interference level.

Proposal 3: As a general requirement, it is proposed to adopt 100% grid shift to derive the in-band blocking level for SBFD-TDD adjacent operator scenario.

Proposal 4: it is proposed to adopt -25 dBm as the in-band blocking level for SBFD-TDD adjacent operator scenario.

Proposal 5: It is proposed to adopt 20% grid shift to be used for RX requirements definition for SBFD-SBFD adjacent operator scenario.

Proposal 6: Using 20% grid shift, to define in-band blocking interferer signal power to -15 dBm/100 MHz for FR1 WA BS and additional emission requirement is included in this release. For some scenarios which could lead smaller site distance than 20% grid shift, it will be addressed by co-location limits.

Proposal 7: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

Proposal 8: the text proposal for clause 12.3.2, 12.3.3, 12.5.2, 12.5.3 and 12.5.4 is approved for SBFD.

References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958, Way Forward for [111][313] NR_duplex_evo,	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416528, Topic summary for [112bis][307] NR_duplex_evo_BSRF, Moderator (Huawei)

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2419245, On modification of existing RX RF requirements for SBFD, Huawei

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF, Huawei

- R4-2504300, Deployment scenarios for FR1 SBFD macro BS, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung

- R4-2504752, Way Forward for [114bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2507683, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon