3GPP TSG-RAN WG4 Meeting #115 	R4-2507685

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	RF requirements for CW

Agenda Item:	7.26.3.3

Document for:	Approval

1 Introduction

At last RAN4#114 meeting, the CW RF requirements have been discussed and agreements can be found in [2].

In this paper we provide further discussion and proposals for open issues.

2 Discussion

Per Study Item discussion, output power, frequency error, unwanted emission including spurious emission are potentially needed. Some progress was made at RAN4#114, the progress is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not (outcome of SI) | WF in R4-2502859 at RAN4#114 | R4-2505097 at RAN4#114bis |
| --- | --- | --- | --- |
| Output power | YES | CW output power [33] dBm | CW output power <=33dBm, and can be declared by manufacture. |
| ON/OFF power | TBD in WI phase | FFS for OFF power | Define CW OFF power Value TBD |
| Frequency error | YES | Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW |  |
| Phase noise | No for inside topology TBD in WI phase for outside topology | Further discuss phase noise requirements | FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI. |
| Unwanted emission | YES | Specify emission requirements, values FFS | FFS on unwanted emission boundary and corresponding values: |
| Spurious emissions | YES | NR spurious emission requirement can be considered as baseline for R19 AIoT CW node | FFS on spurious emissions boundary and corresponding values |
| Intermodulation | TBD in WI phase | FFS |  |

Issue 3-1: CW Output power

- Agreement @RAN4#114bis:

  - CW output power <=33dBm, and can be declared by manufacture.

The text proposal for TS 38.914 can be drafted based on the agreements.

Issue 3-2-1: ON/OFF power

- Agreement:

  - Define CW OFF power

  - Value TBD

On ON/OFF power requirements it was agreed to define CW OFF power. The transmitter is considered OFF when the CW is not allowed to transmit. The NR UE transmit OFF power is -50 dBm per channel bandwidth. If channel bandwidth for CW is defined, we can reuse the UE transmit OFF requirement, otherwise measurement bandwidth can be set as 1MHz as NR UE co-existence requirement.

Proposal 1: the CW OFF power is -50 dBm/CBW or -50 dBm/MHz, depending on whether channel bandwidth is defined for CW.

Issue 3-4: Phase noise

- Agreement:

  - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI.

  - Encourage companies provide phase noise profile

Based on the technical analysis on Phase Noise requirements in SI phase, for inside topology, no explicit phase noise requirement is required. Rationale behind is that the internal CW signal enables convenient single tone signal and phase noise cancellation through calibration mechanisms, eliminating the need for stringent phase noise requirements. And for outside topology, phase noise requirements become solution-dependent and must align with system implementation. For example, if direct RF calibration cable exist between the CW node and Base Station node for real-time cancellation, phase noise impacts on uplink demodulation are significantly mitigated, which is the same situation as the case for inside topology. In such an implementation, there is no need to define phase noise requirements.

Some examples are discussed in TR 38.769 [4]. In the case of separated carrier-wave transmitter and D2R receiver in D1T1-B, considering the path loss of 20 meters distance and antenna mismatch, 67 dB isolation can be assumed. If we set 6 dB as degradation target, around -83 dBc residual integrated phase noise is required. Hence, if 35 dB cancellation is implemented, -48 dBc integrated phase noise is required, which is feasible even without small frequency shift. On the other hand, if phase noise cancellation is not implemented or considered in the solution, the high performance phase noise will be needed, e.g. -83 dBc integrated phase noise.

Figure 2-1: example block diagram of the A-IoT indoor coverage solution

Observation 1: phase noise requirements become solution-dependent

Proposal 2: it is proposed RAN4 to discuss the following options.

Option 1: phase noise requirement is solution-dependent and do not define phase noise requirement for CW node.

Option 2: phase noise is an integer value in the range [-48 to -90 dBc] integrated phase noise at certain frequency offset from xx KHz to yy kHz. The specific value is declared by the vendor. The decision on the value of xx and yy need to wait for RAN1 further agreements on the SFS discussion.

Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

CW node transmits singe tone signal. Except the spurs in the discrete spectrum, the emission in the frequency range immediately outside the CW is phase noise dominate. Table 2.2-1 show one example phase noise performance with carrier frequency 900 MHz. Assuming the CW occupies one PRB, the integrate noise phase at the adjacent PRB is less than -25 dBm/30 kHz, which is already close to the GSM MS emission level (-27 dBm/30 kHz) at 400 kHz offset.

Table 2.2-1: example phase noise performance @ 900 MHz

| Offset | Phase Noise |
| --- | --- |
| 300Hz | -78.5dBc/Hz |
| 500Hz | -83.2dBc/Hz |
| 1kHz | -91dBc/Hz |
| 10kHz | -101dBc/Hz |
| 100kHz | -103dBc/Hz |
| 1MHz | -115dBc/Hz |

Table 2.2-2: GSM emission mask

Table 2.2-3: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

For narrow band signal in sub-1 GHz, NB-IOT and GSM share similar case, and can be used as a reference. The mask of NB-IoT and GSM spectrum emission mask are shown above Table 2.2-2 and Table 2.2-3. The two masks are almost the same in the close end and a little adjust for the far end. And for Local Oscillator, there will be some spurs appear caused by either leakage or mismatch of the charge pump. One example is shown in below figure [3]. The typically value of spur is around 60 dBc below the reference output. Considering this implementation restriction, the low limit of emission can be set to 33-60 =-27 dBm.

Assuming one PRB resource is reserved for CW, the emission mask starts from adjacent PRB. The Δf (foffset) is defined from the center of the CW transmission, the spectrum emission mask for CW node is as defined in Table 2.2-4. Equivalently, the ACLR for 1st adjacent PRB is 40 dBc, 2nd adjacent PRB is 45 dBc and others is 52 dBc. Alternatively defining it using ACLR approach with similar values is also ok.

Proposal 3: it is proposed RAN4 to discuss the following options.

Option 1:

The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

Option 2

If channel bandwidth is defined for CW, defining it using ACLR approach with similar values, i.e. the ACLR for 1st adjacent PRB is 40 dBc, 2nd adjacent PRB is 45 dBc, and others is 52 dBc.

Issue 3-6: Spurious emissions

- Agreement:

  - FFS on spurious emissions boundary and corresponding values:

  - Option 1: The spurious emission of CW starts from the boundary of D2R channel bandwidth.

  - Option 2: The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz

  - Other options are not precluded.

For spurious emission requirement for 30 MHz < f < 1000 MHz is -36 dBm/100KHz, which is 33 - (-36)= 69 dBc below the power of CW. To fulfil such limit, a band level filter will be definitely needed. Hence it is reasonable to reuse the BS approach. Referring to boundary definition of BS, the spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of each supported uplink operating band, up to 10 MHz above the highest frequency of each supported uplink operating band.

Proposal 4：The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as F = 10 MHz

At last meeting, it was agreed that NR spurious emission requirement can be considered as baseline for R19 AIoT CW node. If proposal 4 is agreed, the spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of the supported uplink operating band, up to 10 MHz above the highest frequency of the supported uplink operating band. Spurious emission limits are given in ITU recommendation ITU-R M.1581 and are captured in Table 2.2-5.

Table 2.2-5: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |

Proposal 5: Spurious emissions limits is defined as in Table 2.2-5

Intermodulation

And for intermodulation, since CW node transmits singe tone signal in the Release, the requirement is not needed.

Proposal 6: Intermodulation are not needed for CW node

Issue 3-8: CW others

- Agreement:

  - CW transmission and A-IoT BS downlink data transmission are non-concurrent.

    - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

We agree that CW transmission and A-IoT BS downlink data transmission are non-concurrent. Meanwhile it is not accurate to define as synchronization requirement. The base station and the CW are in the same area, and the CW is controlled by the base station. This is a time sequence control requirement. The control is implementation issue and does not need to be defined in the specification.

Proposal 7: the synchronization requirement between CW node and A-IoT BS is not needed.

3 Conclusion

In this paper we provide discussion and proposals for CW RF requirements.

Proposal 1: the CW OFF power is -50 dBm/CBW or -50 dBm/MHz, depending on whether channel bandwidth is defined for CW.

Proposal 2: it is proposed RAN4 to discuss the following options.

Option 1: phase noise requirement is solution-dependent and do not define phase noise requirement for CW node.

Option 2: phase noise is an integer value in the range [-48 to -90 dBc] integrated phase noise at certain frequency offset from xx KHz to yy kHz. The specific value is declared by the vendor. The decision on the value of xx and yy need to wait for RAN1 further agreements on the SFS discussion.

Proposal 3: it is proposed RAN4 to discuss the following options.

Option 1:

The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

Option 2

If channel bandwidth is defined for CW, defining it using ACLR approach with similar values, i.e. the ACLR for 1st adjacent PRB is 40 dBc, 2nd adjacent PRB is 45 dBc, and others is 52 dBc.

Proposal 4：The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as F = 10 MHz

Proposal 5: Spurious emissions limits is defined as in Table 2.2-5

Proposal 6: Intermodulation are not needed for CW node

Proposal 7: the synchronization requirement between CW node and A-IoT BS is not needed.

References

[1]	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]	R4-2505097, WF on requirements for A-IoT BS and CW, Moderator (Huawei)

[3] PLL Performance, Simulation, and Design

[4] TR 38.769