3GPP TSG-RAN WG4 Meeting #115	R4- 2507722

St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	7.26.3.2

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Document for:	Approval

# 1	Introduction

As per the WF [2] in the last meeting, RAN4 will continue the discussions on the following RF requirements for A-IoT device 1:

Receiver:

- Reference sensitivity

- Maximum input power

Transmitter:

- Backscattering loss

- Modulation quality

- Occupied bandwidth

- Emission requirements

# 2	Discussion

### 2.1 Receiver RF Requirements

Receiver Sensitivity:

Issue 3-4-1: Reference sensitivity

Agreement:

- Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

- At least to define maximum gain direction sensitivity requirements

- FFS on how to different sensitivity requirements on different orientations

The required minimum sensitivity level depends on the application scenarios, which could be categorised as follows.

Table 1: Potential sensitivity levels for different application scenarios

| Sensitivity Level | Use case |
| --- | --- |
| L0 | short distance between the reader and the device, similar applications as UHF RFID tags whose sensitivity ranges from 9dBm to -20dBm |
| L1 | typical D1T1 indoor factory or office scenarios as studied in the SI |
| L2 | applications with larger coupling loss, such as devices attached to metal surface (i.e. large on-object antenna penalty) |

The main concern on introducing multiple levels of sensitivity requirements is the mixed usages of devices meeting different levels in the same deployment. Some devices with weak sensitivity might never be inventoried due to insufficient DL coverage. Before proper control mechanism can be established, it’s worth being cautious to first define the level of sensitivity that meets the need of the indoor factory deployment which was focused in the SI phase.

Following the configuration of the indoor factory deployment (D1T1-B), the minimum received DL power level can be derived as Table 2 below.

Table 2: Worst-case estimation for DL received power

| ISD | 20 meters |  |
| --- | --- | --- |
| Height (H) | 10 meters |  |
| d2D | 20/2 meters |  |
| d3D | 17.3 (=H2+d2D2) meters |  |
| Carrier frequency | 900 MHz |  |
| Tx power | 33 dBm |  |
| Tx antenna gain | 2 dB |  |
| Path loss (InF_DH NLOS) | 59.8 dB |  |
| Shadow fading margin | 4 dB |  |
| Polarisation mismatch loss | 3 dB |  |
| On-object antenna gain penalty | 0.4 dB | 4.7 dB |
| Rx power | -32.2 dBm | -36.5 dBm |

Figure 1 illustrates the indoor factory deployment depicted in Table 2. It’s assumed that the worst-case for DL happens when the device is located in the middle of the square, where the distances to four adjacent readers are equal.

RR

R

R

R

ISD=20M

d2D

Figure 1: Illustration of the indoor factory deployment (Top view)

It can be seen from Table 2 that the minimum received DL power varies from around -32 dBm to -36 dBm. If the Tx power of the reader increases (up to 38dBm), the Rx power would be higher. On the other hand, the on-object antenna gain penalty might increase if the device is attached to metal surfaces (e.g. 10.4 dB on aluminium slab [5]), resulting in lower Rx power.

Additionally, based on initial measurement data of some device prototypes, we think the Rx sensitivity of -33 to -35dBm is feasible.

- Consider to define the minimum sensitivity requirement for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional sensitivity levels can be added in the future if needed.

Table 3: Device Sensitivity at peak antenna gain direction

| Sensitivity Level | Minimum requirement (dBm) |
| --- | --- |
| L1 | ≤ [-33~-35] |

Orientation variation:

Due to the directivity of its antenna, the OTA sensitivity of the device exhibits orientation variation which follows the antenna gain pattern. In 3GPP, two methods have been studied to characterise the orientation property in the Rx sensitivity requirements for NR, i.e.

Option 1: Total Radiated Sensitivity (TRS) in FR1

  - Measure actual EISθ,ϕ at several dozens of grid points, which is time-consuming

  - The result is the average performance over different orientations

The TRS with Anechoic Chamber method is defined as:

TRS=4πθ=0πϕ=02π1EISθθ,ϕ+1EISϕθ,ϕsinθdϕdθ     	   (5.15)

Where the effective isotropic sensitivity (EIS) is defined as the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point.

Where EISθ and EISϕ are the EIS in the corresponding θ and ϕ polarizations.

Option 2: EIS spherical coverage in FR2

- max EIS at Y%-tile CCDF is X dBm

- Unnecessary to measure actual EISθ,ϕ at each grid point, only check against the threshold of X dBm

- The percentage of grid points having EISθ,ϕ>X should not exceed Y%

Use the widely used dipole antenna as an example. The gain pattern of an ideal half-wave dipole can be expressed as:

Gθ=1.64cosπ2cosθ2sin2θ

The HPBW (half-power beamwidth) of the ideal half-wave dipole is around 78 degrees. Hence, for 43% (=78/180) spherical coverage, the degradation is ≤ 3dB. If polarisation mismatch is added, the degradation is ≤ 5.2dB.

- Consider to define the minimum EIS spherical coverage requirements for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional coverage levels can be added in the future if needed.

Table 4: EIS spherical coverage

| Spherical Coverage Level | Minimum requirement |
| --- | --- |
| E1 | ≤ X dBm at Y%-tile CCDF, X and Y are TBD |

Measurement method:

The device cannot access the network unless being triggered by the reader. Unlike other 3GPP RATs, there’re no continuous DL data transmissions for AIoT. One way to evaluate the receiver performance is to measure the success rate of the device that correctly responds to the paging that triggers the random access procedure.

(a)	A-IoT CBRA				(b)	A-IoT CFRA

Figure 2: A-IoT Random Access Procedures

This is similar to the Select-Query-Ack message sequence for measuring Rx sensitivity for RFID tags [4].

- The AIoT paging and CFRA procedure can be used to evaluate the receiver sensitivity with success rate target of >=[90%]. The detailed parameters of the PRDCH and PDRCH (i.e. FRC) are TBD.

Maximum input power:

The device receiver has very limited dynamic range, which is estimated to be around 30 to 40dB. Larger range is possible such as using AGC, but at the cost of increased complexity and power consumption. Given the sensitivity requirement, we propose the max input level as:

- It’s recommended to set the maximum input level to [-5 ~-3] dBm for AIoT devices.

This does not mean that the device wills top working for higher input power levels, but the performance might be degraded.

### 2.2 Transmitter RF requirements

Backscatter output power:

Issue 3-1-1: Transmit output power

Agreement:

- Performance metric of transmit output power:

  - Backscattering loss

- Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power

  - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

- For the Input CW Tx power to define requirements:

  - FFS on the whether to use one value or a range of CW Tx power

  - FFS on the value(s) of CW Tx power

- Consider following aspects to determine backscattering loss to define requirements:

  - modulation loss

  - antenna efficiency

  - backscattering circuitry loss

  - impairment margin

  - minimum distance to the reader

  - differential radar cross section in two different modulation states

  - differential reflection coefficient in two different modulation states

  - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself

  - Other factors not excluded

For A-IoT D1T1-B, the CW node is outside the topology. And the CW node is assumed not to be co-located with the active reader. The CW2D and D2R path loss are likely to be different.

The backscatter loss or reflection gain can be defined as: the backscattered power – incident CW power.

Note that the backscatter power shall be the sum of the power of the left and right side band, excluding the carrier itself.

Similar to DL, the minimum received UL power level can be derived as Table 5 below, following the configuration of the indoor factory deployment (D1T1-B).

Table 5: Worst-case estimation for UL received power

| ISD | 20 meters |  |
| --- | --- | --- |
| Height (H) | 10 meters |  |
| d2D | 10 meters |  |
| d3D | 14.1 (=H2+d2D2) meters |  |
| Carrier frequency | 900 MHz |  |
| Tx power | 33 dBm |  |
| Tx antenna gain | 2 dB |  |
| Path loss (InF_DH NLOS) | 57.9 dB |  |
| Shadow fading margin | 4 dB |  |
| Polarisation mismatch loss | 3 dB |  |
| On-object antenna gain penalty | 0.4 dB | 4.7 dB |
| Total CW2D loss | 65.3 dB | 69.6 dB |
| Rx power @device (CW) | -30.3 | -34.6 |
| Backscatter loss | 6 dB |  |
| Total D2R loss (=total CW2D loss) | 65.3 dB | 69.6 dB |
| Rx antenna gain | 2dB |  |
| Rx power @reader (D2R) | -99.6 dBm | -108.2 dBm |

Figure 2 illustrates the indoor factory deployment depicted in Table 5. It’s assumed that the worst-case for UL happens when the device is located in the middle of two adjacent readers and the CW node is installed in the centre of the square grid.

RR

R

R

R

ISD=20M

d2D

CW

d2D

Figure 3: Illustration of the indoor factory deployment (Top view)

- Assuming the Rx sensitivity of the reader is no worse than -110dBm, the upper bound of the backscatter loss can be in the region of 8~16dB.

As shown in  [8], the ratio of the backscattered power (in the two sidebands) to the incident wave power can be expressed as:

PbsPin=Gtag2λ22π3Γ2-Γ12

where Gtag is the device antenna gain, Γ1,2 are the complex reflection coefficients.

- Further study the suitable requirements on backscatter loss based on the physics of backscattering as well as measurement data.

Modulation quality:

Issue 3-2-1: SFO requirement

Agreement:

- 10% SFO is assumed as the minimum performance for devices.

- FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

Issue 3-2-2: Modulation quality requirements

Agreement:

- Define modulation quality requirements for both BPSK and OOK.

- Consider to define same modulation quality metric for both BPSK and OOK if possible.

- Reference measurement process needs to be discussed

In general, the modulation signal quality sets an upper bound of the SNR seen by the receiver. The potential requirement on the modulation quality depends on the Tx signal design as well as the demodulation processing at the receiver.

- The reference demodulation process should be defined in order to quantify the impact of Tx impairments in the backscattered signal.

As per RAN1 agreement, the device may support OOK or BPSK for D2R. It’s worth noting that for backscatter communication, the modulation is implemented in analogue domain by switching impedances, which is very different from digital modulation seen in other 3GPP RATs.

- The sources and characteristics of the impairments imposed on the backscattered signal should be identified and modelled, so that their impact to the demodulation performance can be quantified.

- To identify the quality metric and corresponding requirements for backscatter modulation, the following need to be studied:

    - The reference demodulation process

    - The sources and characteristics of the impairments, and how to model them

    - To quantify the impact of the backscatter impairments to demodulation

For any fixed load impedance (matched, open-circuit or short-circuit), the device can always backscatter incident CW signals. The spectrum of the backscattered signal will concentrate in the carrier frequency. By switching between two states of load impedance, the spectrum is then split to sidelobes and the first upper sideband and the first lower sideband are used for demodulation.

- Backscatter modulation is implemented by switching impedance in the analogue domain, and the first spectral sidelobes contain the useful information. Without switching, the spectrum of the backscattered signal will concentrate in the centre frequency.

Use OOK as example. Ideally, the bits are modulated to 0s and 1s. So the DC is ½, the average power is ½ and the DC power is ¼. In other words, the average power of the sidelobes is ¼, i.e. half of the backscattered power.

If the backscatter modulation is less effective, the bits might be modulated to 0.5s and 1s. In this case, the DC is 3/4 and DC power is 9/16. The average power is 5/8, and the power of the sidelobes become 1/16. In other words, the majority of the backscattered power is concentrated in the centre frequency, which will be filtered and not contribute to the demodulation.

Table 6 Comparison of OOK modulations

|  | Ideal modulation | Non-ideal modulation (example #1) | Non-ideal modulation (general) |
| --- | --- | --- | --- |
| OOK | {0, 1} | {0.5, 1} | {a0, a0+A} |
| DC | ½ | 3/4 | (2a0 + A)/2 |
| DC power | 1/4 | 9/16 | (2 a0 +A)2/4 |
| Average power | 1/2 | 5/8 | (a02 + (a0 +A)2/2 |
| Average power of sidebands | 1/4 | 1/16 | A2/4 |

- In case a more suitable modulation quality metric cannot be defined in time, the backscatter loss can be viewed as a performance metric to evaluate how effective the backscatter modulation is. The lower the backscatter loss is, the more effective the modulation.

Occupied bandwidth:

Typically, the occupied bandwidth is defined as the bandwidth that contains at least 99% of the signal power. This implies an ACLR requirement of 23dB.

The backscattered D2R signal would contain the odd-order harmonics of the chip rate, and the devices have little suppression of the harmonics due to ultra-low complexity/cost. In theory, the power of the 3rd and 5th harmonics reduce 9.5dB and 14dB compared to the power of the main lobe, respectively. If the D2R channel bandwidth under discussion contains only the main lobe, the channel bandwidth could not contain 99% of the signal power.

Additionally, the measurement receiver would need to cancel the CW leakage directly from the CW node very accurately in order to measure the D2R signal backscattered by the device.

- Consider not to define OBW requirement for backscattering devices.

SEM + Spurious emissions:

Conventionally, the NR/LTE SEM starts from the immediate boundary of the channel bandwidth, i.e. ∆fOOB=0, and spurious emission requirements start from ∆fOOB=CBW+5MHz. And for ∆fOOB=0-1MHz, the emission limit is -13dBm/1% of CBW.

As per current discussion of the D2R channel bandwidth, the spectrum of the 3rd harmonics and beyond is not included. Given that the power of the 3rd harmonic is only 9.5dB lower than the main lobe, the conventional SEM definition could be too tight for D2R transmissions.

According to the ETSI harmonised standard for UHF RFID [6], the spectrum mask starts from +/-400kHz for tag bandwidth of 320kHz, and from +/-800kHz for tag bandwidth of 640kHz. In both cases, the guard band between the channel bandwidth and the SEM is 0.75*CBW.

- Sufficient guard band should be considered when defining the SEM requirements, owing to the presence of odd-order harmonics in the backscattered signal.

For NR or LTE,  the SEM is independent of power classes. In other words, it doesn’t change with the MOP of a UE. It’s expected that device 2 will be introduced in Rel-20 [7]. Since device 2 is also targeted for short distance, indoor deployment, it’s proposed that:

- It’s preferrable that the SEM and spurious emission requirements are forward-compatible with device 2 which is to be introduced in Rel-20.

# 3	Conclusion

In this paper, we have shared further analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Proposal 1: Consider to define the minimum sensitivity requirement for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional sensitivity levels can be added in the future if needed.

Table 3: Device Sensitivity at peak antenna gain direction

| Sensitivity Level | Minimum requirement (dBm) |
| --- | --- |
| L1 | ≤ [-33~-35] |

Proposal 2: Consider to define the minimum EIS spherical coverage requirements for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional coverage levels can be added in the future if needed.

Table 4: EIS spherical coverage

| Spherical Coverage Level | Minimum requirement |
| --- | --- |
| E1 | ≤ X dBm at Y%-tile CCDF, X and Y are TBD |

Proposal 3: The AIoT paging and CFRA procedure can be used to evaluate the receiver sensitivity with success rate target of >=[90%]. The detailed parameters of the PRDCH and PDRCH (i.e. FRC) are TBD.

Proposal 4: It’s recommended to set the maximum input level to [-5 ~-3] dBm for AIoT devices.

Observation 1: Assuming the Rx sensitivity of the reader is no worse than -110dBm, the upper bound of the backscatter loss can be in the region of 8~16dB.

Proposal 5: Further study the suitable requirements on backscatter loss based on the physics of backscattering as well as measurement data.

Observation 2: The reference demodulation process should be defined in order to quantify the impact of Tx impairments in the backscattered signal.

Observation 3: The sources and characteristics of the impairments imposed on the backscattered signal should be identified and modelled, so that their impact to the demodulation performance can be quantified.

Proposal 6: To identify the quality metric and corresponding requirements for backscatter modulation, the following need to be studied:

    - The reference demodulation process

    - The sources and characteristics of the impairments, and how to model them

    - To quantify the impact of the backscatter impairments to demodulation

Observation 4: Backscatter modulation is implemented by switching impedance in the analogue domain, and the first spectral sidelobes contain the useful information. Without switching, the spectrum of the backscattered signal will concentrate in the centre frequency.

Proposal 7: In case a more suitable modulation quality metric cannot be defined in time, the backscatter loss can be viewed as a performance metric to evaluate how effective the backscatter modulation is. The lower the backscatter loss is, the more effective the modulation.

Proposal 8: Consider not to define OBW requirement for backscattering devices.

Proposal 9: Sufficient guard band should be considered when defining the SEM requirements, owing to the presence of odd-order harmonics in the backscattered signal.

Proposal 10: It’s preferrable that the SEM and spurious emission requirements are forward-compatible with device 2 which is to be introduced in Rel-20.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN#106

- R4-2505230 WF on A-IoT device requirements, CMCC, RAN4#114bis

- 3GPP TR 38.870 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS), v18.4.0

- GS1 Tagged-Item Performance Protocol (TIPP) Tagged-Item Grading: Testing Methodology Guideline

- Complete link budgets for backscatter-radio and RFID systems, J. D. Griffin and G. D. Durgin, IEEE antennas and propagation magazine, vol. 51, No. 2, April 2009.

- ETSI EN 302 208, Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum

- RP-243280 Way forward for ambient IoT for REL-19 and REL-20, RAN1 Vice Chair (Huawei), RAN#106

- On complex radar cross section and backscatter modulation efficiency in RFID systems, C. Degen, et al, 2020 14th European Conference on Antennas and Propagation (EuCAP), Copenhagen, Denmark, 2020, pp. 1-5

- On the measurement of the Delta Radar Cross Section for UHF tags, S. Skali, et al, IEEE International Conference on RFID, 2009