# 3GPP TSG-RAN WG4 Meeting #115		        R4-2507723

# St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	5.26.1

Source: 	Huawei, HiSilicon

Title:	(PHS_System_NRonly) On the requirements of modified MPR behavior

Document for:	Approval

# 1	Introduction

As per the WF [1] from last meeting and the current meeting agenda, we continue to discuss the definition of modifiedMPR-Behavior for NS_05(U).

# 2	Discussion

It has been pointed out in our previous contribution [2][3] that there exist ambiguity and even contradiction in the definitions of the modified MPR behaviour in TS 38.101-1.

- There are mixed usages of “shall” and “may” in the definitions of the modified MPR behaviour. Different bands and even different bits for the same band follow different rationales, leading to confusion.

Table 1: Three rationales used in the definitions of modifiedMPR-behaviour

| Rationale | Instances |
| --- | --- |
| #1: “may be set to 1” for earlier releases, “shall be set to 1” for current and future releases | Band n30, n71 n41 bit 0, bit 1 |
| #2: “may be set to 1” for earlier releases, not needed for current and future releases | Band n96, n102 n41 bit 2 |
| #3: “may be set to 1” for any releases | Band n34, n39, n40, , n78, n79 n41 bit 3 |

A band of particular intertest is n41, for which the definitions are duplicated below.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| … |  |  |  |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

It can be observed that:

- For band n41, both bit 0 and bit 1 “shall be set to 1”. It’s unclear how to interpret bit 2 if it’s also set to 1. It’s questionable whether bit 2 can fulfil its intended purpose under existing definitions.

In our view, the main use case for the modifiedMPR-Behavior is when an MPR/A-MPR requirement is changed in an open release (Rel-N). An early release (Rel-M) UE may optionally support the new requirement and indicate it by the modifiedMPR-Behavior parameter. It’s unnecessary for a Rel-N UE to indicate this parameter, since the new MPR/A-MPR requirement is the only one left in the Rel-N spec.

- The modifiedMPR-Behavior IE is useful for a UE of a legacy release to indicate the support of new MPR/A-MPR requirements made after the release was frozen. The network could not tell which MPR/A-MPR requirement the UE is compliant with if using the reported release version alone. However, the problem does not exist for the open release.

Use the new NS_05 requirements [1] for band n1 as example. All Rel-19 UEs shall comply with the updated NS_05 requirements. Unlike legacy UEs, there would be no ambiguity even if modifiedMPR-behavior IE is not signalled.

Furthermore, the initial Rel-20 specs will inherit the requirements of the Rel-19 specs. If the bit “shall be set to 1” in Rel-19, it would remain as “shall be set to 1” in Rel-20. In other words, a Rel-20 UE in the future would use modifiedMPR-behavior IE to indicate the support of some MPR/A-MPR changes made in the past (i.e. Rel-19). This would be illogical and prevent potential update of the MPR/A-MPR requirements in the future.

- Mandatorily setting the modifiedMPR-behavior bit to 1 is unnecessary for the open release, and prevents further update in the future.

To mitigate the forward-compatibility problem, it’s worth noting that some efforts have been made in the FR2 spec [5]. For example, the spec version number is removed from the “Definition” for some bands as shown below.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n257 |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
| n260 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n260 |
| n261 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n261 |

- The release version numbers are omitted in the definitions of modifiedMPRbehavior in TS 38.101-2 for the sake of better forward-compatibility.

Based on the above analysis, we propose to adopt the following principle for the usage of modifiedMPR-Behavior IE.

- Use the modifiedMPR-Behavior IE in legacy releases to allow a UE to indicate the support of new MPR/A-MPR requirements introduced after the release was frozen. Do not use the modifiedMPR-Behavior IE in open release for the sake of forward-compatibility.

Based on the above principle, we propose to define new modifiedMPR-Behavior for band n1 as below, following the update of the requirements for NS_05 and NS_05U [1].

- Add the definition of modifiedMPR-Behavior for band n1 in Rel-16 to Rel-18 specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19. The bit is reserved from Rel-19 onwards.

Table 3: Changes for Rel-16 to Rel-18 specs

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a UE supporting this release of the specification. |

Table 4: Changes for Rel-19 spec

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit is reserved for a Rel-18 or earlier UE. |

Additionally, some maintenance for the modifiedMPR-Behavior table is needed. Hence, we propose:

- Follow the rationale in defining modifiedMPR-Behavior for band n1, align the requirements defined in TS 38.101-1 for other bands.

# 3	Conclusion

Based on our analysis on the current modifiedMPR-Behavior table in existing NR specs, we have made the following observations and proposals:

Observation 1: There are mixed usages of “shall” and “may” in the definitions of the modified MPR behaviour. Different bands and even different bits for the same band follow different rationales, leading to confusion.

Observation 2: For band n41, both bit 0 and bit 1 “shall be set to 1”. It’s unclear how to interpret bit 2 if it’s also set to 1. It’s questionable whether bit 2 can fulfil its intended purpose under existing definitions.

Observation 3: The modifiedMPR-Behavior IE is useful for a UE of a legacy release to indicate the support of new MPR/A-MPR requirements made after the release was frozen. The network could not tell which MPR/A-MPR requirement the UE is compliant with if using the reported release version alone. However, the problem does not exist for the open release.

Observation 4: Mandatorily setting the modifiedMPR-behavior bit to 1 is unnecessary for the open release, and prevents further update in the future.

Observation 5: The release version numbers are omitted in the definitions of modifiedMPRbehavior in TS 38.101-2 for the sake of better forward-compatibility.

Proposal 1: Use the modifiedMPR-Behavior IE in legacy releases to allow a UE to indicate the support of new MPR/A-MPR requirements introduced after the release was frozen. Do not use the modifiedMPR-Behavior IE in open release for the sake of forward-compatibility.

Proposal 2: Add the definition of modifiedMPR-Behavior for band n1 in Rel-16 to Rel-18 specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19. The bit is reserved from Rel-19 onwards.

Table 3: Changes for Rel-16 to Rel-18 specs

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a UE supporting this release of the specification. |

Table 4: Changes for Rel-19 spec

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit is reserved for a Rel-18 or earlier UE. |

Proposal 3: Follow the rationale in defining modifiedMPR-Behavior for band n1, align the requirements defined in TS 38.101-1 for other bands.

# References

- R4-2505222 WF on PHS protectionupdate, KDDI, RAN4#114bis

- R4-2504558 (PHS_System_NRonly) On the requirements of modifiedMPR-Behavior, Huawei, HiSilicon, RAN4#114bis

- R4-2502236 On the requirements of modified MPR behavior, Huawei, HiSilicon, RAN4#114

- 3GPP TS 38.101-1 v18.9.0

- 3GPP TS 38.101-2 v18.9.0