3GPP TSG-RAN WG4 Meeting #115	R4-2507728

St Julian's, Malta, May 19th – 23rd 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | draftCR | rev | -- | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.104 on adding RF requirements for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding RF requirements for SBFD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 12.2 and 12.5 are introduced to capture conducted and radiated transmitter characteristics requirements for SBFD-capable BS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirements for SBFD-capable BS are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2, 12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------Start of change-----------------------------------------------------

## 12.2 Conducted transmitter characteristics for SBFD-capable BS

### 12.2.1     General

Conducted transmitter characteristics requirements apply on the SBFD-capable BS type 1-H including all its functional components active and for all foreseen modes of operation of the BS unless otherwise stated.

### 12.2.2     Base station output power for SBFD

#### 12.2.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD period, in which the rated carrier output power for SBFD period may be different from the rated carrier output powers for transmitter ON period as specified in clause 6.2.1.

#### 12.2.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

### 12.2.3     Output power dynamics for SBFD

#### 12.2.3.1	General

The requirements in clause 12.2.3 apply during SBFD period. Transmitted signal quality (as specified in clause 12.2.5) shall be maintained for the output power dynamics requirements of this clause.

Power control is used to limit the interference level.

#### 12.2.3.2		RE power control dynamic range

#### 12.2.3.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) during SBFD period for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

#### 12.2.3.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.3.2.2 is applicable to RE power control dynamic range requirement for SBFD operation.

#### 12.2.3.3	Total power dynamic range for SBFD

##### 12.2.3.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the total power dynamic range is the average power for single RB transmission as non-SBFD. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 12.2.3.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

## 12.5 Radiated transmitter characteristics for SBFD-capable BS

### 12.5.1     General

Radiated transmitter characteristics requirements apply on the SBFD-capable BS type 1-H, BS type 1-O, or BS type 2-O including all its functional components active and for all foreseen modes of operation of the BS unless otherwise stated.

### 12.5.2     Radiated transmit power for SBFD

#### 12.5.2.1	General

The BS radiated transmit power for SBFD operation is specified over SBFD period, in which the EIRP for SBFD period may be different from the EIRP for transmitter ON period as specified in clause 9.2.1.

#### 12.5.2.2	Minimum requirement for BS type 1-H and BS type 1-O

The same requirement as clause 9.2.2 is applicable to BS radiated transmit power requirement for SBFD operation.

#### 12.5.2.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.2.3 is applicable to BS radiated transmit power requirement for SBFD operation.

### 12.5.3     OTA base station output power for SBFD

#### 12.5.3.1	General

The OTA base station output power for SBFD operation is specified over SBFD period, in which the TRP for SBFD period may be different from the TRP for transmitter ON period as specified in clause 9.3.1.

#### 12.5.3.2	Minimum requirement for BS type 1-O

The same requirement as clause 9.3.2 is applicable to OTA base station output power requirement for SBFD operation.

#### 12.5.3.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.3.3 is applicable to OTA base station output power requirement for SBFD operation.

### 12.5.4     OTA output power dynamics for SBFD

#### 12.5.4.1	General

The requirements in clause 12.5.4 apply during SBFD period. Transmit signal quality (as specified in clause 12.5.6) shall be maintained for the output power dynamics requirements of this clause.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

#### 12.5.4.2		OTA RE power control dynamic range

#### 12.5.4.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) during SBFD period for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

#### 12.5.4.2.2	Minimum requirement for BS type 1-O

The OTA RE power control dynamic range for SBFD operation is specified the same as the conducted RE power control dynamic range requirement for BS type 1-H in clause 12.2.3.2.

#### 12.5.4.3	OTA total power dynamic range

##### 12.5.4.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range for SBFD is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

#### 12.5.4.3.2	Minimum requirement for BS type 1-O and BS type 2-O

The OTA total power dynamic range minimum requirement for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

------------------------------------------End of change------------------------------------------------------