3GPP TSG-RAN WG4 Meeting #115	R4-2507747

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM requirements

Agenda item:	7.30.3

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on open issues after RAN4 #114bis on RRM requirements for XR enhancements [1] and provide respective observations and proposals. Note, agreements reached in RAN4#114bis are contained in [2].

Discussion

Discussion Highlights

In particular, the following aspects are treated in this contribution and respective observations and proposals are made.

- Section 2.2: Measurement gap types

  - Suitable gap types for measurement skipping are Type-2 concurrent MG and NCSG.

  - Pre-configured MG and Rel-18 concurrent gaps are excluded from measurement skipping.

- Section 2.3: Formulas for measurement delay extension

  - Measurement delay extension is determined with skipped MG occasions not included in the CSSF scaling calculation.

  - This applies both for non-DRX mode and DRX mode.

- Section 2.4: XR skipping when multiple layers are configured

  - High priority frequency layers without measurement delay extension may be configured.

- Section 2.5: Conditions for measurement skipping

  - No conditions are needed for measurement skipping per measurement period or related to MG configuration.

- Section 2.6: Semi-static schemes

  - Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

  - RAN1 made already down-selection between DCI and RRC schemes, hence RAN4 should consider DCI scheme only.

- Section 2.7: UE feature list

  - UE feature for support of UAI is optional with per UE capability.

  - Additional UE capabilities are not precluded.

Types of measurement gaps

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 2-1: Types of measurement gaps In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

Nokia preference can be summarized in the following table

| Gap types | Can be skipped | Cannot be skipped |
| --- | --- | --- |
| Type 2 | Nokia (compromise) |  |
| NCSG | Nokia (compromise) |  |
| Rel 18 concurrent gap |  | Nokia |
| PreConfigured |  | Nokia |

Rel-17 Concurrent measurement gaps

Rel-17 concurrent measurement gaps, as specified in TS 38.133, clause 9.1.8, allow to distribute measurement objects between MG configurations. Each MG configuration is assigned a priority level which is relevant in case of gap collision in time domain. In addition, a concurrent measurement gap can be defined for positioning measurements only via the gapAssociationPRS parameter, in which case it should not be cancelled, as previously agreed for MUSIM and Positioning gaps.

Additionally, when considering multiple concurrent gaps, it is not clear whether the UAI information refers to all the configured gaps or to a particular gap. Therefore, if RAN4 agrees to define requirements for Type 2 gaps, we need discussion how the UAI agreement from RAN4 #114bis extends for multiple gap configurations.

When considering Rel-17 concurrent measurement gaps for measurement gap cancellation as second priority, following conditions apply:

- If concurrent measurement gaps are colliding as specified in clause 9.1.8.3, the dropping rule is applied before measurement gap cancellation.

- A concurrent measurement gap with the gapAssociationPRS parameter set, is not subject to measurement gap cancellation.

- The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG.

Concurrent measurement gaps are subject to measurement gap cancellation and following conditions apply:

- If concurrent measurement gaps are colliding as specified in clause 9.1.8.3, the dropping rule is applied before measurement gap cancellation.

- A concurrent measurement gap with the gapAssociationPRS parameter set, is not subject to measurement gap cancellation.

- The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG.

Rel-17 NCSG

NCSG, as specified in TS 38.133, clause 9.1.9, may be in use for a CA capable UE receiver with idle receive chain and thus significantly reduce the interruption time compared to MG (e.g. from MGL=6ms to 2*VIL=2ms), allowing to schedule XR traffic during ML, and therefore is a good candidate for use in combination with XR services. Therefore, we consider that single NCSG (Rel-17) can be added as second priority for defining RRM requirements for measurement gap cancellation.

The only question that might be unclear regarding NCSG is how VIL1 and VIL2 are skipped. There has been at least in RAN1 discussion whether DCI indication applies for VIL1 and VIL2 individually. However, we understand that one NCSG occasion includes a pair of VIL1 and VIL2 with embedded measurement window ML, since VIL is dedicated for turning on/off or retuning an spare RF chain. For example, if only VIL1 is skipped, the UE cannot turn ON the RF chain and perform the measurement, and VIL2 is anyway not needed. If only VIL2 is skipped, the UE is able to perform the measurement, but it cannot turn OFF the spare RF chain, leaving to increased power consumption.

- Case 1: DCI received before VIL1

- Case 2: DCI received between VIL1 and VIL2

Figure 1 Proposed measurement skipping behaviour when NCSG is configured for (a) Case 1 with DCI containing skipping command received at least Toffset before VIL1, and b) Case 2 with DCI containing skipping command received between VIL1 and VIL2

Figure 1 shows 2 cases for discussion on the specified behaviour for skipping of NCSG. Figure 1 a) Case 1 is shown where the DCI is received at least Toffset before the start of VIL1. In this case, the UE should skip the next NCSG occasion that is composed of VIL1, ML and VIL2 in this order.

In Figure 1 b) Case 2 is shown, where the skip indication is received between VIL1 and VIL2. In this case the skip indication cannot apply for VIL2, otherwise the spare RF chain will have to be ON until the next VIL1 occasion. Therefore, we propose that the DCI with skipping indication received between VIL1 and VIL2 is applicable to the next NCSG occasion with at least Toffset before the start of VIL1.

Measurement gap cancellation applies to NCSG.

  - The skipping indication on a DCI applies for an entire NCSG occasion, where the NCSG occasion is composed of a combination of VIL1, ML and VIL2.

  - The DCI with skipping indication for a NCSG gap occasion must be received by the UE at least Toffset before the start of VIL1.

  - The DCI with skipping indication for a NCSG gap occasion received between VIL1 and VIL2 applies for the next NCSG gap occasion.

Rel-17 Pre-configured measurement gaps

Pre-configured measurement gaps (Pre-MG), as specified in TS 38.133, clause 9.1.7, are used either with UE autonomous activation/deactivation or NW controlled activation/deactivation. In case the active BWP of the serving cell covers the SSB to be measured, the Pre-MG can be deactivated. However, in many cases depicted in clause 9.1.7.3.1 the measurement cannot be performed without measurement gaps, see below (UTRA Inter-RAT measurements are not relevant for XR enhancements).

A measurement cannot be performed by the UE without measurement gaps if any of the following conditions is met: - The UE is configured with SSB based intra-frequency measurements, and the conditions defined for SSB based intra-frequency measurement without gaps in clause 9.2.1 are not met, or - The UE is configured with SSB based inter-frequency measurements, and the conditions defined for SSB based inter-frequency measurement without gaps in clause 9.3.1 are not met, or - The UE is configured with any of the following measurements: - CSI-RS based inter-frequency measurements, or - E-UTRA Inter-RAT measurements, or - UTRA Inter-RAT measurements.

In addition, Pre-MG is also dynamic in nature, triggered by several events as depicted in clause 9.1.7.3.1.

The UE shall also autonomously determine the Pre-MG status based on all the concurrent triggering conditions occurring jointly: - DCI, timer or RRC based active BWP switching, - Activation/deactivation of SCell(s), - Addition/removal of any measurement object(s) - Addition/release/change of a SCell in carrier aggregation,

Thus, enabling measurement gap skipping in combination with Pre-MG rules for activation/deactivation of gaps can be quite complicated. There may be many different cases to be evaluated about activation and how the timing of the activation/deactivation procedure should be considered in combination with DCI command for measurement gap skipping. Thus, we do not see justification to apply measurement gap cancellation to Pre-MG in Rel-19.

Pre-configured measurement gaps are not subject to measurement gap cancellation.

Rel-18 Concurrent measurement gaps

Concurrent measurement gaps with Pre-MG, as specified in clause 9.1.12, and concurrent measurement gaps with NCSG, as specified in clause 9.1.13, will again require many cases to be studied to analyse the RRM impact from measurement gap cancellation. Thus, we do not see justification to apply measurement gap cancellation to them in Rel-19.

Rel-18 concurrent measurement gaps are not subject to measurement gap cancellation.

Formulas for measurement delay extension

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 3-2: Formulas for measurement delay extension <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter

The Rel-18 requirements of measurement period with measurement gaps in FR1 are referred as one example for the analysis of the measurement delay extension. The requirements without DRX on Table 9.3.5-1 are represented by the formula:

- Max(200 ms, Ceil(8 * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter

Where

- 200 ms is the lower bound for the measurement delay calculation

- 8 is the maximum number of samples for inter-frequency measurements

- MGRP is the gap periodicity

- SMTC period is the period for the SMTC window

- CSSF is the carrier specific scaling factor.

- Kgap is used to take into account the overlap of concurrent gaps, that factor will be ignored for now so that we can focus on type 1 gaps.

Considering a simple example of a single carrier measurement (CSSF=1), without concurrent gaps (Kgap=1), and MGRP=SMTC period, we can simplify the formula above as

- Max(200 ms, 8 ´ MGRP)

So if we skip N samples on a measurement cycle, we could extend the formula as:

- Max(200 ms, (8 + N) ´ MGRP)

That would still keep the 200 ms as lower bound, and in some cases the measurement delay is not extended. If we follow that logic, we can extend the measurement delay formula as done in NR-U requirements:

- Max(200 ms, Ceil(8 + Nskip) ´ Max(MGRP , SMTC period)) ´ CSSFinter

One problem of this simple extension arises if we consider multiple frequency layers with CSSFinter > 1. In that case, every skipped sample would result in an extension of CSSFinter ´ Max(MGRP,SMTC) according to the round robin approach. Therefore, we propose one alternative solution that allows for extending the measurement delay just by a single SMTC/MRGP

- Max(200 ms ´ CSSFinter, 8  ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period))

Instead of the round robin approach for applying the CSSF between different frequency layers, this stops the round robin at the skipped MG occasion and continues it at the next available MG occasion. Thus, the overall delay can be remarkably reduced.

Table 1 Comparison of measurement delay calculation for Option 1 and Option 2 in Issue 3-2

|  |  |  | Total delay (ms) |  |  | Delay extension (ms) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC=MRGP | CSSF | Nskip | Legacy (no skipping) | Option 1 | Option 2 | Option 1 | Option 2 |
| 40 | 1 | 1 | 320 | 360 | 360 | 40 | 40 |
| 40 | 1 | 3 | 320 | 440 | 440 | 120 | 120 |
| 40 | 1 | 5 | 320 | 520 | 520 | 200 | 200 |
| 40 | 3 | 1 | 960 | 1000 | 1080 | 40 | 120 |
| 40 | 3 | 3 | 960 | 1080 | 1320 | 120 | 360 |
| 40 | 3 | 5 | 960 | 1160 | 1560 | 200 | 600 |
| 40 | 5 | 1 | 1600 | 1640 | 1800 | 40 | 200 |
| 40 | 5 | 3 | 1600 | 1720 | 2200 | 120 | 600 |
| 40 | 5 | 5 | 1600 | 1800 | 2600 | 200 | 1000 |

Table 1 shows the comparison between the formulas in Option 1 and Option 2 in Issue 3-2. Option 1 is in line with the proposed formula in this document, while option 2 is the plain adaptation of the NR-U formula. As it can be observed in the table, the formulas result in the same measurement delay when CSSF=1, however there is a significant difference int eh measurement delay extension for CSSF=3 and CSSF=5.  When CSSF =3 and Nskip=5, the measurement delay extension is 200 ms for Option 1, whereas it is 600 ms for option 2. When CSSF=5 and Nskip=5 the measurement delay extension is 200 ms for Option 1, whereas it Is 1000 ms for Option 2.

The measurement delay of option 2 is 44,4 % larger than the delay for Option 1 when CSSF=5 and Nskip=5.

RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms ´ CSSFinter, 8   ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period)), where Nskip is the number of  skipped MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

As an alternative solution, we can also consider the formula as adding the skipping term an additional term to the total measurement delay.

Alternative proposal

  - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - Tskip = Nskip ´ Max(MGRP , SMTC period)

    - Nskip is the number of  skipped MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Additionally, the same issue is open for the case with DRX configured [2]:

## Issue 3-3: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter

For the case with DRX we propose extend our previous proposals sto consider DRX. The first alternative proposal is

Alternative 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms ´ CSSFinter, ceil(8 ´ 1,5)  ´ Max(MGRP , SMTC period, DRX cycle) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period, DRX cycle))

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

  - Use this formula as example and follow similar approach for the other cases.

And the 2nd alternative proposal is

Alternative proposal

  - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle

XR skipping when multiple layers are configured

Additionally, we also have to consider the case where multiple frequency layers are measured. When multiple layers are configured, the current RAN4 assumption is that the UE is using a single searcher for measurements with gaps, and a single frequency layer is measured at every given measurement occasion. In some network deployments, the importance of certain frequencies is not equal in relation to others. Therefore, it is desirable that the measurement skipping is not affecting equally the measurement delay for all the frequency layers. This can be achieved by the UE prioritizing higher priority frequency layers to be measured first in case a measurement gap is skipped, while low priority gaps may be measured on remaining gap occasions and hence experience longer measurement delays.

RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

One example is if one frequency layer is used for mobility, while another is more often used for additional carriers in carrier aggregation. Figure 1 shows one example where one carrier is assigned a high priority. In this example the skipping of few SMTCs is compensated by measuring more often for the high priority carrier, while the low priority carrier has its delay extended.

Figure 1 – Example assigning priority to measure one frequency layer, where measurement delay is not extended to that layer, while it is extended to the low priority layer

When that approach is used, we can consider the high priority carriers to have unchanged measurement delay as long as the number of skipped measurement occasions in the measurement cycle is Nskip < 8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip < 5 ´ (CSSF - Nhigh) for intra-frequency measurements.

Examples if Nskip and the resulting measurement skipping ratio are shown on Table 2 for different values of CSSF and high priority layers. In some examples with small number of Nhigh the maximum skipping ratio that can be achieved without impact to high priority layers can get between 50% to 80%.

It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer.

High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

  - Nskip ≤8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5 ´ (CSSF - Nhigh) for intra-frequency measurements

  - where Nhigh is the number of high priority layers.

Table 2 Calculation of maximum skipping ratio without high priority delay extension

| Number of inter-freq samples (per frequency layer) | CSSF | Nhigh | Maximum Nskip | Maximum skipping ratio % |
| --- | --- | --- | --- | --- |
| 8 | 2 | 1 | 8 | 50 |
| 8 | 3 | 1 | 16 | 66,7 |
| 8 | 3 | 2 | 8 | 33,3 |
| 8 | 4 | 1 | 24 | 75 |
| 8 | 4 | 2 | 16 | 50 |
| 8 | 4 | 3 | 8 | 25 |
| 8 | 5 | 1 | 32 | 80 |
| 8 | 5 | 2 | 24 | 60 |
| 8 | 5 | 3 | 16 | 40 |
| 8 | 5 | 4 | 8 | 20 |

Conditions for measurement skipping

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 3-4: Conditions for measurement skipping <Way forward>: - Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

In this issue few companies proposed that a limitation condition where measurement skipping may be used. These included maximum number of occasions to be skipped or measurement gap configurations.

Regarding Option 1 of issue 3-3, we understand that some companies have concerns on potential impacts if measurement skipping exceeds a certain limit. However, it is not very clear how to define a limit to measurement gap skipping that is general enough to account for all possible scenarios.

In our companion paper we provided analysis of the need for measurement skipping and mobility impacts [7]. The results in our paper show that it can be beneficial in some situations to increase the measurement gap skipping to about 60%. Additionally, at least in one scenario we could observe that 75% skipping could be used without significant performance impact.

Our simulation results show the benefit of using about 60% skipping ratio.

In at least some scenarios skipping ratio can be increased to about 75% without significant mobility degradation.

In other scenarios the UE will be performing mobility mostly on intra-frequency without gaps. In that case, if gaps are used for inter-frequency there should be no mobility performance impact of skipping inter frequency measurements.

Additionally, in other cases the network may configure gaps for the UE to report measurements on potential candidates for CA. In that case, the extension of the measurement delay will not have any damaging impact on mobility performance.

Measurement gaps can be configured by the network to receive reports on potential carrier aggregation candidates.

Therefore, we think that it is natural that in the network deployment there is a limitation on the maximum number of skipping. However, this is an optimization that can be determined case by case, for example considering the typical mobility of XR UEs in a given location, the ISD, or layers to monitor. If we define a limit in the specification, we will simply limit available options on the deployment and the benefit of the feature will be impacted.

Maximum number of skipping depends on the deployment, e.g. number of layers, measurement gap configuration, deployment inter-site distance.

When considering Option 3, it is not clear what is the relationship between the measurement gap length and the impact to measurement skipping. We understand that the measurement skipping will have a direct impact on measurement delay requirements, however those requirements do not depend on the measurement gap length.

No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration.

Semi-static schemes

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

During RAN1#120, the following was agreed:

Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI..

That means, RAN1 already confirmed the working assumption regarding Alt. 1-1, and there is no need for specifying duplicated solutions for XR.

RAN1 already confirmed working assumption regarding Alt 1-1.

In our view, measurement gap skipping should be fully under network control. The network has all information available on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. Hence Alt 1-1 selected by RAN1 based on DCI scheduling is appropriate for this.

The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Additionally, many companies had concerns regarding Alt 1-1 because of the potential short time for reaction between DCI and measurement gap to be skipped. We already addressed in the last RAN4 meeting when we agreed with a time offset for measurement skipping of 5 ms as baseline.

RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline.

Overall, the performance of semi-static schemes for XR is judged inferior versus the DCI based scheme, as shown in our simulation paper  in RAN4 #113 R4-2418890.As shown in Figure 4 and 5 of R4-2418890, the capacity gain of the semi static scheme (Alt-3) in relation to measurements with gaps is very minor in comparison to the dynamic (DCI-based) scheme. This is also reasoned by the fact that Alt-3 is highly depending on prediction (of configured gaps subject to be skipped) and hence generates useless skipping, whilst XR traffic is highly dynamic, has jitter and hence cannot be matched with a predicted traffic pattern.

Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Another problem related to Alt-3 is that it relies on good traffic pattern prediction before the network has enough information to configure the skipping pattern. Figure 4 shows one example of what happens during the call start of a XR traffic stream. If Alt 3 approach is used as in Figure 4 a), the network needs a training phase in order to determine the XR traffic pattern. Once the prediction is ready at time t1, the network can determine the skipping pattern to be used that matches the XR traffic, and send an RRC command configuring the UE to use that pattern at t2. Therefore, before t2, the UE would experience worst performance related to the XR traffic. On the other hand, Figure 4 b) shows the example of using dynamic approach, as decided by RAN1. When using this approach, the traffic prediction training is not a mandatory step, and the network can start skipping measurement gaps from the start of the XR traffic in t0.

Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

a)

b)

Figure 2 Comparison of XR traffic quality variation over time when using a) semi-static approach (Alt 3) and b) dynamic approach as the decided by RAN1 working assumption (Alt 1-1)

Next, when using semi-static schemes, the network cannot quickly react to changes in channel condition. Consider the example where a UE is configured with a semi-static pattern with 50% skipping and approaches the cell edge. In that situation, once the network notices that the UE is moving toward cell edge, it would need to send RRC reconfiguration to change the pattern. Only after that the UE could perform measurements with short delay. In that case the Alt 1-1 can in a very dynamic way adapt the skipping ratio and prioritize mobility measurements.

Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

Furthermore, RAN1 has already made a down-selection between DCI and RRC schemes

RAN1 already made down selection between DCI and RRC schemes

Follow RAN1 agreement and consider DCI scheme only.

Discussion on UE feature list

RAN1 already discussed the UE capability for measurement gap skipping [11]. The UE feature group includes the optional time offset of 3 ms with 5 ms as baseline.

For the UAI reporting for measurement gap skipping, RAN4 indicated that UAI reporting is optional [13], and RAN2 is discussing whether the UE capability for reporting ratio of gap occasions should be specified by RAN4 or RAN2.

Given the status of the discussions on RAN1, in our view we cant identify the need for additional UE capabilities.

RAN1 already defined UE feature for XR measurement gap skipping.

RAN2 started discussion on UE feature for UAI reporting based on RAN4 LS.

RAN4 to discuss whether RAN4 should define the UE capability for UAI.

Define UE feature for support of UAI as optional with per UE capability.

RAN4 to discuss the need for additional UE feature for XR on top of RAN1 and RAN2 discussed features.

Conclusion

The following Observations and Proposals were made:

- Concurrent measurement gaps are subject to measurement gap cancellation and following conditions apply:

- If concurrent measurement gaps are colliding as specified in clause 9.1.8.3, the dropping rule is applied before measurement gap cancellation.

- A concurrent measurement gap with the gapAssociationPRS parameter set, is not subject to measurement gap cancellation.

- The UE may provide the UAI related information on how many MG occasions may be skipped for each configured MG.

Measurement gap cancellation applies to NCSG.

  - The skipping indication on a DCI applies for an entire NCSG occasion, where the NCSG occasion is composed of a combination of VIL1, ML and VIL2.

  - The DCI with skipping indication for a NCSG gap occasion must be received by the UE at least Toffset before the start of VIL1.

  - The DCI with skipping indication for a NCSG gap occasion received between VIL1 and VIL2 applies for the next NCSG gap occasion.

Pre-configured measurement gaps are not subject to measurement gap cancellation.

Rel-18 concurrent measurement gaps are not subject to measurement gap cancellation.

- The measurement delay of option 2 is 44,4 % larger than the delay for Option 1 when CSSF=5 and Nskip=5.

RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms ´ CSSFinter, 8   ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period)), where Nskip is the number of  skipped MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Alternative proposal

  - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - Tskip = Nskip ´ Max(MGRP , SMTC period)

    - Nskip is the number of  skipped MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Alternative 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms ´ CSSFinter, ceil(8 ´ 1,5)  ´ Max(MGRP , SMTC period, DRX cycle) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period, DRX cycle))

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

  - Use this formula as example and follow similar approach for the other cases.

Alternative proposal

  - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle

RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer.

High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

  - Nskip ≤8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5 ´ (CSSF - Nhigh) for intra-frequency measurements

  - where Nhigh is the number of high priority layers.

Our simulation results show the benefit of using about 60% skipping ratio.

In at least some scenarios skipping ratio can be increased to about 75% without significant mobility degradation.

Measurement gaps can be configured by the network to receive reports on potential carrier aggregation candidates.

Maximum number of skipping depends on the deployment, e.g. number of layers, measurement gap configuration, deployment inter-site distance.

No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration.

RAN1 already confirmed working assumption regarding Alt 1-1.

The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline.

Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

RAN1 already made down selection between DCI and RRC schemes

Follow RAN1 agreement and consider DCI scheme only.

RAN1 already defined UE feature for XR measurement gap skipping.

RAN2 started discussion on UE feature for UAI reporting based on RAN4 LS.

RAN4 to discuss whether RAN4 should define the UE capability for UAI.

Define UE feature for support of UAI as optional with per UE capability.

RAN4 to discuss the need for additional UE feature for XR on top of RAN1 and RAN2 discussed features.

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2504911, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2504972, Reply LS on UE assistance information, Nokia

- R4-2502700, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2500542, Topic summary for [113][228] NR_XR_Ph3, Moderator (Nokia)

- R4-2502241, General aspects for XR enhancements, Nokia

- R4-2502229, RRM requirements for XR enhancements, Qualcomm Incorporated

- R4-2502150, Discussion on XR ph3 RRM requirements, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, Nokia

- R4-2504508, Discussion on XR ph3 RRM requirements, Nokia

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, AT&T, NTT DOCOMO, INC.

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, To: RAN1, cc: RAN2, Source: Qualcomm

- R4-2504972, Reply LS on UE assistance information, Nokia