3GPP TSG-RAN WG4 Meeting #115	R4-2507749

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		7.10.4.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for NTN less than 5 MHz

Document for:		Approval

## 1	Introduction

3 MHz channel bandwidth was specified for TN in rel-18. In the work item approved for NTN, 3 MHz channel bandwidth is to be added with RAN1-design based on rel-18, with no expected RAN1 impact. In this contribution UE RF requirements for 3 MHz channel bandwidth are discussed.

## 2	Discussion

A-MPR

In previous meeting nearly all A-MPR values were agreed, NS_24 being still open. On face value the proposed tentative agreements looks acceptable. However, -50 dBm/MHz requirement at 5 MHz frequency offset is very stringent, and more comprehensive evaluation needs to be done.

Typically for emissions, PA non-linearity dominates the interference, and additionally there are some significant components in the spectrum like CIM3 and CIM5. For 3 MHz those distortion components typically fall close, and do not cause A-MPR for 3 MHz CBW. However, this analysis method typically used in RAN4 ignores the impact of wideband noise sources:

- Transceiver noise floor which gets amplified in the PA

- Phase noise

- PA noise floor

In most emission scenarios these are not dominant, but for NS_24 there may be impact.

Observation 1: NS_24 requirement warrants a look into wideband noise floor and phase noise.

At the time of Tdoc writing all data for these was not yet available. Therefore, we suggest to keep the NS_24 A-MPR as tentative until the composite impact of wideband noise and PA non-linearity has been checked.

Proposal 1: Keep NS_24 A-MPR tentative until composite impact of wideband noise floor, phase noise and PA non-linearity has been evaluated.

Relationship with NTN HPUE for NR NTN

A separate objective of the same work item is to define HPUE requirements for NR NTN. However, if PC3 requirements for 3 MHz get introduced and at the same time PC2 requirements for other channel bandwidths are introduced, RAN4 specifications get broken. PC2 UE cannot support 3 MHz CBW at all as PC2 requirements for 3 MHz, namely A-MPR, is not defined.

Observation 1: Simultaneous introduction of PC3 for 3 MHz and PC2 for other channel bandwidth for same bandwidths means that 3 MHz means that PC2 UE cannot support 3 MHz CBW.

To avoid this situation, RAN4 should work on 3 MHz PC2 requirements to ensure that both features can be supported together.

Proposal 2: Work on 3 MHz PC2 requirements either in this track or in NTN HPUE track to ensure that both features can be supported together.

Release independence

In previous meeting an LS was sent to RAN2 stating that 3 MHz CBW can be release independent from rel-18. Therefore, an update to release independence specification is needed. Release independence was not included in CR split, but we have provided a draft CR to this meeting with following update:

#########

Table 9.1-3: NR NTN UE channel bandwidth in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth, all unless otherwise stated | FDD | Rel-17 | Table F.1.1-1 |
| Channel bandwidth of 3 MHz | FDD | Rel-18 | Table F.1.1-1 |

#########

Proposal 3: Update release independence specification to include 3 MHz CBW support from rel-18 for NR NTN in FR1-NTN.

## 3	Conclusion

In this contribution UE RF requirements for less than 5 MHz for NTN were discussed. Following observations and proposals were made.

Observation 1: NS_24 requirement warrants a look into wideband noise floor and phase noise.

Proposal 1: Keep NS_24 A-MPR tentative until composite impact of wideband noise floor, phase noise and PA non-linearity has been evaluated.

Observation 1: Simultaneous introduction of PC3 for 3 MHz and PC2 for other channel bandwidth for same bandwidths means that 3 MHz means that PC2 UE cannot support 3 MHz CBW.

Proposal 2: Work on 3 MHz PC2 requirements either in this track or in NTN HPUE track to ensure that both features can be supported together.

Proposal 3: Update release independence specification to include 3 MHz CBW support from rel-18 for NR NTN in FR1-NTN.

## 4	References

[1]  R4-2502282, Way Forward for [114][311] NR_IoT_NTN_less_than_5MHz_UERF , Xiaomi