3GPP TSG-RAN WG4 Meeting #115	R4-2507751

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		6.14.1

Source: 		Qualcomm Inc.

Title:	 	B252 NS-signaling

Document for:		Approval

## 1	Introduction

UE RF requirements for n252 (UL 2000-2020, DL 2180-2200) has been discussed for a few meetings, with main focus on UE-to-UE coexistence requirements towards bands n2 and n25. The requirement limits were finally concluded in RAN4#114. In RAN4#114-bis it was agreed to specify at least one NS-value. This contribution discussed the NS-signaling framework.

## 2	Discussion

In RAN4#114bis an agreement on NS-signaling was reached and captured in WF [1].

While the requirements do not need A-MPR for PC3, it is possible that A-MPR is needed for higher power class later on, which is highly likely to be specified. This causes a potential backwards compatibility issue: rel-19 UE is not aware of any NS for this operating band, but a new NS is introduced for PC2. If this NS is signaled, rel-19 UE may not be able to access a cell.

RAN2 specifications have means to go around this issue:

- UE needs to support multi-NS capability, which is optional

- Networks needs to signal both NS-values in a list

If there is any UE which does not support the multi-NS capability, then only NS-value provided SIB2 matters. As such a risk for backwards compatibility remains, and can only be solved by both network and UE taking the correct steps.

Observation 1: There is a risk that some UEs may be barred access to a cell if a new NS is introduced in later release for higher power class.

Therefore, it is a safer option to specify new NS already now. As the A-MPR needed is likely different for -30 and -40 dBm/MHz limits, two NS-values are preferred.

Proposal 1: Specify two NS-values: one for -40 dBm/MHz and another for -30 dBm/MHz. No A-MPR is allowed for PC3.

This arrangement ensures that rel-19 UEs are aware of the NS, and do not unintentionally get barred from accessing the cells in band 252.

## 3	Conclusion

In this contribution UE RF requirements for B252 were discussed. Following observations and proposals were made:

Observation 1: There is a risk that some UEs may be barred access to a cell if a new NS is introduced in later release for higher power class.

Proposal 1: Specify two NS-values: one for -40 dBm/MHz and another for -30 dBm/MHz. No A-MPR is allowed for PC3.

This arrangement ensures that rel-19 UEs are aware of the NS-values, and do not unintentionally get barred from accessing the cells in band 252.

## 4	References