3GPP TSG-RAN WG4 Meeting #115							         		      R4-2507791

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	6.3.1 Rapporteur input (WID/TR/big CR) [NR_CADC_SUL_R19-Core]

Title: 	PRD input on IMD range definition for inter-band MSD due to 1UL 2CC UL configuration

Source: 	Skyworks Solutions, Inc.

Document for:	Information

# 1	Background

In R4#114 we proposed optional MSD test points for inter-band combinations with CA_n41C UL in [1]. A way forward [2] captured these MSD test points, accounting for other companies’ contributions. Additionally, the way forward encouraged companies to provide clarification on the source of IMD for such MSD test points. In this contribution, we provide our definition and suggest that such definition is captured in a TP for the RAN4 band-combination PRD in August.

# Discussion

## 2.1 Way forward input

The way forwards [2] captured a need for clarification on the source of IMD reproduced here italics:

Background:

First, we can observe in the agreed test points that the source of IMD should be the same for the mandatory 1RB+1RB test point and the optional Full+Full test point. It is based on the calculations of IMD collision table for UL contiguous intra-band CA table in the 2DL and 1UL with 2CC TP to TR template.

One of the possible clarification is that the IMD order captured in the table is the lowest IMD order of the two UL CCs that dominates the interfering power in the victim channel.

As an example, a fully allocated single CC/two contiguous CCs ACLR1 power is dominated by IMD3 while all the odd higher order IMDs overlap with ACLR1.

Way Forward:

Companies are encouraged to provide a clarification of the source of IMD for the test point where there is contiguous fully allocated RB in each CC based on the IMD order dominating the interference power.

Companies are encouraged to provide their view on which document(s) should capture such clarification:

- In the PRD as a rule and/or explanation to the TP to TR process.

- In the related band combination TP to TR as an addtional information to the table calculating the IMD collisions for the mandatory test point.

- In the section 7.3A.5 of TS 38.101-1 as a note or general text.

## 2.2 Proposed Definition to be introduced in PRD

As discussed in the background part of the way forward, the source of IMD is basically the lowest IMD range overlapping with the other band DL based on the specific table calculating IMD collisions for UL contiguous intra-band CA table in the 2DL and 1UL with 2CC TP to TR template. This calculation does not make any assumption on using 1RB+1RB or Full+Full allocation as it calculates the IMD ranges of the two CC bandwidth.

It covers multiple IMD orders with all possible indexes, but only retains IMDs that are not already covered by other 1UL MSD test cases like harmonics and are sufficiently powerful to grant an MSD investigation. Also, it should not be confused with the ACLR range, which is the source of MSD for cross-band, as some of the calculated IMD ranges of the two CCs fall at baseband or near harmonics of the two CCs which cross-band does not account for. Even close to the UL band, when using fully allocated CCs, the IMDs product present within the ACLR1 range as defined by the aggregated CA BW, is the overlap of many odd order IMDs of the allocation in the two CCs. They are just more spread in frequency and even with their lower PSD, the resulting MSD is not very different from the 1RB+1RB case and is related to the same non linearities.

One of the sources of confusion, is the term “source of IMD” used in the MSD test point tables itself. As in any MSD table, the source that is captured is the range of IMD, harmonic or ACLR that overlaps with the DL victim channel, not the source that could be the PA non-linearities for example. And when there are multiple UL interference ranges that overlap with the DL channel, only the lowest even and odd order is retained in the table as “source of IMD”.

For information: We propose to capture the following definition for “source of IMD” in the RAN4 PRD in May:

- For the case of a single band UL with two contiguous CCs as the aggressor, the “source of IMD” is the IMD order of the lowest IMD order and index overlapping with the victim DL channel as in specific table calculating IMD collisions for UL contiguous intra-band CA table in the 2DL and 1UL with 2CC TP to TR template with reference to that table that should also be part of the PRD.

  - This source of IMD should be same for the mandatory 1RB+1RB mandatory test point and the optional Full+Full test points.

  - The agreed parameters for the design of both test points should also be captured in the PRD.

- We are open for better wording and whether equations should be provided (the ones on which the collision table is based).

- FFS if “source of IMD” should also be defined for other types of UL configurations like 2band UL with 2CC and 3CC (IMDs and triple beat cases).

# Conclusion

In this contribution, we share our view on the definition of the “source of IMD” for 1UL 2CC contiguous configuration and inform about our intention for a TP to PRD in the August meeting.

For information: We propose to capture the following definition for “source of IMD” in the RAN4 PRD in August:

- For the case of a single band UL with two contiguous CCs as the aggressor, the “source of IMD” is the IMD order of the lowest IMD order and index overlapping with the victim DL channel as in the specific table calculating IMD collisions for UL contiguous intra-band CA table in the 2DL and 1UL with 2CC TP to TR template with reference to that table that should also be part of the PRD.

  - This source of IMD should be same for the mandatory 1RB+1RB mandatory test point and the optional Full+Full test points.

  - The agreed parameters for the design of both test points should also be captured in the PRD.

- We are open for better wording and whether equations should be provided (the ones on which the collision table is based).

- FFS if “source of IMD” should also be defined for other types of UL configurations like 2band UL with 2CC and 3CC (IMDs and triple beat cases).

# References

[1] R4-2500461 Additional test points for CA_n40A-n41C and CA_n41C-n79A with n41C UL, Skyworks Solutions Inc., R4#114

[2] R4-2502955 WF on ULCA_n41C MSD test point and IMD range definition, Skyworks Solutions Inc. Qualcomm, Huawei, R4#114