3GPP TSG-RAN WG4 Meeting #115	R4-2507812

Malta, MT, May 19 – 23, 2025

Agenda Item:	7.28.5.1

Source:	MediaTek Inc.

Title:	Simulation assumptions for R19 LP-WUS

Document for:	Discussion

# Introduction

In the last meeting, RAN4 discussed the simulation for LP-WUS/WUR in FR1 and the progress is captured in the WF [1]. In this contribution, we further discuss the remaining aspects related to the simulation assumption/results for RRM performance evaluation for LP-WUS/WUR UE.

# Simulation assumptions and results

Regarding the target SNR for LP-WUR RRM measurements, it was agreed (in RAN4#114) to include -6dB as side condition requirement in addition to the other agreed values (-3dB, 0.5dB, 2dB dB), which align with RAN1 conclusion. The agreement in RAN4#114 is captured below [2]:

Issue 2-1-1: SINR setting Agreement: Include -6 dB SINR in simulation assumptions SINR and -6 dB SINR is low priority and depending on company input; Treat -0.5dB and 2dB SINR as low priority cases;

Therefore, considering the worst-case scenario, we provide our simulation results based on two values of SNR (-3dB and -6dB) for RRM measurement accuracy as shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |  |  |
| M=1, L=6 | [2] | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | [2] | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | [2] | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | [2] | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | [2] | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Set 2 from [3] | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

The simulation results are demonstrated in terms of CDF percentage for the measured RSRP accuracy for LP-SS, including 5%, 50%, 95% as well as the Absolute accuracy. Note that Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point (based on the agreement in Issue 2-2-1 from the WF [2]). The simulation results above demonstrate the RSRP error using different number of samples in TDL-C channel (3 and 5 samples). Regarding the LP-SS binary sequence, we use the corresponding LPSS sequences agreed in [2]. However, for M=4, L=32, we also provide the results based on the set 2 LP-SS sequence discussed in RAN1 [3] as captured below:

Agreement For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: - Set 1: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] - Set 2 [0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0] [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0] [0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0] [0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0]

Proposal 1: For LP-SS binary sequence, consider the following candidates:

- For M=1, L=6:

  - Serving cell: 		[1 0 1 0 1 0]

  - Interference cell: [0 1 0 1 0 1]

- For M=2, L=8:

  - Serving cell: 		[0 1 0 1 1 0 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1]

- For M=2, L=16:

  - Serving cell: 		[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

  - Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

- For M=4, L=16:

  - Serving cell:		[0110100110101010]

  - Interference cell: [0110101010011010]

- For M=4, L=32 (set 1):

  - Serving cell:		[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

- For M=4, L=32 (set 2):

  - Serving cell:		[0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0]

  - Interference cell: [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0]

Proposal 2: RSRP error (dB) in TDL-C channel for OOK shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK (using I&Q branches)

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |  |  |
| M=1, L=6 | [2] | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | [2] | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | [2] | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | [2] | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | [2] | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Set 2 from [3] | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

It can be noticed that even with 3 sample measurement LP-WUS capable UE can achieve satisfactory results for the given simulation assumption in Table 2, 3 and 4. However, to select number of samples for RRM performance evaluation for LP-WUS capable UE, we propose to use at least 3 or 5 samples to guarantee achieving the required performance.

Proposal 3: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

Furthermore, the overall simulation assumption list for RRM performance evaluation for LP-WUS/WUR UE is provided below including the general parameters, cell specific parameters and UE specific parameters.

Table 2: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) | 3/5 |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 3: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = 1,2,4 | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 |
| LP-SS binary sequence |  | Indicated in accuracy table | Indicated in accuracy table |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OOK based receiver 10 ppm | N/A |
| Timing error |  | T= 5us for OOK-4 with M=1 T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
| Ês/Iot | dB | -3dB / -6dB | N/A |
| Propagation conditions | - | FR1: TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 4: UE-specific parameters

| [Receiver Filter] | 3rd Order Butterworth with 3.84MHz bandwidth |
| --- | --- |
| [Receiver ADC bit width] | 4-bit ADC |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz |

Another issue was discussed in the last meeting was about the number of branches assumed for the OOK-based LP-WUR to calculate the ideal RSRP and the measured RSRP. In the last meeting, we noticed that some companies provided their results based on both (I&Q) branches while others used single branch (I). The simulation results, therefore, cannot be comparable for two different assumptions. RAN4 need to align their assumption for the number of branches used by the OOK receiver for the collected simulation results. In our view, both (I&Q) branches are needed to derive the ideal RSRP and the measured RSRP. The reason is that the transmitted LPSS binary sequency will experience a complex channel anyway, and therefore, both branches (I&Q) are required to detect the transmitted signal correctly.

Proposal 4: RAN4 needs to align the assumption for the number of branches (I/Q) used by the OOK receiver for the provided simulation results.

Proposal 5: The ideal RSRP and the measured RSRP to be derived based on the assumption that the OOK-based LP-WUR is equipped with both I and Q branches.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: For LP-SS binary sequence, consider the following candidates:

- For M=1, L=6:

  - Serving cell: 		[1 0 1 0 1 0]

  - Interference cell: [0 1 0 1 0 1]

- For M=2, L=8:

  - Serving cell: 		[0 1 0 1 1 0 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1]

- For M=2, L=16:

  - Serving cell: 		[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

  - Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

- For M=4, L=16:

  - Serving cell:		[0110100110101010]

  - Interference cell: [0110101010011010]

- For M=4, L=32 (set 1):

  - Serving cell:		[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

- For M=4, L=32 (set 2):

  - Serving cell:		[0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0]

  - Interference cell: [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0]

Proposal 2: RSRP error (dB) in TDL-C channel for OOK shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK (using I&Q branches)

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |  |  |
| M=1, L=6 | [2] | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | [2] | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | [2] | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | [2] | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | [2] | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Set 2 from [3] | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

Proposal 3: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

Proposal 4: RAN4 needs to align the assumption for the number of branches (I/Q) used by the OOK receiver for the provided simulation results.

Proposal 5: The ideal RSRP and the measured RSRP to be derived based on the assumption that the OOK-based LP-WUR is equipped with both I and Q branches.

# References

- R4-2504908, WF on RRM requirements for NR_LPWUS, vivo.

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo.

- R1-2501583, Summary #5 of discussions on LP-WUS and LP-SS design, vivo