3GPP TSG-RAN WG4 Meeting #115	R4-2507815

Malta, MT, May 19 – 23, 2025

Agenda Item:	7.18.3.2

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on remaining issues for the first objective, particularly on optimizing CSSF outside gap in CA/DC scenarios for L3 measurement delay reduction.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 further discussed on whether to expand solution#1 to FR1 as shown below from WF [2]:

Issue 1-2-2: Expand solution 1 to FR1 case [Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

From the above options, if RAN4 decide to expand solution#1 to FR1, we support option3, i.e., to limit the applicability of solution#1 to FR1 intra-band contiguous. Otherwise, we support not to expand solution#1 to FR1 band.

Proposal 1: Either not to support expanding solution#1 to FR1 or limit the applicability of solution#1 to FR1 intra-band contiguous.

On the other hand, the remaining issue for solution#3 is captured below:

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC) [Way forward]: - FFS: - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF. - Note: 3-searchers will be used by UE if UE report this capability. - FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

In the last meeting, RAN4 discussed the restriction of solution#3 (i.e., 3 searchers) when UE is configured with per-FR MG. In our view, option 3 should be supported when per-FR measurement gaps and 3-searceher capability are supported simultaneously. The legacy requirements (of 2 searchers) should apply when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR MG in another FR (even if UE support 3 searchers). Otherwise, the new requirements of 3 searchers can be applied when such overlap does not happen in other SMTC occasions outside gap.

Proposal 2: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Either not to support expanding solution#1 to FR1 or limit the applicability of solution#1 to FR1 intra-band contiguous.

Proposal 2: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.