# 3GPP TSG-RAN WG4 Meeting #115	R4-2507827

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 		Discussion on open issues related to the implementation of framework simplification for co-location/co-existence requirements

Agenda Item:	10.1

Document for:	Discussion

# Introduction

During RAN4#114bis, the following WF was agreed in [1] identifying multiple open issues to resolve:

We decide to choose Option 1 (i.e., maintain tables for co-existence and co-location in each specification but simplify them). The following aspects need be considered when drafting the CRs: - All notes deleted from the legacy tables are properly reflected in new table format. - For conformance testing specifications, special care needs to be taken to correctly reflect frequency-dependent MU/TT values. - FFS: how to reassure that unnecessary co-location requirements are NOT unintentionally added with this new framework, e.g., IMT BS and FRMCS BS, IMT BS and broadcasting BS, V2X, Unlicensed bands, etc. - FFS: how to reassure that unnecessary co-existence requirements are NOT unintentionally added with this new framework. - FFS: how to consider potential BS-class specific co-existence limits – refer to the BS Evo objective (TFES request).

In this contribution, we provide further discussion on open issues related to the implementation of framework simplification for co-location/co-existence requirements.

# Discussion

While RAN4 decided to proceed with the exercise of implementation of simplified tables for co-location and co-existence requirements, we shall first have decision on the open issues from the last WF. Otherwise, it would not be possible to keep consistency among inputs from different companies.

Secondly, the high-level goal is to keep the same requirement scope, i.e., even if the tables are simplified, the end results must be the same. For now, this is far from clear. While the idea of a simplified co-ex and co-location table is very tempting, there are still unresolved issues originating from the fact, that we are planning to delete significant amount of technical information, which is currently NOT properly reflected in the simplified tables.

We do see a risk that those issues are not resolve properly due to time constraints. If this would be the case, RAN4 shall not proceed with such imperfect solution, i.e., we shall not modify scope of the requirement just to “keep table simple”. An alternative which would resolve the time constraint issues, and which would also solve the initial objective of CR number reduction is the previously discussed solution of a new spec capturing all co-ex and co-location requirements (1-to-1 requirement tables reuse from legacy specs, and then further optimized without time pressure based on the simplified table approach).

Proposal 1: Before proceeding with the implementation of (draft)CRs, the following open issues are to be resolved:

- How to reassure that all notes deleted from the legacy tables are properly reflected in new table format.

- How to reassure that unnecessary co-location requirements are NOT unintentionally added with this new framework, e.g., IMT BS and FRMCS BS, IMT BS and broadcasting BS, V2X, Unlicensed bands, etc.

- How to reassure that unnecessary co-existence requirements are NOT unintentionally added with this new framework.

Proposal 2: RAN4 to re-consider the idea of a new spec capturing all co-ex and co-location requirements (1-to-1 requirement tables reuse from legacy specs, and then further optimized without time pressure based on the simplified table approach).

Referring to RP-243288, the following was agreed in RAN#106:

On Proposal 2 above: at that time, it was not clear if RAN4 would ask for a new specification for co-ex/co-location requirements handling, or if RAN4 would try to optimize content of the already existing specs. With limited progress right now, RAN4 shall inform RAN on the status and potential next steps. Here it shall be also clarified, that the above Proposal 2 does NOT mean that RAN is expecting final CRs by RAN#108. RAN#108 was a checkpoint set to allow assignment of a potential new spec to be created for Rel-19 timeframe.

Proposal 3: RAN4 shall inform RAN on the status and potential next steps via LS.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Before proceeding with the implementation of (draft)CRs, the following open issues are to be resolved:

- How to reassure that all notes deleted from the legacy tables are properly reflected in new table format.

- How to reassure that unnecessary co-location requirements are NOT unintentionally added with this new framework, e.g., IMT BS and FRMCS BS, IMT BS and broadcasting BS, V2X, Unlicensed bands, etc.

- How to reassure that unnecessary co-existence requirements are NOT unintentionally added with this new framework.

Proposal 2: RAN4 to re-consider the idea of a new spec capturing all co-ex and co-location requirements (1-to-1 requirement tables reuse from legacy specs, and then further optimized without time pressure based on the simplified table approach).

Proposal 3: RAN4 shall inform RAN on the status and potential next steps via LS.

# References

[1] 	R4-2504741, Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex, RAN4#114bis