3GPP TSG-RAN WG4 Meeting #115	R4-2507879

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.25.4.1

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# Introduction

For release 19 work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD), the main objective is to specify the sub-band full duplex operation. In addition to that, enhancements to CLI handling scheme is another WID [1] objective that is expected to have impact on RRM requirements. RAN4 has already reached quite a few agreements [2,3,4,5,6] in the last meetings. In this paper, we discuss few remaining issues related to L1-CLI requirements.

# L1 based UE-to-UE CLI measurement requirements

As we know, RAN1 has agreed to define L1-SRS-RSRP and L1-CLI-RSSI as two new measurement report quantities and has finalized upon method #1, #2 and#3 to be used to perform CLI measurements.

In the last few meetings, RAN4 made significant progress towards defining RRM requirements for the above three methods, including the measurement delays, scheduling restrictions, measurement restrictions etc.

Measurement requirements

- Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

- When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options

  - Option 1: Prioritize L1-SRS-RSRP measurement

  - Option 2: Prioritize L1-CLI-RSSI measurement

  - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

One of the issues discussed during the last meeting was related to the measurement restrictions and RAN4 made the following agreements:

While RAN1 has agreed that Method#1 and Method#3 cannot be configured together, it is still possible that a UE may be configured with Method#1 and Method#2 or with Method#2 and Method#3. In these cases, the measurement resources may collide with each other and that is the issue RAN4 is trying to address. We think prioritizing one over the other measurement, like in option 1 and 2, may not always work as there are different factors which may affect such prioritization such as UL-DL timing switching, subband switching etc. and are heavily dependent on UE implementation. One way could be that the network determines which measurements should be performed but that too adds a lot of complexity. So, it’s not straightforward to decide which measurement should be prioritized over the other. This leaves us with Option 3, that it is upto UE implementation which measurements to be performed but this is also not efficient as it may lead to wastage of resources for the interfering UE which needs to transmit SRS for L1-SRS-RSRP measurements which may not even be measured by the victim UE. Furthermore, this also leads to ambiguity on the network side as it’s not aware which measurements the UE performed. So, the best way to handle this is to impose a restriction on the resource scheduling itself, i.e., network should not schedule measurement resources on the same OFDM symbols. This would avoid wastage of resources as well as ambiguity on the network side.

Proposal 1:  UE does not expect to be configured with L1-CLI-RSSI measurements resources in DL subband(s) (Method#1) and L1-SRS-RSRP (Method#2) measurement resources on the same symbol.

Proposal 2: UE does not expect to be configured with L1-CLI-RSSI measurements resources in UL subband(s) (Method#3) and L1-SRS-RSRP (Method#2) measurement resources on the same symbol.

Moving on, we now have R19 L1 CLI measurements in addition to R16 L3 CLI measurements. To reduce the CLI measurement burden on the UE side, we think network should refrain from configuring both L3 and L1 CLI measurements simultaneously.

Proposal 3: UE does not expect to be simultaneously configured with both L3 and L1 UE-to-UE CLI measurement resources, if UE supports both features.

Scheduling Restrictions

Now, talking about the simultaneous configuration of DL data and RSSI measurement resources using Method#1, we want to draw RAN4’s attention towards the objective of Method#1, which is to measure the leakage in DL subband and this is different from the objectives of R16 CLI-RSSI measurements to support dynamic TDD. In this case, because of filtering the UL subband, the leakage may be substantially small as compared to the DL data and an AGC tuned to accommodate data may lead to poor RSSI measurements. So, we think the basic feature should be not schedule any data on the RSSI symbols – basically specify DL scheduling restrictions on the L1-CLI-RSSI measurement symbols.

Proposal 4: UE does not expect to be scheduled/configured with DL data channel/RS and L1-CLI-RSSI measurements resources in DL subband(s) (Method#1 for leakage measurement) on the same symbol.

- For all Method #1, Method #2 and Method #3,

  - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

During the last meeting some companies wanted to check if we need DL scheduling restrictions on symbols after the measurement resources configured for Method#2 and Method#3 to accommodate UE processing time to switch signal reception in the UL subband to DL subband. We would like to note the following assumptions agreed in the RAN4 during the last meeting:

With the above assumption, we don’t think we need additional scheduling restrictions for the UE to switch from receiving in the UL subband to DL subband(s) at-least for a UE supporting simultaneous PDSCH reception on the two DL subbnads in DUD configuration.

Proposal 5: For a UE supporting simultaneous PDSCH reception on the two DL subbnads in DUD configuration - No need to specify scheduling restrictions on the symbol(s) after RSSI measurement resource(s) in Method#3 or after SRS measurement resource(s) in Method#2 to consider the switching between UL subband and DL subband.

That being said, since RAN1 is also discussing a basic UE feature where the UE can only receive in one of the DL subbands, it may be possible that such UEs may not have the SRS/RSSI measurement resource within the active BWP and may need additional scheduling restrictions in case BWP switching or RF tuning is needed to perform the measurements. Or the UE performs measurements over the resources which fall within the active DL BWP of the UE. However, this is a separate problem and may need to be carefully studied. Perhaps it’s best if the group discusses what is the assumption on DL BWP for such UEs.

Proposal 6: RAN4 to discuss the assumptions on RF bandwidth and BWP and their impact on CLI measurement requirements for an SBFD UE which only supports the basic feature, i.e., ability to receive only in one of the DL subbands.

Measurement requirements

Moving on to the side conditions and corresponding measurement delays and accuracy requirements, we think the R16 SRS bandwidth (48PRBs) can be assumed for SBFD as well, but the side conditions on SRS Es/Iot need to be re-evaluated. For instance, R16 requirements were defined to support dynamic TDD where the DL was not protected from the aggressor UL, but for SBFD, the UL transmission takes place in the UL sub-band. So, the actual leakage to the DL sub-bands may be very low due to filtering at the UE side (which was not possible in dynamic TDD). Using SRS SNR of 1db as specified in R16 may show very little leakage in the DL subband and may not cause any degradation. So we think, to have meaningful measurements of the SRS RSRP, the SRS SNR should be considered at a sufficiently high level, e.g., at-least 10db.

Proposal 7: Reuse R16 assumptions on SRS bandwidth (i.e., 48 PRBs) for L1-SRS-RSRP measurements.

Proposal 8: RAN4 to consider higher side conditions of at-least 10 dB on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (1 dB).

# Conclusion

Proposal 1:  UE does not expect to be configured with L1-CLI-RSSI measurements resources in DL subband(s) (Method#1) and L1-SRS-RSRP (Method#2) measurement resources on the same symbol.

Proposal 2: UE does not expect to be configured with L1-CLI-RSSI measurements resources in UL subband(s) (Method#3) and L1-SRS-RSRP (Method#2) measurement resources on the same symbol.

Proposal 3: UE does not expect to be simultaneously configured with both L3 and L1 UE-to-UE CLI measurement resources, if UE supports both features.

Proposal 4: UE does not expect to be scheduled/configured with DL data channel/RS and L1-CLI-RSSI measurements resources in DL subband(s) (Method#1 for leakage measurement) on the same symbol.

Proposal 5: For a UE supporting simultaneous PDSCH reception on the two DL subbnads in DUD configuration - No need to specify scheduling restrictions on the symbol(s) after RSSI measurement resource(s) in Method#3 or after SRS measurement resource(s) in Method#2 to consider the switching between UL subband and DL subband.

Proposal 6: RAN4 to discuss the assumptions on RF bandwidth and BWP and their impact on CLI measurement requirements for an SBFD UE which only supports the basic feature, i.e., ability to receive only in one of the DL subbands.

Proposal 7: Reuse R16 assumptions on SRS bandwidth (i.e., 48 PRBs) for L1-SRS-RSRP measurements.

Proposal 8: RAN4 to consider higher side conditions of at-least 10 dB on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (1 dB).

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112

[3] R4-2416858, “WF on SBFD RRM”, Huawei, RAN4#112bis

[4] R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#113

[5] R4-2502699, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#114

[6] R4-2504904, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#114bis