3GPP TSG-RAN WG4 Meeting #115			    					           R4-2507910

# St. Julian’s, Malta, May 19-23, 2025

Title: 	WF on simultaneous Rx/Tx requirements

Agenda Item:	6.1

Source: 	Huawei

Document for:	Approval

# Issue#1: General simultaneous Rx/Tx requirements in Rel-19

Background:

RAN4 has concluded simultaneous Rx/Tx requirements for different types of inter-band CA and SUL configurations:

- TDD + TDD -> Conditional mandatory support for simultaneous RxTx with signaling

- FDD + TDD -> Mandatory support for simultaneous RxTx with signaling

- TDD + SDL -> Mandatory support for simultaneous RxTx with signaling

- FDD/TDD + SUL -> Mandatory support for simultaneous RxTx with signaling

- FDD + FDD -> Mandatory support for simultaneous RxTx without signaling

- FDD + SDL -> Mandatory support for simultaneous RxTx without signaling

<Way forward>:

- For 2-band inter-band CA, SUL, MR-DC band combinations with the duplex types that are mandatory supportive of simultaneous Rx/Tx with signalling (e.g., FDD+TDD, TDD+SDL, FDD/TDD+SUL), the Note of mandatory simultaneous Rx-Tx attached is not needed in the two-band BC table in the TS. Such Note is required for the band combinations that are conditionally supportive for simultaneous Rx-Tx.

  - Further discuss whether indication of mandatory support of simultaneous Rx/Tx are needed for inter-band CA, SUL, MR-DC band combinations with more than two bands. Take into consideration of RAN2 capability signalling of simultaneous Rx/Tx per band pair.

# Issue#2: Simultaneous Rx/Tx requirements for CA_n5-n8 in Rel-19

# Background:

For carrier aggregation involving a low-low band combination where the UL of one band partially overlaps with the DL of the other band, such as CA_n5-n8, RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.:

1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

In Rel-18, only Option 1 was introduced.

In RAN4#113, RAN4 sent LS [2] to ask RAN2 to define UE capability related to non-simultaneous on (n8UL, n5DL) in the FDD-FDD band combination CA_n5-n8. And RAN2 replied the LS with further question on the details about the requested capability. RAN4 need to provide sufficient information to RAN2 for reference, including the conclusions and progress in NR_700800900_combo_enh, LTE_NR_R19_Simult_RxTx and NR_LBCA_Sw, to design the capability.

In RAN4#114, we had agreement in [1] for FDD-FDD band combination that

- For FDD-FDD band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this due to UL CA_n5-n8 is TBD - For FDD-SDL band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this above due to LB-LB RX switching including LB-LB configurations in RP-242532 are TBD.

It is pointed out in [5] that RAN4 shall ensure RAN2 understands the need for a capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

<Way forward>:

For CA_n5-n8, further consider the following solution with side conditions to support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19:

Support case 1 and case 2 via RRC reconfiguration.

- Case 1: 2UL/2DL configuration, where NW shall keep all the four carriers activated, allocate resource allocation for a dedicated user only to n5 UL, n8 UL and n8 DL (no n5 DL resource allocation to a dedicated user) and both two bands are collocated.

- Case 2: 1UL/2DL configuration

Companies are encouraged to study necessity of additional RRM spec or additional requirement(s) of other working groups. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19.

Whether new capability is needed or not would be further discussed.

# Reference

[1] R4-2502840, WF on simultaneous Rx-Tx, Huawei, RAN#114, Athens, February, 2025

[2] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#113, Orlando, November, 2024

[3] R4-2411176, Motivation for a new work item on LB+LB CA based on switching in Rel-19, Apple, , Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc, RAN4#112.

[4] RP-243317, New WID on low band carrier aggregation via switching, RAN#106

[5] R4-2507374, On simultaneous RxTx and capability for overlapping FDD bands, Nokia, RAN4#115, Malta, May, 2025.