3GPP TSG-WG4 Meeting #115	R4-2507916

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_ENDC_RF_Ph4-Core] DraftCR to 38101-1-j10 on introducing the 1UL band 1CC HPUE MSD requirements simplification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc., Nokia, Samsung, Murata Manufacturing Co Ltd., Qualcomm France, ZTE, Huawei, Apple, Vivo, Mediatek. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Refer to agreed WF R4-2505143 and discussion paper R4-2507699. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following 1UL band 1CC inter-band CA HPUE MSD requirements are introduced in clause 7.3A.2.3: - Core requirements text specifying the eligible band combinations, - 1UL band 1CC simplification look-up tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-band CA HPUE MSD simplification for 1UL HPUE operation is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

In case the applicable reference sensitivity exceptions  (MSD) for PC2 aggressor NR UL band or PC1.5 aggressor NR UL band are not specified in clause 7.3A.4, 7.3A.5 or 7.3A.6 then,

PCx MSD = PC31Tx MSD + MSD

where,

- PCx = PC21Tx, PC22Tx, or PC1.52Tx,

- PC31Tx MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1,

and for PC2 or PC1.5 MSD due to harmonic, harmonic mixing, and cross-band isolation caused by the same aggressor UL band as PC3,

- MSD is specified in Table 7.3A.2.3-1 with three output columns, each denoted “MSDmax 3, 6, 9”, and,

- the correspondence between the MSDmax specified in Table 7.3A.2.3-1, the source of interference and the 1UL HPUE power-class is specified in Table 7.3A.2.3-2,

- for cases where the PC2 MSD is specified in Table 7.3A.4-2a or Table 7.3A.4-4a-1 or Table 7.3A.6-1a-1 but the PC3 MSD is not specified in Table 7.3A.4-1, or Table 7.3A.4-4 or Table 7.3A.6-1 and the PC1.5 MSD is not specified in Table 7.3A.4-4b or in Table 7.3A.6-1b, then the PC1.5 MSD is specified as PC1.52Tx MSD = PC21Tx MSD + MSD, where the MSD is specified in Table 7.3A.2.3-1 column “MSDmax 6” and where the input column uses the specified PC21Tx MSD instead of PC31Tx MSD,

when the following criterias are met:

- a PC3 reference sensivity exception requirement is specified either in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1, and,

- PC2 achieved with one Tx antenna connector “PC21Tx”, or PC2 achieved with two Tx antenna connectors “PC22Tx”, or PC1.5 achieved with two Tx antenna connectors “PC1.52Tx” and is specified as a valid UL configuration in Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1n, and,

- the aggressor NR UL band is neither band n46, band n96 nor band n102.

Table 7.3A.2.3-1: MSDmax look-up-table for MSD due to harmonic, harmonic mixing, cross-band isolation

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ PC31Tx MSD <1.0 | 0.7 | 1.1 | 2.5 |
| 1.0≤ PC31Tx MSD <3.0 | 1.6 | 2.7 | 5.1 |
| 3.0≤ PC31Tx MSD <5.0 | 2.0 | 3.8 | 6.5 |
| 5.0≤ PC31Tx MSD <7.0 | 2.3 | 4.5 | 7.3 |
| 7.0≤ PC31Tx MSD <9.0 | 2.5 | 5.0 | 7.9 |
| PC31Tx MSD ≥9.0 | 3 | 6 | 9 |

Table 7.3A.2.3-2: 1UL band 1UL CC MSDmax correspondence look-up-table

| Source of interference | MSDmax |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |
| Harmonic mixing | PC21Tx | PC22Tx | PC1.52Tx |
| Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

The reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation and power class 2 or power class 3 CA intermodulation interferences are applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

<END OF CHANGES>