# 3GPP TSG-RAN WG4 Meeting # 115	                                                           R4-2507922

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS37.141: Introduction of 7 MHz NR FR1 channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2025-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of 7 MHz NR FR1 channel bandwidth requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Required changes to support 7 MHz NR FR1 channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7 MHz NR FR1 channel bandwidth requirements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.1, 6.6.4.5.4, 6.6.4.5.6, 7.4.5.2, 7.7.5.1, 7.7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised version of R4-2506473. |  |  |  |  |  |  |  |  |

# << Start of change >>

4.1.2.1	Measurement of transmitter

Table 4.1.2-1: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.1 Base Station maximum output power | ±0.7 dB for UTRA, E-UTRA and NR, f ≤ 3.0 GHz ±1.0 dB, 3.0 GHz < f ≤ 4.2 GHz for UTRA, E-UTRA and NR ±1.0 dB for GSM/EDGE or standalone NB-IoT |  |
| 6.4 Transmit ON/OFF power | ±2.0 dB, f ≤ 3.0 GHz ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.6.1.5.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB |  |
| 6.6.1.5.2 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB |  |
| 6.6.1.5.3 Transmitter spurious emissions, Additional BC2 Requirement | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 12.75 GHz: ±4.0 dB |  |
| 6.6.1.5.4 Transmitter spurious emissions, Protection of BS receiver | ±3.0 dB |  |
| 6.6.1.5.5 Transmitter spurious emissions, Additional spurious emission requirements | ±2.0 dB for > -60dBm, f ≤ 3.0 GHz ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.0 dB for ≤ -60dBm, f ≤ 3.0 GHz ±3.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.6.1.5.6 Transmitter spurious emissions, Co-location | ±3.0 dB |  |
| 6.6.2 Operating band unwanted emissions | ±1.5 dB, f ≤ 3.0 GHz ±1.8 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.6.3 Occupied bandwidth | For NR: 5 MHz, 7 MHz, 10 MHz BS Channel BW: ±100 kHz 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz BS Channel BW: ±300 kHz 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz BS Channel BW: ±600 kHz For E-UTRA: 1.4MHz, 3MHz Channel BW: ±30kHz 5MHz, 10MHz Channel BW: ±100kHz 15MHz, 20MHz: Channel BW: ±300kHz For UTRA: ±100kHz |  |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) | ACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3.0GHz Absolute power ±2.5 dB, 3.0GHz < f ≤ 4.2GHz CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB CACLR absolute power ±2.0 dB, f ≤ 3.0 GHz CACLR absolute power ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.7 Transmitter intermodulation (interferer requirements) This tolerance applies to the stimulus and not the measurements defined in 6.6.1, 6.6.2 and 6.6.4 | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests (6.6.1, 6.6.2 and 6.6.4) which have to be carried out in the presence of the interferer. ±1.0 dB | The uncertainty of interferer has double the effect on the result due to the frequency offset |

# << Next change >>

6.6.4.5.4	Cumulative ACLR requirement in non-contiguous spectrum

The following test requirement applies for sub-block or Inter RF Bandwidth gap sizes listed in Table 6.6.4.5.4-1,

-	Inside a sub-block gap within an operating band for a BS operating in non-contiguous spectrum.

-	Inside an Inter RF Bandwidth gap for a BS operating in multiple bands, where multiple bands are mapped on the same antenna connector.

The Cumulative Adjacent Channel Leakage Power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The requirement applies to adjacent channels of NR, E-UTRA or UTRA carriers allocated adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap. The assumed filter for the adjacent channel frequency is defined in Table 6.6.4.5.4-1 and the filters on the assigned channels are defined in Table 6.6.4.5.4-2.

NOTE:	If the RAT on the assigned channel frequencies is different, the filters used are also different.

For Wide Area Category A BS, either the CACLR limits in Table 6.6.4.5.4-1 or the absolute limit of -13dBm/MHz shall apply, whichever is less stringent.

For Wide Area Category B BS, either the CACLR limits in Table 6.6.4.5.4-1 or the absolute limit of -15dBm/MHz shall apply, whichever is less stringent.

For Medium Range BS, either the CACLR limits in Table 6.6.4.4-1 or the absolute limit of -25 dBm/MHz shall apply, whichever is less stringent.

For Local Area BS, either the CACLR limits in Table 6.6.4.4-1 or the absolute limit of -32 dBm/MHz shall apply, whichever is less stringent.

The CACLR for E-UTRA and UTRA carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap shall be higher than the value specified in Table 6.6.4.5.4-1.

Table 6.6.4.5.4-1: Base Station CACLR in non-contiguous spectrum or multiple bands

| Band Category | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| BC1, BC2 | 5 ≤ Wgap < 15 (Note 3) | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| BC1, BC2 | 10 ≤ Wgap < 20 (Note 3) | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| BC3 | 5 ≤ Wgap < 15 (Note 3) | 2.5 MHz | 5MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| BC3 | 10 < Wgap < 20 (Note 3) | 7.5 MHz | 5MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| BC1, BC2, BC3 | 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| BC1, BC2, BC3 | 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| BC1, BC2, BC3 | 20 ≤ Wgap < 30 (Note 3, 5) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| BC1, BC2, BC3 | 20 ≤ Wgap < 60 (Note 4) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| BC1, BC2, BC3 | 40 ≤ Wgap < 50 (Note 3, 5) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| BC1, BC2, BC3 | 40 ≤ Wgap < 80 (Note 4) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| NOTE 1: For BC1 and BC2 the RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [3], with a chip rate as defined in this table. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the channel bandwidth of the carrier transmitted at the other edge of the gap is 5, 7, 10, 15, 20 MHz. NOTE 4: Applicable in case the channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. NOTE 5: Applicable in case the channel bandwidth of the NR carrier transmitted adjacent to sub-block gap or inter RF Bandwidth gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

Table 6.6.4.5.4-2: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| E-UTRA | E-UTRA of same BW |
| UTRA FDD | RRC (3.84 Mcps) |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [3], with a chip rate as defined in this table. |  |

# << Next change >>

6.6.4.5.6	NR test requirement

For NR, the requirements shall apply outside the Base Station RF Bandwidth or Radio Bandwidth whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer’s specification.

For a BS operating in non-contiguous spectrum, the ACLR requirement shall apply in sub-block gaps for the frequency ranges defined in table 6.6.4.5.6-2a, while the CACLR requirement shall apply in sub-block gaps for the frequency ranges defined in table 6.6.4.5.4-1.

For BS operating in multiple bands, where multiple bands are mapped onto the same antenna connector, the ACLR requirement shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.4.5.6-2a, while the CACLR requirement in clause 6.6.4.5.4 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.4.5.4-1.

The requirement shall apply during the transmitter ON period. The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

The ACLR absolute limit in table 6.6.4.5.6-2 or the ACLR (CACLR) limit in table 6.6.4.5.6-1, 6.6.4.5.6-2a or 6.6.4.5.4-1, whichever is less stringent, shall apply for each antenna connector.

For Band 41 operation in Japan, absolute ACLR limits shall be applied to the sum of the absolute ACLR power over all antenna connectors.

For operation in paired and unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.4.5.6-1.

Table 6.6.4.5.6-1: Base station ACLR limit

| Channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44.2 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44.2 dB |
| FR1 channel bandwidth as defined in Table 5.3.2-1 of TS 38.104 [27] | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44.2 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44.2 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |

The ACLR absolute limit is specified in table 6.6.4.5.6-2.

Table 6.6.4.5.6-2: Base station ACLR absolute limit

| BS category / BS class | ACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

For operation in non-contiguous spectrum or multiple bands, the ACLR shall be higher than the value specified in Table 6.6.4.5.6-2a.

Table 6.6.4.5.6-2a: Base Station ACLR limit in non-contiguous spectrum or multiple bands

| Channel bandwidth of NR carrier transmitted adjacent to sub-block gap or inter RF Bandwidth gap BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the channel bandwidth of the carrier transmitted at the other edge of the gap is 5, 7, 10, 15, 20 MHz. NOTE 4: Applicable in case the channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

# << Next change >>

7.4.5.2	General narrowband blocking test requirement

For the narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in Annex A.3.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS operating in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least 3MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the Base Station antenna input, using the parameters in Table 7.4.5.2-1 the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], clause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], clause 7.2.

-	For any measured NB-IoT carrier(standalone or operating in E-UTRA in-band/guard band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured NB-IoT carrier (operating in NR in-band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined inTS 38.104 [27], clause 7.2.

-	For any measured NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

Table 7.4.5.2-1: Narrowband blocking requirement

| Base Station Type | RAT of the carrier | Wanted signal mean power [dBm] (Note 1, 2, 6) | Interfering signal mean power [dBm] | Interfering RB (Note 3) centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap [kHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | NR, E-UTRA, NB-IoT (Note 4), |  | -49 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 (Note 5) |
| Medium Range BS | UTRA and | PREFSENS + x dB | -44 | ±(550 +m*180), |
| Local Area BS | GSM/EDGE |  | -41 | m=0, 1, 2, 3, 4 (Note 6) |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104. NOTE 2: "x" is equal to 6 in case of NR, E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. "x" is specified in Table 7.4.2-2 for NB-IoT standalone and NB-IoT operation in E-UTRA in-band/guard band and in Table 7.4.2-3 for NB-IoT operation in NR in-band. NOTE 3: Interfering signal (E-UTRA 3MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 4: For NB-IoT, the mentioned desensitized values consider only one NB-IoT PRB in the guard band, which is placed adjacent to the E-UTRA PRB edge as close as possible (i.e., away from edge of channel bandwidth). NOTE 5: Applicable for channel bandwidths equal to or below 20 MHz. NOTE 6: Applicable for channel bandwidths above 20 MHz. NOTE 6: 7.5 kHz shift is not applied to the wanted signal of NR. NOTE 7: Void |  |  |  |  |

Table 7.4.5.2-2: "x" for NB-IoT wanted signals operation in E-UTRA in-band/guard band and NB-IoT standalone

| Operation mode | LTE channel bandwidth for in-band/guard band operation | x |
| --- | --- | --- |
| Standalone | - | 12 |
| In Band | 3 MHz | 11 |
|  | 5 MHz | 9 |
|  | 10 MHz | 6 |
|  | 15 MHz | 6 |
|  | 20 MHz | 6 |
| Guard band | 5 MHz | 13 |
|  | 10 MHz | 6 |
|  | 15 MHz | 6 |
|  | 20 MHz | 6 |

Table 7.4.5.2-3: "x" for NB-IoT wanted signals operation in NR in-band

| Operation mode | NR channel bandwidth for in-band operation | x |
| --- | --- | --- |
| In Band | 5 MHz | 9 |
|  | 7 MHz | 6 |
|  | ≥ 10 MHz | 6 |

# << Next change >>

7.7.5.1	General intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal, as specified in Annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the Base Station antenna input, using the parameters in Table 7.7.5.1-1 and 7.7.5.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], clause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], clause 7.2.

-	For any measured NB-IoT carrier (standalone or operating in E-UTRA in-band/guard band),, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured NB-IoT carrier (operating in NR in-band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

-	For any measured NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

Table 7.7.5.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48+y (Note 6) | PREFSENS +x dB (Note 2, 5) |  |
| Medium Range BS | -44+y (Note 6) | PREFSENS +x dB (Note 3, 5) | See Table 7.7.5.1-2 |
| Local Area BS | -38+y (Note 6) | PREFSENS +x dB (Note 4, 5) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [2]. For E-UTRA channel bandwidths 10, 15 and 20 MHz this requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 2: For WA BS supporting GSM and/or UTRA, "x" is equal to 6 in case of NR or E-UTRA or UTRA or NB-IoT wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS supporting GSM and/or UTRA, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of NR or E-UTRA or NB-IoT wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS supporting GSM and/or UTRA, "x" is equal to 12 in case of NR or E-UTRA or NB-IoT wanted signals, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS neither supporting GSM nor UTRA, x is equal to 6 for all BS classes if NR is supported, otherwise x is equal to 6 for WA BS or, 9 for MR BS and 12 for LA BS if NR is not supported. NOTE 6: For a BS supporting NR but neither UTRA nor GSM; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. For all other cases, "y" is equal to zero for all BS classes. |  |  |  |

Table 7.7.5.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2.0 (BC1 and BC3) / ±2.1 (BC2) | CW |
|  | ±4.9 | 1.4MHz E-UTRA signal |
| E-UTRA 3 MHz or E-UTRA with NB-IoT in-band | ±4.4 (BC1 and BC3) / ±4.5 (BC2) | CW |
|  | ±10.5 | 3MHz E-UTRA signal |
| UTRA FDD and E-UTRA or E-UTRA with NB-IoT in-band/guard band 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 10 MHz | ±7.375 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band15 MHz | ±7.25 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 20 MHz | ±7.125 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| GSM/EDGE | ±7.575 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NB-IoT standalone | ±7.575 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| 1.28 Mcps UTRA TDD | ±2.3 (BC3) | CW |
|  | ±5.6 (BC3) | 1.28Mcps UTRA TDD signal |
| NR 5 MHz or NR with NB-IoT operation in NR in-band | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 7 MHz or NR with NB-IoT operation in NR in-band | ±7.45 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 10 MHz or NR with NB-IoT operation in NR in-band | ±7.465 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 15 MHz or NR with NB-IoT operation in NR in-band | ±7.43 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 20 MHz or NR with NB-IoT operation in NR in-band | ±7.395 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 25 MHz or NR with NB-IoT operation in NR in-band | ±7.465 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 30 MHz or NR with NB-IoT operation in NR in-band | ±7.43 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 35 MHz or NR with NB-IoT operation in NR in-band | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz or NR with NB-IoT operation in NR in-band | ±7.45 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 45 MHz or NR with NB-IoT operation in NR in-band | ±7.37 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz or NR with NB-IoT operation in NR in-band | ±7.35 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20MHz E-UTRA signal |

# << Next change >>

7.7.5.2	General narrowband intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal, as specified in Annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS operating in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in Table 7.7.5.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in Table 7.7.5.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the Base Station antenna input, using the parameters in Table 7.7.5.2-1 and 7.7.5.2-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], clause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], clause 7.2.

-	For any measured GSM/EDGE carrier, the conditions are specified in TS 45.005 [6], Annex P.2.2.

-	For any measured NB-IoT carrier (standalone or operating in E-UTRA in-band/guard band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], clause 7.2.

-	For any measured NB-IoT carrier (operating in NR in-band), the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

-	For any measured NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

-	Maximum throughput of the reference measurement channel defined in TS 38.104 [27], clause 7.2.

Table 7.7.5.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 |  |  |
| Medium Range BS | -47 | PREFSENS +x dB | See Table 7.7.5.2-2 |
| Local Area BS | -44 | (NOTE 1) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104. "x" is equal to 6 in case of NR, NB-IoT,E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. "x" is specified in Table 7.7.5.2-1a for NB-IoT |  |  |  |

Table 7.7.5.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge | Interfering signal centre frequency offset from the Base Station RF Bandwidthedge or sub-block edge inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1.4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3.0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 5 MHz | ±360 (NOTE 3) | CW |
|  | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 10 MHz | ±325 (NOTE 3) | CW |
| (NOTE 2) | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 15 MHz | ±380 (NOTE 3) | CW |
| (NOTE 2) | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA with NB-IoT in-band/guard band 20 MHz | ±345 (NOTE 3) | CW |
| (NOTE 2) | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| NB-IoT standalone | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| 1.28Mcps UTRA TDD | ±190 (BC3) | CW |
|  | ±970 (BC3) | 1.4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz or NR with NB-IoT operation in NR in-band | ±360 | CW |
|  | ±1420 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 7 MHz or NR with NB-IoT operation in NR in-band | ±400 | CW |
|  | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz or NR with NB-IoT operation in NR in-band | ±370 | CW |
|  | ±1960 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz or NR with NB-IoT operation in NR in-band | ±380 | CW |
| (Note 2) | ±1960 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz or NR with NB-IoT operation in NR in-band | ±390 | CW |
| (Note 2) | ±2320 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz or NR with NB-IoT operation in NR in-band | ±325 | CW |
| (Note 2) | ±2350 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz or NR with NB-IoT operation in NR in-band | ±335 | CW |
| (Note 2) | ±2350 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 35 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±345 | CW |
|  | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz or NR with NB-IoT operation in NR in-band | ±355 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 45 MHz or NR with NB-IoT operation in NR in-band (Note 2) | ±365 | CW |
|  | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz or NR with NB-IoT operation in NR in-band | ±375 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz | ±395 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz | ±415 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz | ±435 | CW |
| (Note 2) | ±2710 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz | ±365 | CW |
| (Note 2) | ±2530 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz | ±385 | CW |
| (Note 2) | ±2530 | 20 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge or sub-block edge inside a gap. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 or NR G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT in-band/guard band operation. NOTE 4: The frequency offset shall be adjusted to accommodate the IMD product to fall in the NB-IoT RB for NB-IoT in-band/guard band operation. NOTE 5: If a BS RF receiver fails the test of the requirement, the test shall be performed with the CW interfering signal frequency shifted away from the wanted signal by 180 kHz and the E-UTRA interfering signal frequency shifted away from the wanted signal by 360 kHz. If the BS RF receiver still fails the test after the frequency shift, then the BS RF receiver shall be deemed to fail the requirement. |  |  |

# << End of change >>